| 版本 | 1.0.1 |
| --- | --- |

| 修订日期 | 2022/03/21 |
| --- | --- |

# 新大陆外设密码键盘指令集

# 说明文档

# 目录表

新大陆外设密码键盘指令集	1

说明文档	1

目录表	2

1	文档介绍	9

1.1	文档目的	9

1.2	文档范围	9

1.3	文档缩略语定义	9

2	密码键盘通讯协议	10

2.1	报文结构	10

2.2	报文长度	11

2.3	通讯用波特率	11

2.4	关于时间的注意事项	11

3	设备基础功能指令	12

3.1	获取软件版本号 (0x32 0x30)	12

3.2	显示选项并获取键值 (0x33 0x30)	12

3.3	蜂鸣器开关 (0x36 0x69)	13

3.4	蜂鸣器鸣响 (0x33 0x39)	13

3.5	获取设备SN号 (0x36 0x34)	14

3.6	获取设备TUSN (0x36 0x36)	14

3.7	设备属性 (0x39 0x30)	14

3.8	载入配置 (0x39 0x32)	15

3.9	获取当前配置 (0x39 0x34)	16

3.10	升级指令集应用 (过程描述)	17

3.10.1	创建文件 (0x34 0x36)	17

3.10.2	写入应用文件数据 (0x34 0x38)	17

3.10.3	升级应用 (0x34 0x34)	18

3.11	控制LED灯 (0x41 0x38)	18

3.12	重启设备 (0x41 0x32)	19

4	屏幕显示指令	20

4.1	显示文本 (0x33 0x43)	20

4.2	显示图像 (0x33 0x44)	20

4.3	载入图像 (0x33 0x45)	21

4.4	显示图像（分包） (0x44 0x30)	21

4.5	显示QR码 (0x44 0x32)	22

4.6	设置字体颜色 (0x34 0x32)	23

4.7	设置字体大小 (0x35 0x61)	23

4.8	设置是否自动清屏 (0x64 0x36)	24

4.9	清空屏幕 (0x34 0x65)	24

4.10	回到主界面 (0x38 0x38)	25

4.11	设置文本显示的对齐方式 (0x35 0x63)	25

4.12	显示彩图 (0x37 0x61)	26

4.13	显示中文 (0x38 0x34)	27

4.14	点阵显示 (0x33 0x36)	27

5	卡类指令	29

5.1	非接卡命令说明 (0x48 0x30)	29

5.1.1	初始化读卡器 (0x48 0x30 0x2F 0x01)	29

5.1.2	检测读卡器区域 (0x48 0x30 0x2F 0x02)	29

5.1.3	激活感应区域 (0x48 0x30 0x2F 0x30)	30

5.1.4	检测非接卡在位 (0x48 0x30 0x2F 0x31)	30

5.1.5	非接卡APDU指令交互 (0x48 0x30 0x2F 0x32)	31

5.1.6	感应区下电 (0x48 0x30 0x2F 0x33)	31

5.1.7	LED灯闪烁(0x48 0x30 0x2F 0x34)	32

5.1.8	明文APDU指令交互 (0x48 0x30 0x2F 0x35)	32

5.1.9	检测Felica卡在位 (0x48 0x30 0x2F 0x36)	33

5.1.10	Felica卡APDU指令交互 (0x48 0x30 0x2F 0x37)	33

5.1.11	多种类卡激活 (0x48 0x30 0x2F 0x40)	34

5.1.12	(M1卡)外部密钥认证 (0x48 0x30 0x2F 0x44)	35

5.1.13	(M1卡)读取块数据 (0x48 0x30 0x2F 0x45)	36

5.1.14	(M1卡)写入块数据 (0x48 0x30 0x2F 0x46)	36

5.1.15	(M1卡)增量操作 (0x48 0x30 0x2F 0x47)	37

5.1.16	(M1卡)减量操作 (0x48 0x30 0x2F 0x48)	37

5.2	IC卡指令说明 (0x42 0x30)	38

5.2.1	IC卡交互 (0x42 0x30 0x2F 0x00)	38

5.2.2	IC卡上电 (0x42 0x30 0x2F 0x01)	39

5.2.3	IC卡下电 (0x42 0x30 0x2F 0x02)	40

5.2.4	IC卡读写 (0x42 0x30 0x2F 0x03)	40

5.2.5	IC卡检测 (0x42 0x30 0x2F 0x04)	42

5.2.6	设置接触卡上电类型 (0x42 0x30 0x2F 0x05)	42

5.3	读磁卡 (0x4a 0x30)	43

5.4	多卡类寻卡(旧称三界面寻卡)	45

5.4.1	三重寻卡 (0x42 0x42)	45

5.4.2	获取磁道数据 (0x42 0x44)	46

5.4.3	取消三重寻卡 (0x42 0x46)	48

6	PIN输入指令	49

6.1	设置有效输PIN长度 (0x42 0x48)	49

6.2	设置PIN输入密码显示行数 (0x64 0x34)	49

6.3	完成PIN输入 (0x42 0x4C)	49

6.4	取消PIN输入 (0x33 0x38)	50

6.5	PIN输入 (0x33 0x32)	50

6.6	脱机PIN输入(0x64 0x38)	52

6.7	敏感信息输入 (0x36 0x65)	54

6.8	获取键值 (0x36 0x67)	55

6.9	脱机PIN验证 (0x64 0x41)	55

7	安全相关指令	57

7.1	密钥装载 (0x35 0x30)	57

7.2	获取密钥KCV (0x35 0x32)	57

7.3	获取密钥KCV(新) (0x35 0x41)	58

7.4	DES数据加解密(0x38 0x30)	59

7.5	MAC生成 (0x38 0x43)	60

7.6	装载DUKPT密钥 (0x37 0x63)	61

7.7	数据加解密(DUKPT) (0x37 0x65)	62

7.8	获取DUKPT密钥的KSN (0x33 0x34)	63

7.9	KSN增加 (0x37 0x67)	63

7.10	AES数据加解密(0x38 0x32)	63

7.11	SM4数据加解密(0x37 0x6d)	64

7.12	删除密钥 (0x42 0x4A)	65

7.13	清除密钥 (0x36 0x30)	65

7.14	生成SM2密钥对 (0x39 0x36)	66

7.15	装载主控密钥 (0x39 0x38)	66

7.16	装载厂商公钥 (0x39 0x41)	67

7.17	获取设备公钥 (0x39 0x43)	67

7.18	生成密钥认证信息 (0x39 0x45)	67

7.19	获取SM2算法签名信息 (0x39 0x47)	68

7.20	数据加解密(银商DES) (0x39 0x4F)	68

7.21	数据加解密(银商SM4) (0x39 0x51)	70

8	EMV Command Oerder Set	71

8.1	EMV Configuration Command	71

8.1.1	Termial Configuration Commands	71

8.1.1.1	Update Terminal Configuration Command	71

8.1.1.2	Get Terminal Configuration Command	71

8.1.2	AID Configuration Commands	72

8.1.2.1	Update AID Configuration Command	72

8.1.2.2	Get AID Configuration Command	73

8.1.2.3	Remove AID Configuration Command	74

8.1.2.4	Remove All AID Configuration Command	74

8.1.3	CAPK Management Command	75

8.1.3.1	Update CAPK Command	75

8.1.3.2	Get CAPK Command	76

8.1.3.3	Remove One CAPK Command	77

8.1.3.4	Remove All CAPK Command	77

8.1.4	Revocation List Management Command	78

8.1.4.1	Update Revocation List Command	78

8.1.4.2	Get Revocation List command	78

8.1.4.3	Remove One Revocation List Command	79

8.1.4.4	Remove All Revocation List command	79

8.1.5	Exception List Management Command	80

8.1.5.1	Update Exception List Command	80

8.1.5.2	Get Exception List Command	80

8.1.5.3	Remove One Exception List Command	81

8.1.5.4	Remove All Exception List Command	82

8.1.6	GET AID/CAPK NUM	82

8.1.6.1	Get Aid Num	82

8.1.6.2	Get CAPK Num	83

8.2	Function Command	84

8.2.1	Send Debug Message Command	84

Request message	84

Response message	84

8.2.2	Init EMV Command	84

Request message	84

Response message	85

8.2.3	Set Data Command	85

Request message	85

Response message	85

8.2.4	Get Data Command	85

Request message	85

Response message	88

8.2.5	Set Tlv List Data Command	88

Request message	89

Response message	89

8.2.6	Get Tlv List Data Command	89

Request message	89

Response message	89

8.2.7	Set Debug Mode Command	90

Request message	90

Response message	90

8.2.8	Get Version Command	90

Request message	91

Response message	91

8.2.9	CALLBACK DEAL Management Command	91

8.2.9.1	L3_CALLBACK_Notification	91

8.2.9.2	L3_CALLBACK_SELECT_CANDIDATE_LIST	94

8.2.9.3	L3_CALLBACK_AFTER_FINAL_SELECT	96

8.2.9.4	L3_CALLBACK_PIN_ENTRY_DEAL	97

8.2.9.5	L3_CALLBACK_CHECK_CREDENTIALS	100

8.2.9.6	L3_CALLBACK_CARDNUM_CONFIRM	101

8.2.9.7	L3_CALLBACK_DEK	102

8.2.9.8	L3_CALLBACK_DET	102

8.3	Transaction Command	102

8.3.1	Performed Transaction Command	102

Request message	102

Response message	106

8.3.2	Complete Transaction Command	106

Request message	107

Response message	108

8.3.3	Terminate Transaction Command	108

Request message	109

Response message	109

8.3.4	Transaction PreProcess Command	109

Request message	109

Response message	110

8.3.5	Cancel Transaction Command	110

8.4	Use the EMV Command Order Set	110

8.5	Terminal Configuration	111

8.5.1	Contact	111

8.5.2	Contactless	128

8.6	AID Configuration	148

8.6.1	Contact EMV AIDs	149

8.6.2	Contactless Visa AIDs (Kernel 3)	154

8.6.3	Contactless MasterCard AIDs (Kernel 2)	158

8.6.4	Contactless American Express AIDs (Kernel 4)	172

8.6.5	Contactless Discover AIDs (Kernel 6)	176

8.6.6	Contactless JCB AIDs (Kernel 5)	179

8.6.7	Contactless Unionpay AIDs (Kernel 7)	183

8.6.8	Contactless Pure(MCCS/MADA) AIDs	186

8.6.9	Contactless Rupay AIDs	189

8.6.10	Contactless Interac AIDs	196

8.6.11	Contactless MIR AIDs	200

8.6.12	Contactless Multibanco AIDs	205

9	Annex A: Sample Communication Traces	210

9.1	Display Text Request	210

9.2	Card Insertion Status Request	210

10	ANNEX B. ADDITIONAL SPECIFICATION ASPECTS	211

10.1	Public Key Infrastructure for Newland Devices	211

Document Modification History

| Revision | Description | Author | Date |
| --- | --- | --- | --- |
| 02.00 | First draft |  | 2014-07-01 |
| 02.01 | Add SP100 Terminal |  | 2014-10-20 |
| 02.10 | Add AES command | zhangjw | 2017-07-06 |
| 02.17 | Add New EMV command | George | 2019-09-11 |
| 02.18 | Add date key index to Get Data and Get Tlv List Data command to support CBC encryption for response data. | Liaosc | 2019-09-17 |
| 02.20 | - Delete Init Reader Command in Chapter 6 - Delete Check Reader Command in Chapter 7 - Add Send Debug Message Command - Change the cancel command | George | 2019-09-19 |
| 02.22 | Modify some issues | George | 2019-10-16 |
| 02.23 | Change the Cancel Transaction Command | Christine | 2019-10-22 |
| 02.24 | 1. Add 1F8131 in “Performed Transaction”and”Complete Transaction” 2. Add display data “Terminata Transation” | Christine | 2019-11-07 |
| V1.7 | 1.update EMV command | Wangsx | 2021-05-27 |

- 文档介绍

  - 文档目的

本文档是为外设密码键盘上的指令集应用而写的说明文档。旨在帮助开发人员了解指令集通讯协议，了解各项指令的设计目的、使用方法，使得上位机应用开发有效地进行。

  - 文档范围

本文档仅适用于当前国内在用的密码键盘型号（即SP100和ME51）上运行的指令集应用。若用于海外指令集可能出现某些指令不支持或未实现的情况。

  - 文档缩略语定义

Var：变长数据域

Msg：Message，消息/报文

ddh：十六进制数dd

- 密码键盘通讯协议

  - 报文结构

| STX | LLLL | Msg Type | Separator | Msg Data | ETX | LRC |
| --- | --- | --- | --- | --- | --- | --- |
| 1Byte(02h) | 2Byte(BCD) | 2Byte | 1Byte(2Fh) | Var | 1Byte(03h) | 1Byte |

| 名称 | 说明 |
| --- | --- |
| STX | 报文头，固定为1字节02h。 |
| LLLL | 数据域长度，2字节BCD编码数据，指定包括指令类型、分隔符、指令数据在内的所有数据字节数。 |
| Msg Type | 指令类型，指定了读取指令数据时所依据的格式 |
| Separator | 分隔符，分隔指令类型和指令数据部分，固定为1字节2Fh |
| Msg Data | 指令数据，按照指令类型所对应的指令格式上传的数据。 |
| ETX | 报文尾固定为1字节03h。 |
| LRC | 将从2字节数据域长度开始直到报文尾（包括报文尾）的所有字节进行校验，异或结果即为LRC。 |

除以上说明外，指令集应用收到上位机下发的报文后会返回一个ACK(06h)或NAK(15h)给上位机，ACK表示指令结构正确，LRC没问题。NAK表示指令结构有问题，如数据域长度不符合实际字节数，缺少指令尾（通常是没有正确读到指令尾），LRC校验失败。

返回06后表示指令开始解析并进行相应操作，操作完成后会返回一个应答报文。上位机收到应答报文后需要返回一个ACK(06h)给密码键盘表示收到数据。如果收到的报文被发现在传输过程中出现了错误，上位机无需返回NAK给密码键盘，因为密码键盘无论是没有收到ACK还是收到了NAK，都会重发报文，每3秒重发1次，发三次。

另外需要注意的是，并非所有指令类型都会返回，无返回报文的会在文档指令名后标注。还有，如无特殊说明，各指令在发回响应报文并收到上位机的ACK后才会执行其他指令。

下列表格说明了在指令数据区域的各种数据类型，即描述中（如果有）取值范围代表的数据：

| Type | Range | Description |
| --- | --- | --- |
| A | 20h-7Fh | ASCII 数据 ，可显示的字符取值范围。 |
| B | 00h-FFh | 二进制数据，用十六进制字节表示，如无特殊说明，数据均代表十六进制数值，否则会在说明中标注解析采用的编码格式，如BCD编码。 |
| S | 00h-1Fh | 特殊的控制字符 |

  - 报文长度

由于缓冲区的限制，一次传送的完整报文大小不能超过4096字节。如无特殊说明数据不能分包传送。

  - 通讯用波特率

使用RJ11水晶头的设备，黑白屏的密码键盘默认使用57600波特率进行通讯，彩屏密码键盘默认使用115200波特率进行通讯。USB头的设备都是使用115200波特率通讯。

  - 关于时间的注意事项

当密码键盘在非恒定功率模式下运行时（即密码键盘电源在交易之间打开/关闭），密码键盘被认为准备好接收第一个消息，在它上电后 500 毫秒。 这是允许密码键盘启动并准备好接受传入消息的最长时间。 这并不意味着密码键盘需要立即处理这个请求，而是希望第一条消息被正确接收，不会丢失，并且会在 3 秒的重传时间窗口内收到 ACK 响应，以避免任何重传和额外的 延迟给用户。

响应消息到请求消息的时间因请求而异，并且未定义最大允许时间。可以通过发送新的请求消息或关闭密码键盘的电源来中断对已接收到的需要用户交互的请求消息（例如 PIN 输入）的处理。

- 设备基础功能指令

  - 获取软件版本号 (0x32 0x30)

Request

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “20” |  |
| Separator | 1 | A |  |  |

Response

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “21” |  |
| Separator | 1 | A |  |  |
| Response Code | 2 | A |  | “00” Good “02” General error |
| SOFTWAREVERLEN | 1 | B |  | Software version number length |
| SOFTWAREVER | var. | A |  | Software version number |

e.g.

Send: 02 00 03 32 30 2F 03 2D

Receive: 06 02 00 15 32 31 2F 30 30 09 56 30 35 2E 30 30 2E 30 31 03 61

SOFTWAREVERLEN：09h

SOFTWAREVER：”V05.00.01”

  - 显示选项并获取键值 (0x33 0x30)

Request

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “30” |  |
| Separator | 1 | A |  |  |
| Display Data | var | A |  | Display data for line 1. * |
| FS | 1 | S | 1Ch |  |
| Display Data | var | A |  | Display data for line 2. |
| FS | 1 | S | 1Ch |  |
| Display Data | var | A |  | Display data for line 3. |
| FS | 1 | S | 1Ch |  |
| Display Data | var | A |  | Optional, application dependent. Display data for line 4. |
| FS | 1 | S | 1Ch |  |

说明：这条指令下发后，在等待获取按键的过程中可以被其他下发的任何指令打断。

需要注意文档中所有可自定义显示的文本内容如无特殊说明均应使用UTF-8编码，若是要显示中文则需使用GB2312编码。国内小屏SP100不支持通过编码显示中文。

Response

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “31” |  |
| Separator | 1 | A |  |  |
| Function Key | 1 | A | X | Returns the key selected. 0 – Enter, 1 – F1, 2 – F2, 3 – F3 5 – Clear, 6 – Cancel |

e.g.

Send: 02 00 04 33 30 2F 30 03 1B

Receive: 06 02 00 04 33 31 2F 30 03 1A

  - 蜂鸣器开关 (0x36 0x69)

The buzzer switch request controls the on-off status of the buzzer. After turning off the buzzer, pressing buttons or prompting messages won’t make the buzzer beep.

Request

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “6i” |  |
| Separator | 1 | A |  |  |
| Switch Code | 1 | B |  | 0 – turn off 1 – turn on |

Response

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “6j” |  |
| Separator | 1 | A |  |  |
| Response Code | 2 | A |  | “00” Good “04” Error |

e.g. 02 00 04 36 69 2F 00 03 77

  - 蜂鸣器鸣响 (0x33 0x39)

Request

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “39” |  |
| Separator | 1 | A |  |  |
| Tone | 1 | B |  | Buzzer tone: 0 – Default 1 – Alert (750Hz) 2 – Success (1500Hz) |
| Duration | 2 | B |  | Duration time, in 10ms units, big endian. (BCD encoding) |

Response

说明：接收到本指令后密码键盘不会返回响应报文，仅会返回06表示接收到了报文并进行解析处理。

e.g.

Send: 02 00 06 33 39 2F 01 00 20 03 01

Receive: 06

  - 获取设备SN号 (0x36 0x34)

Request

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “64” |  |
| Separator | 1 | A |  |  |

Response

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “65” |  |
| Separator | 1 | A |  |  |
| Serial Number | 16 | A |  | PIN pad serial number |

说明：本指令返回的SN号从分隔符后第1个字节开始到遇到第1个00h字节结束为有效长度（相当于字符串遇到结束符结束）。16字节实际是预留长度，应以实际返回的有效长度为准。

e.g.

Send: 02 00 03 36 34 2F 03 2D

Receive: 06 02 00 19 36 35 2F 30 35 35 30 30 30 30 30 30 30 32 38 00 00 00 00 03 3C

  - 获取设备TUSN (0x36 0x36)

Request

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “66” |  |
| Separator | 1 | A |  |  |

Response

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “67” |  |
| Separator | 1 | A |  |  |
| Response Code | 2 | A |  | “00” Good “55” Unsupported |
| TUSN Length | 1 | B |  |  |
| TUSN | VAR |  |  |  |

e.g.

Send: 02 00 03 36 36 2F 03 2F

  - 设备属性 (0x39 0x30)

Request

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “90” |  |
| Separator | 1 | A |  |  |

Response

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “91” |  |
| Separator | 1 | A |  |  |
| Feature Bitmap | 2 | B |  | Feature Bitmap shows interface availability 0x01 – Smart Card interface present 0x02 – Contactless Card interface present 0x04 – Magnetic Stripe reader present 0x08 – Graphical display available 0x10 – Colour display available 0x20 – Beeper available 0x40 – Backlight available Others: RFU |
| PCI Firmware Identification Number (FWID) | var | A |  | PCI Firmware ID number, according to its specification and to the version of the device. |
| FS | 1 | S | 1Ch |  |

说明：该条指令主要用到FWID来判断机型，Feature Bitmap内容目前尚未有指令集版本通过实际调用接口去判断。

e.g.

Send: 02 00 03 39 30 2F 03 26

Receive: 06 02 00 33 39 31 2F 00 6B 53 50 31 30 30 5F 56 30 36 2E 30 30 2E 30 31 5F 30 30 5F 31 39 30 36 32 34 30 31 1C 03 55

  - 载入配置 (0x39 0x32)

Request

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “92” |  |
| Separator | 1 | A |  |  |
| Baud Rate | 2 | A |  | Selects Port baud rate. Mode Baud Rate 00 Do not change the baud rate 12 19200 8N1 14 2400 8N1 15 9600 8N1 17 38400 8N1 18 57600 8N1 19 115200 8N1 |
| Set Working key decrypt mode | 1 | A |  | 0 = Use CBC mode for TDEA working key decryption (Default). 1 = Use ECB mode for TDEA working key decryption. |
| Beeper Control | 1 | A |  | (Optional field. If this field is not sent, default mode is assumed.) 0 = Normal (keypad and selected messages) 1 = Keypad only 2 = Off Default mode is Normal (0). |

Response

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “93” |  |
| Separator | 1 | A |  |  |
| Response Code | 2 | A |  | “00” Good “02” General error(Failed to write file) “05” Invalid Parameter |

说明：本指令的三个属性中仅波特率的设置会生效，设置生效后会断开串口连接重连。本指令适用于RJ11水晶头的设备，不适用USB头的设备。

e.g.

Send: 02 00 07 39 32 2F 30 30 30 30 03 20

Receive: 06 02 00 05 39 33 2F 30 30 03 23

  - 获取当前配置 (0x39 0x34)

Request

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “94” |  |
| Separator | 1 | A |  |  |

Response

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “95” |  |
| Separator | 1 | A |  |  |
| Baud Rate | 2 | A |  | Selects Port baud rate. Mode Baud Rate 00 Do not change the baud rate 12 19200 8N1 14 2400 8N1 15 9600 8N1 17 38400 8N1 18 57600 8N1 19 115200 8N1 |
| Set Working key decrypt mode | 1 | A |  | 0 = Use CBC mode for TDES working key decryption (Default). 1 = Use ECB mode for TDES working key decryption. |
| Beeper Control | 1 | A |  | 0 = Normal (keypad and user prompt messages) 1 = Keypad only 2 = Off Default mode is Normal (0). |

e.g.

Send: 02 00 03 39 34 2F 03 22

Receive: 06 02 00 07 39 35 2F 31 32 30 30 03 24

  - 升级指令集应用 (过程描述)

升级过程的指令调用一定要按如下顺序进行：

创建文件->写入应用文件数据->升级应用

创建文件的文件名长度不能超过12字节。如果创建文件后设备关机了，重启后需要重新创建文件。在升级应用时如果选择了升级后立即重启，则载入的应用文件不会被删除；如果选择了升级后不重启则应用文件会被删除。

升级应用也可以通过进入设备的下载模式使用下载工具直接下载新的应用。

    - 创建文件 (0x34 0x36)

Request

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “46” |  |
| Separator | 1 | A |  |  |
| Length of app name | 1 | B |  | Length<=12 |
| App name | var | B |  | App name for loading |

Response

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “47” |  |
| Separator | 1 | A |  |  |
| Response Code | 2 | A |  | 00 – Good 01 – File delete error 02 – File create failed |

    - 写入应用文件数据 (0x34 0x38)

Request

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “48” |  |
| Separator | 1 | A |  |  |
| Length of data | 2 | B |  | Length less than 2048 |
| App data | var | B |  |  |

Response

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “49” |  |
| Separator | 1 | A |  |  |
| Response Code | 2 | A |  | “00” – Good “01”– File delete error “02” – File open failed “03” – File write failed |

    - 升级应用 (0x34 0x34)

Request

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “44” |  |
| Separator | 1 | A |  |  |
| Length of app name | 1 | B |  |  |
| App name | var | B |  | App name for loading |
| Reboot flag | 1 | B |  | 00 – not reboot , Update end delete file 01 – reboot ,without response command and do not delete file |

Response

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “45” |  |
| Separator | 1 | A |  |  |
| Response Code | 2 | A |  | 00 – Good 01 – File delete error 02 – File create failed 03 – Device open error 04 – NLD info error 05 – Sign Decrypt Error 06 – Sign Check Error 07 – Update Error |

  - 控制LED灯 (0x41 0x38)

Request

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “A8” |  |
| Separator | 1 | B |  |  |
| Led type | 1 | B |  | 0x01-red 0x02-yellow 0x04-green 0x08-blue |
| Operation type | 1 | B |  | 0x01-ON 0x02-OFF 0x03-FLICK |

Response

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “A9” |  |
| Separator | 1 | B |  |  |
| Response Code | 1 | B |  | “00” Good “01” Parameter error |

说明：有的设备4个灯都是绿灯，则设备从左至右的LED灯与四色的LED灯对应的位置关系为：1蓝灯，2黄灯，3绿灯，4红灯。

Send: 02 00 05 41 38 2F 0F 01 03 5E

Receive: 06 02 00 05 41 39 2F 30 30 03 51

  - 重启设备 (0x41 0x32)

Request

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “A2” |  |
| Separator | 1 | A |  |  |

Response

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “A3” |  |
| Separator | 1 | A |  |  |
| Response Code | 2 | A |  | 00 Good 02 Command Failed 04 Invalid command sequence |

说明：执行本指令后设备将重启，可结合升级应用的指令在删除文件后重启。

e.g.

Send: 02 00 03 41 32 2F 03 5C

Receive: 06 02 00 05 41 33 2F 30 30 03 5B

- 屏幕显示指令

  - 显示文本 (0x33 0x43)

Request

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “3C” |  |
| Separator | 1 | A | “.” |  |
| Display Data | var. | A |  | Display data for line 1. |
| FS | 1 | S | 1Ch |  |
| Display Data | var. | A |  | Display data for line 2. |
| FS | 1 | S | 1Ch |  |
| Display Data | var. | A |  | Display data for line 3. |
| FS | 1 | S | 1Ch |  |
| Display Data | var. | A |  | Display data for line 4. |
| FS | 1 | S | 1Ch |  |

Response

说明：接收到本指令后密码键盘不会返回响应报文，仅会返回06表示接收到了报文并进行解析处理。传入数据并不需要够4个1Ch（1Ch实际是作为换行符使用）超过一行的数据会自动切换到下一行继续显示。

e.g.

Send: 02 00 04 33 43 2F 31 03 69

Receive: 06

  - 显示图像 (0x33 0x44)

Request

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “3D” |  |
| Separator | 1 | A |  |  |
| Image ID | 1 | B |  | Image ID, 1..250, if ID = 0, show the image data on following field in this CMD |
| X | 2 | B |  | Display X coordinate, big endian |
| Y | 2 | B |  | Display Y coordinate, big endian |
| Image Data Length | 2 | B |  | Length of the image data, big endian. * |
| Image Data | … | B |  | Image data. * |

*Only exist when the Image ID equal to 0.

Response

说明：接收到本指令后密码键盘不会返回响应报文，仅会返回06表示接收到了报文并进行解析处理。本条指令显示的图像只支持单色位图。

  - 载入图像 (0x33 0x45)

Request

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “3E” |  |
| Separator | 1 | A |  |  |
| Image ID | 1 | B |  | Image ID, 1..250, used to reference the image |
| Image Data Length | 2 | B |  | Length of the image data, big endian |
| Image Data | … | B |  | Image data |

Response

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “3F” |  |
| Separator | 1 | A |  |  |
| Response Code | 2 | A |  | 00 – Good 02 – General error 10 – Customer Card error or not present |

说明：载入的图像数据与指令其他部分总共字节长度不能超过4096，且本指令只能载入单色位图供显示图像指令使用。

  - 显示图像（分包） (0x44 0x30)

Request

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “D0” | “D0” |
| Separator | 1 | A |  |  |
| X | 2 | B |  | Top left abscissa of picture(Pixel) |
| Y | 2 | B |  | The upper left ordinate of picture(Pixel) |
| Width | 2 | B |  | Picture width(Pixel) |
| Height | 2 | B |  | Picture height (Pixel) |
| Current Pack Number | 2 | B |  | The number of current package |
| Subsequent Pack Number | 2 | B |  | The number of subsequent package |
| Data | <=256 | B |  | Picture data(Each package is fixed to 256 bytes except the last package whose length is the actual length) |

Response

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “D1” |  |
| Separator | 1 | A |  |  |
| Response Code | 2 | A |  | 00 Good 01 Device failed to open 02 The device does not support scanning heads |

说明：本指令可通过分包传入较大的单色位图数据进行显示，一般比较少用，不建议使用。

  - 显示QR码 (0x44 0x32)

Request

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “D2” |  |
| Separator | 1 | A |  |  |
| Level | 1 | B |  | Error Correct Level(default 1, range 0-3) |
| Mask | 1 | B |  | Mask number, default 1,range 0-7 |
| Version | 1 | B |  | Default 0, range 0-11 |
| Auto Centering | 1 | B |  | 1-open ;0-close |
| x | 1 | B |  | <320. It is effective when Auto Centering is closed |
| y | 1 | B |  | <240. It is effective when Auto Centering is closed |
| Text position | 1 | B |  | 1-top of QR code ;0-bottom of QR code |
| Text string length | 1 | B |  | Maximum 256 bytes |
| Text string | …… | B |  | Pass this parameter when Text string length > 0 |
| Data Len | 2 | B |  | Maximum 512 bytes |
| Data | …… |  |  |  |

Response

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “D3” |  |
| Separator | 1 | A |  |  |
| Response Code | 2 | A |  | “00” Success “01” Error correct level error “02” Mask number error “03” Type error “04” Center automatically error “05” Initial abscissa error “06” Initial ordinate error “07” Text position error “08” Text length error “09” QR code length error “10” The width of QR code image is out of the screen range “11” The height of QR code image is out of the screen range “12” The height of text is out of the screen range “13” The ordinate position of QR code image is occupied by text “14” Operation error |

说明：其中version会影响二维码显示的大小。特别注意由于早期设计原因，Text string length字节标注的长度应该为实际的要显示的文本长度加1。

e.g.

Send:

02 00 50 44 32 2F 01 00 00 01 01 13 70 6c 65 61 73 65 20 73 63 61 6E 20 51 52 63 6F 64 65 00 15 68 74 74 70 3A 2F 2F 77 77 77 2E 62 61 69 64 75 2E 63 6F 6D 2F 03 72

Description:

Level: 01

Mask: 00

Version: 00

Auto Centering: 01

X: none

Y: none

Text position: 01

Text string length: 13

Text string: 70 6c 65 61 73 65 20 73 63 61 6E 20 51 52 63 6F 64 65

Data Len: 00 15

Data: 68 74 74 70 3A 2F 2F 77 77 77 2E 62 61 69 64 75 2E 63 6F 6D 2F

Receive: 02 00 05 44 33 2F 00 00 03 5E

Description: Success

  - 设置字体颜色 (0x34 0x32)

Request

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “42” |  |
| Separator | 1 | A |  |  |
| Color | 2 | B |  | 0x0000~0xFFFF |
| Set object | 1 | B |  | 00:Normal font |

Response

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “43” |  |
| Separator | 1 | A |  |  |
| Response Code | 2 | A |  | 00 – Good 01 – General error 02 – Set Display-Mode error |

e.g.

Send: 02 00 06 34 32 2F FF FF 00 03 2C

Receive: 06 02 00 05 34 33 2F 30 30 03 2E

  - 设置字体大小 (0x35 0x61)

Request

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “5a” |  |
| Separator | 1 | A |  |  |
| Font Size | 1 | B |  | 01 – Normal Font 02 – Small Font 03 – Large Font |

Response

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “5b” |  |
| Separator | 1 | A |  |  |
| Response Code | 2 | A |  | “00” – Good “01” – General error |

e.g.

Send: 02 00 04 35 61 2F 01 03 7D

Receive: 06 02 00 05 35 62 2F 30 30 03 7E

  - 设置是否自动清屏 (0x64 0x36)

Request

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “d6” |  |
| Separator | 1 | A |  |  |
| Direction | 1 | B |  | 01 – Automatic screen cleaning Other Values Manual screen cleaning |

Response

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “d7” |  |
| Separator | 1 | A |  |  |
| Direction | 2 | A |  | “00” – Good “01” – General error |

说明：本条指令可设置文本显示或PIN输入时的文本显示在显示前是否清屏。

e.g.

Send: 02 00 04 64 36 2F 01 03 7B

Receive: 06 02 00 05 64 37 2F 30 30 03 7A

  - 清空屏幕 (0x34 0x65)

Request

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “4e” |  |
| Separator | 1 | A |  |  |

Response

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “4f” |  |
| Separator | 1 | A |  |  |
| Response Code | 2 | A |  | “00” – Good “01” – clear screen error |

e.g.

Send: 02 00 03 34 65 2F 03 7E

Receive: 06 02 00 05 34 66 2F 30 30 03 7B

  - 回到主界面 (0x38 0x38)

Request

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “88” |  |
| Separator | 1 | B |  |  |

Response

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “89” |  |
| Separator | 1 | B |  |  |
| mode | 1 | B |  | “00” Good |

e.g.

Send: 02 00 03 38 38 2F 03 2F

Receive: 06 02 00 05 38 39 2F 30 30 03 28

  - 设置文本显示的对齐方式 (0x35 0x63)

Request

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “5c” |  |
| Separator | 1 | A |  |  |
| Direction | 1 | B |  | 00 – Restore default settings 01 – Align left 02 – Centre 03 – Align right |

Response

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “5d” |  |
| Separator | 1 | A |  |  |
| Direction | 2 | A |  | “00” – Good “01” – General error |

e.g.

Send: 02 00 04 35 63 2F 00 03 7E

Receive: 06 02 00 05 35 64 2F 30 30 03 78

  - 显示彩图 (0x37 0x61)

Request

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “7a” |  |
| Separator | 1 | A |  |  |
| Display mode | 1 | B |  | 00-only display 01-display and when width = 320&height=240 set it as background image 02-only load image to sp100 03-display the image that mode 2 load |
| X | 2 | B |  | (At present only prompt “display mode !=02” is available.) Top left abscissa of picture(Pixel) |
| Y | 2 | B |  | (At present only prompt “display mode !=02” is available.) The upper left ordinate of picture(Pixel) |
| Width | 2 | B |  | (At present only prompt “display mode !=02” is available.) Picture width(Pixel) |
| Height | 2 | B |  | (At present only prompt “display mode !=02” is available.) Picture height (Pixel) |
| Image number | 1 | B |  | (At present only prompt “display mode =02”or “display mode =03” is available.) 0<=number<=1 |
| Current Pack Number | 2 | B |  | (At present only prompt “display mode !=03” is available.) The number of current package |
| Subsequent Pack Number | 2 | B |  | (At present only prompt “display mode !=03” is available.) The number of subsequent package |
| Data | <=2048 | B |  | (At present only prompt “display mode !=03” is available.) Picture data(Each package is fixed to 1024bytes except the last package whose length is the actual length) |
| timeout | 2 | B |  | (At present only prompt “display mode =03” is available.) (hex, big endian) |

Response

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “7b” |  |
| Separator | 1 | A |  |  |
| Response Code | 2 | A |  | “00” Good “01” Invalid parameter “02” Beyond screen range “03” File operate failed “04” General error |

  - 显示中文 (0x38 0x34)

Request

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “84” |  |
| Separator | 1 | B |  |  |
| mode | 1 | B |  | 0:using show info 1:custom display |
| Show Information | 1 | B |  | Only present for mode ‘0’ 0x00: 交易成功 0x01: 交易失败 0x02: 交易超时 0x03: 交易取消 |
| X Coordinates | 1 | B |  |  |
| Y Coordinates | 1 | B |  |  |
| Length of Lattice data | 2 | B |  | Only present for mode ‘1’, Input Data Length (hex, big endian) |
| Lattice data |  | B |  |  |

Response

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “85” |  |
| Separator | 1 | B |  |  |
| Response Code | 2 | B |  | “00” Good “01” par error “02” Lattice data error “03” mode error “04” data error |

e.g.

Send: 02 00 09 38 34 2F 00 00 00 00 00 05 03 2C

Receive: 06 02 00 05 38 35 2F 30 30 03 24

  - 点阵显示 (0x33 0x36)

Request

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “36” |  |
| Separator | 1 | A | “.” |  |
| First Line Len | 1 | B |  | <=16 |
| Second Line Len | 1 | B |  | <=16 |
| Bitmap Data | var | B |  | 第一行点阵数据+换行符0x1C 第二行点阵数据+换行符0x1C |

Response

说明：接收到本指令后密码键盘不会返回响应报文，仅会返回06表示接收到了报文并进行解析处理。

- 卡类指令

  - 非接卡命令说明 (0x48 0x30)

    - 初始化读卡器 (0x48 0x30 0x2F 0x01)

Request message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “H0” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 01 |  |

Response message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “H1” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 01 |  |
| Response Code | 2 | A |  | 00 Good 01 Invalid command parameters 02 Command Failed 03 Reader not configured |

说明：本指令将会初始化非接卡数据结构。在调用其他读卡器功能前本指令至少要被调用过一次，以确保内部数据都设置为了默认的合法值。

e.g.

Send: 02 00 04 48 30 2F 01 03 51

Receive: 06 02 00 06 48 31 2F 01 30 30 03 52

    - 检测读卡器区域 (0x48 0x30 0x2F 0x02)

Request message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “H0” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 02 |  |

Response message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “H1” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 02 |  |
| Response Code | 2 | A |  | 00 Good 01 Invalid command parameters 02 Command Failed 03 Reader not configured |
| Response data | 16 | A |  | Reader Serial Number, right justified and left padded with zeroes |

说明：本指令将检查感应区是否有非接卡，并激活A/B卡。（无法检测M卡）

e.g.

Send: 02 00 04 48 30 2F 02 03 52

Receive: 06 02 00 06 48 31 2F 02 30 32 03 53

    - 激活感应区域 (0x48 0x30 0x2F 0x30)

Request message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “H0” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 30 |  |

Response message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “H1” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 30 |  |
| Response Code | 2 | A |  | 00 Success 01 Invalid parameters 02 Command Failed |

说明：该指令功能与初始化读卡器指令功能相同。

e.g.

Send: 02 00 04 48 30 2F 30 03 60

Receive: 06 02 00 06 48 31 2F 30 30 30 03 63

    - 检测非接卡在位 (0x48 0x30 0x2F 0x31)

Request message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “H0” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 31 |  |
| Timeout | 2 | B |  | Contactless Activation Timeout, in 10ms interval, big endian.(BCD) |

Response message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “H1” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 31 |  |
| Response Code | 2 | A |  | 00 Card present 01Invalid parameters 02 Command Failed 10 Card Not Present |

说明：本指令将检查感应区是否有非接卡，可识别M1卡（目前国内SP100支持判断M1卡在位），仅寻卡。

e.g.

Send: 02 00 06 48 30 2F 31 01 00 03 62

Receive: 06 02 00 06 48 31 2F 31 30 30 03 62

    - 非接卡APDU指令交互 (0x48 0x30 0x2F 0x32)

Request message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “H0” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 32 |  |
| Key Index | 1 | B |  | Used to encrypt the command data. Only support Data Key Type and ID from 129~250. |
| Command length | 2 | B |  | The length of actual command data (without padding) |
| Command Data | var | B |  | Card command (Encrypted by the specified key, padded to multiple of 8 bytes with zero) |

Response message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “H1” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 32 |  |
| Response Code | 2 | A |  | 00 Card present 01 Invalid parameters 02 Decryption Failed 10 Card Not Present |
| Response data length | 2 | B |  | The length of actual response data (without padding) |
| Response Data | var | B |  | Card response (Encrypted by the specified key, padded to multiple of 8 bytes with zero) |

说明：本指令将会发送APDU指令给非接卡（非接卡需先激活）并接收响应。本指令传入的参数为APDU指令内容，且APDU指令内容经过加密，为密文。

e.g.

Send: 02 00 15 48 30 2F 32 81 00 05 E7 9B F0 7B 4C 4B BB 34 03 88(TDK为16个0x16)

Receive: 06 02 00 24 48 31 2F 32 30 30 00 0A B4 40 24 BB 3D 95 36 4D 8E 58 C8 A0 D7 E9 8D FC 03 00

    - 感应区下电 (0x48 0x30 0x2F 0x33)

This command will power off the field. Response status will always be good (00).

Request message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “H0” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 33 |  |

Response message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “H1” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 33 |  |
| Response Code | 2 | A |  | 00 Success 01 Invalid parameters 02 Command Failed |

说明：本指令将关闭感应区磁场。

e.g.

Send: 02 00 04 48 30 2F 33 03 63

Receive: 06 02 00 06 48 31 2F 33 30 30 03 60

    - LED灯闪烁(0x48 0x30 0x2F 0x34)

This command will make the selected RFID LED blink for the desired interval.

Request message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “H0” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 34 |  |
| LED ID | 1 | B |  | 1,2,3 or 4 |
| LED Blink Time | 2 | B |  | Unit: 10ms, big endian.(BCD) |

Response message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “H1” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 34 |  |
| Response Code | 2 | A |  | 00 Success 01 Invalid parameters 02 Command Failed |

说明：本指令将控制设备的LED灯闪烁，但并非持续闪烁，而是亮一段时间后熄灭式的“闪烁”。

e.g.

Send: 02 00 07 48 30 2F 34 01 01 00 03 67

Receive: 06 02 00 06 48 31 2F 34 30 30 03 67

    - 明文APDU指令交互 (0x48 0x30 0x2F 0x35)

Request message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “H0” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 35 |  |
| Command Data | var | B |  | Card command |

Response message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “H1” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 35 |  |
| Response Code | 2 | A |  | 00 Card present 01 Invalid parameters 02 Command Failed 10 Card Not Present |
| Response Data | var | B |  | Card response |

说明：本指令功能与密文APDU指令交互指令功能相同，但发送的APDU指令将不经过加密。

e.g.

Send: 02 00 09 48 30 2F 35 00 84 00 00 08 03 E4

Receive: 06 02 00 16 48 31 2F 35 30 30 22 9F FC 19 BE 0D 04 35 90 00 03 3C

    - 检测Felica卡在位 (0x48 0x30 0x2F 0x36)

This command will check to see if there is a felica card present in the field.

The data for this command will be the timeout value to wait for (LSB first).

Request message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “H0” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 36 |  |
| Timeout | 2 | B |  | Contactless Activation Timeout, in 10ms interval, big endian. |

Response message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “H1” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 36 |  |
| Response Code | 2 | A |  | 00 Card present 01 Invalid parameters 02 Command Failed 10 Card Not Present |
| ID Length | 1 | B |  |  |
| ID | var | B |  |  |

说明：本指令可检测感应区是否有Felica卡，注意Felica必须要用本指令来检测卡在位。

e.g.

Send: 02 00 06 48 30 2F 36 01 00 03 65

Receive: 06 02 00 15 48 31 2F 36 30 30 08 01 2E 4C E1 5C 95 85 BB 03 0B

    - Felica卡APDU指令交互 (0x48 0x30 0x2F 0x37)

Note: This command will send the APDU command to the felica card and return a response.

The data for this command will be the APDU and data to be sent to the felica card, for example the SELECT or READ RECORD command.

Request message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “H0” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 37 |  |
| Command length | 2 | B |  | The length of actual command data |
| Command Data | var | B |  | Card command |

Response message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “H1” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 37 |  |
| Response Code | 2 | A |  | 00 Card present 01 Invalid parameters 02 Command Failed 10 Card Not Present |
| Response Data Length | 2 | B |  | Card response length |
| Response Data | var | B |  | Card response |

说明：本指令将会发送APDU指令给Felica卡并接收应答，Felica无需激活。但要注意发送的APDU指令的格式与其他非接卡不同。

e.g.

Send: 02 00 22 48 30 2F 37 00 10 10 06 01 2E 4C E1 5C 95 85 BB 01 09 00 01 80 00 03 BB

Receive: 06 02 00 37 48 31 2F 37 30 30 00 1D 1D 07 01 2E 4C E1 5C 95 85 BB 00 00 01 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 03 26

    - 多种类卡激活 (0x48 0x30 0x2F 0x40)

Request message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “H0” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 40 |  |
| Type | 1 | B |  | Card type 0x01—ACard 0x02—BCard 0x03—BCard +ACard 0x04—MCard 0x05—MCard +ACard 0x06—MCard +BCard 0x07—MCard +BCard +ACard 0x08—身份证 |
| Mode | 1 | B | 0x26 or 0x52 | A卡、B卡二者均可，无差异 0x26 REQA 0x52 WUPA 建议一般应用使用 |
| Timeout | 2 | B |  | Contactless Activation Timeout, in 1s interval, big endian. |

Response message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “H1” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 40 |  |
| Response Code | 2 | A |  | 00 Card present 01 Invalid parameters 02 防冲突失败 03 选卡失败 06 超时 09 检测到多张卡 10 接口设备初始化异常 |
| Card Type | 1 | B | 0x0A or 0x0B or 0x0C | 0x0A TYPE A卡 0x0B TYPE B卡 0x0C TYPE M1卡 |
| ATQlen | 1 | B |  | 寻卡返回数据长度 |
| ATQ | var | B |  | ATQlen字节，寻卡返回 |
| SNRlen | 1 | B |  | 防冲突返回数据长度 |
| SNR | var | B |  | SNRlen字节，防冲突返回数据 |
| SAKlen | 1 | B |  | 选卡返回数据长度 |
| SAK | Var | B |  | SAKlen字节， 选卡返回数据 |

说明：本指令可寻卡并激活A/B/M1卡片（不能同时激活多张卡片，多卡时会产生多卡冲突），并返回其相应的卡信息。

e.g.

Send: 02 00 08 48 30 2F 40 07 52 00 05 03 4C

Receive:

    - (M1卡)外部密钥认证 (0x48 0x30 0x2F 0x44)

Request message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “H0” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 44 |  |
| Key Mode | 1 | B |  | Key_A :0x60 Key_B :0x61 |
| SNR | 4 | B |  | Serial Number |
| Block ID | 1 | B |  | 0~250 |
| key | 6 | B |  | External key |

Response message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “H1” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 44 |  |
| Response Code | 2 | A |  | 00 Success 01 Invalid parameters 02 Other Error 03 No card |

说明：认证M1卡的某一块数据域。

e.g.

Send: 02 00 16 48 30 2F 44 00 F7 47 6E F0 02 FF FF FF FF FF FF 03 2A

Receive: 06 02 00 06 48 31 2F 44 30 30 03 17

    - (M1卡)读取块数据 (0x48 0x30 0x2F 0x45)

Request message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “H0” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 45 |  |
| Block No | 1 | B | 0~250 |  |

Response message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “H1” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 45 |  |
| Response Code | 2 | A |  | 00 Success 01 Invalid parameters 02 Other Error |
| Data | 16 | B |  |  |

说明：本指令读取16字节的块数据。

e.g.

Send: 02 00 05 48 30 2F 45 02 03 16

Receive: 06 02 00 22 48 31 2F 45 30 30 69 45 23 01 96 BA DC FE 69 45 23 01 02 FD 02 FD 03 3C

    - (M1卡)写入块数据 (0x48 0x30 0x2F 0x46)

Request message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “H0” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 46 |  |
| Block No | 1 | B | 0~250 |  |
| Data | 16 | B |  | data |

Response message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “H1” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 46 |  |
| Response Code | 2 | A |  | 00 Success 01 Invalid parameters 02 Other Error |

说明：本指令写入16字节的块数据。

e.g.

Send: 02 00 21 48 30 2F 46 02 69 45 23 01 96 BA DC FE 69 45 23 01 02 FD 02 FD 03 3F

Receive: 06 02 00 06 48 31 2F 46 30 30 03 15

    - (M1卡)增量操作 (0x48 0x30 0x2F 0x47)

Request message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “H0” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 47 |  |
| Block No | 1 | B | 0~250 |  |
| Value | 4 | B |  | Increment value which have same format with block data |

Response message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “H1” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 47 |  |
| Response Code | 2 | A |  | 00 Success 01 Invalid parameters 02 Other Error |

说明：对M1卡某一块数据进行增量（数据需符合特定格式）。

e.g.

Send: 02 00 09 48 30 2F 47 02 01 00 00 00 03 19

Receive: 06 02 00 06 48 31 2F 47 30 30 03 14

    - (M1卡)减量操作 (0x48 0x30 0x2F 0x48)

Request message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “H0” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 48 |  |
| Block No | 1 | B | 0~250 |  |
| Value | 4 | B |  | Decrement value which have same format with block data |

Response message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “H1” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 48 |  |
| Response Code | 2 | A |  | 00 Success 01 Invalid parameters 02 Other Error |

说明：对M1卡某一块数据进行减量（数据需符合特定格式）。

e.g.

Send: 02 00 09 48 30 2F 48 02 01 00 00 00 03 16

Receive: 06 02 00 06 48 31 2F 48 30 30 03 1B

  - IC卡指令说明 (0x42 0x30)

    - IC卡交互 (0x42 0x30 0x2F 0x00)

Request message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “B0” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 00 |  |
| Key Index | 1 | B |  | Used to encrypt the command data. Only support Data Key Type and ID from 0,129~250. |
| Key Type | 1 | B |  | Only present for Key Index !=0 0 – DES 1 – AES |
| Encrypting key mode | 1 | B |  | Only present for Key Index !=0 0-ECB 1- CBC |
| CBC Initial value | var | B |  | Only present for CBC Modes DES CBC Initial value is 8 bytes. AES CBC Initial value is 16 bytes. |
| Timeout | 2 | B |  | Read IC card Timeout, in 1s interval, big endian. |
| Command length | 2 | B |  | The length of actual command data (without padding) |
| Command Data |  | B |  | Data of instructions (if Key Index !=0 Encrypted by the specified key, padded to multiple of block size with zero) |
| Display Data | 16 | A |  | Display data for line 1 |
| FS | 1 | S | 1Ch |  |
| Display Data | 16 | A |  | Display data for line 2. |
| FS | 1 | S | 1Ch |  |
| Display Data | 16 | A |  | Display data for line 3. |
| FS | 1 | S | 1Ch |  |
| Display Data | 16 | A |  | Display data for line 4. |
| FS | 1 | S | 1Ch |  |

Response message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “B1” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 00 |  |
| Response Code | 2 | A | X | “00” Good “01”times out “02” IC card read error “03” function id error “04” key index error “05” cancel “06” Key type error “07” Encrypting key mode error “08” decrypt error “09” encrypt error “46” cancel |
| Key Index | 1 | B |  |  |
| Data Length | 2 | B |  | Length of data plaintext |
| Encrypted Data Length | 2 | B |  | if Key Index==0, Encrypted Data Length = Data Length |
| Data | var | B |  | If Key Index !=0, the Data is encrypted |

说明：本指令进行完整的IC卡交互流程，包括检测IC卡->IC卡上电->发送IC卡指令(IC卡协议指令)->IC卡下电。

e.g.

Send: 02 00 14 42 30 2F 00 00 00 05 00 05 00 84 00 00 08 03 C6

Receive: 06 02 00 13 42 31 2F 00 30 30 00 00 02 00 02 6D 00 03 21

    - IC卡上电 (0x42 0x30 0x2F 0x01)

Request message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “B0” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 01 |  |
| timeouts | 2 | B |  | s |
| Display Data | 16 | A |  | Display data for line 1 |
| FS | 1 | S | 1Ch |  |
| Display Data | 16 | A |  | Display data for line 2. |
| FS | 1 | S | 1Ch |  |
| Display Data | 16 | A |  | Display data for line 3. |
| FS | 1 | S | 1Ch |  |
| Display Data | 16 | A |  | Display data for line 4. |
| FS | 1 | S | 1Ch |  |

Response message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “B1” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 01 |  |
| Response Code | 2 | A | X | “00” Good “01”times out “02” IC card read error “46” cancel |
| AtrLen | 2 | B |  |  |
| AtrBuf |  | B |  |  |

e.g.

Send: 02 00 06 42 30 2F 01 00 FF 03 A6

Receive: 06 02 00 26 42 31 2F 01 30 30 00 12 3B 6E 00 00 80 31 80 66 B0 84 0C 01 6E 01 83 00 90 00 03 2D

    - IC卡下电 (0x42 0x30 0x2F 0x02)

Request message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “B0” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 02 |  |

Response message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “B1” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 02 |  |
| Response Code | 2 | A | X | “00” Good “01”error |

e.g.

Send: 02 00 04 42 30 2F 02 03 58

Receive: 06 02 00 06 42 31 2F 02 30 30 03 5B

    - IC卡读写 (0x42 0x30 0x2F 0x03)

Request message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “B0” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 03 |  |
| Key Index | 1 | B |  | Used to encrypt the command data. Only support Data Key Type and ID from 0,129~250. |
| Key Type | 1 | B |  | Only present for Key Index !=0 0 – DES 1 – AES |
| Encrypting key mode | 1 | B |  | Only present for Key Index !=0 0- ECB 1- CBC |
| CBC Initial value | var | B |  | Only present for CBC Modes DES CBC Initial value is 8 bytes. AES CBC Initial value is 16 bytes. |
| Command length | 2 | B |  | The length of actual command data (without padding) |
| Command Data | var | B |  | Data of instructions (if key index!=0 Encrypted by the specified key, padded to multiple of block size with zeros) |

Response message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “B1” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 03 |  |
| Response Code | 2 | A | X | “00” Good “01”times out “02” IC card read error “03” function id error “04” key index error “05” cancel “06” Key Type error “07” Encrypting key mode error “08” decrypt error “09” encrypt error |
| Key Index | 1 | B |  |  |
| Data Length | 2 | B |  | Length of data plaintext |
| Encrypted Data Length | 2 | B |  | if Key Index==0, Encrypted Data Length = Data Length |
| Data | var | B |  | If Key Index !=0, the Data is encrypted |

e.g.

Send: 02 00 12 42 30 2F 03 00 00 05 00 84 00 00 08 03 C6

Receive: 06 02 00 13 42 31 2F 03 30 30 00 00 02 00 02 6D 00 03 22

    - IC卡检测 (0x42 0x30 0x2F 0x04)

Request message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “B0” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 04 |  |
| timeouts | 2 | B |  | s |

Response message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “B1” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 04 |  |
| Response Code | 2 | A | X | “00” Good “01”times out “02” IC card read error |

e.g.

Send: 02 00 06 42 30 2F 04 00 FF 03 A3

Receive: 06 02 00 06 42 31 2F 04 30 30 03 5D

    - 设置接触卡上电类型 (0x42 0x30 0x2F 0x05)

Request message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “B0” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 05 |  |
| IC Card Type | 1 | 1 |  | 0x00 - ICTYPE_IC, /**<接触式IC卡*/ 0x01 - ICTYPE_SAM1, /**<SAM1卡*/ 0x02 - ICTYPE_SAM2, /**<SAM2卡*/ 0x03 - ICTYPE_SAM3, /**<SAM3卡*/ 0x04 - ICTYPE_SAM4, /**<SAM4卡*/ 0x05 - ICTYPE_M_1, /**<at24c01 at24c02 at24c04 at24c08 at24c16 at24c32 at24c64*/ 0x06 - ICTYPE_M_2, /**<sle44x2*/ 0x07 - ICTYPE_M_3, /**<sle44x8*/ 0x08 - ICTYPE_M_4, /**<at88sc102*/ 0x09 - ICTYPE_M_5, /**<at88sc1604*/ 0x0A - ICTYPE_M_6, /**<at88sc1608*/ 0x0B - ICTYPE_ISO7816, /**<ISO7816 standard*/ 0x0C - ICTYPE_M_7, /**<at88sc153*/ 0x0D - ICTYPE_M_1_1,/*<at24c01 */ 0x0E - ICTYPE_M_1_2,/*<at24c02 */ 0x0F - ICTYPE_M_1_4,/*<at24c04 */ 0x10 - ICTYPE_M_1_8,/*<at24c08 */ 0x11 - ICTYPE_M_1_16,/*<at24c16 */ 0x12 - ICTYPE_M_1_32,/*<at24c32 */ 0x13 - ICTYPE_M_1_64,/*<at24c64 */ |

Response message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “B1” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 04 |  |
| Response Code | 2 | A |  | 00 – Good 04 – Other Error |

e.g.

Send: 02 00 06 42 30 2F 05 00 FF 03 A3

  - 读磁卡 (0x4a 0x30)

Request

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “J0” |  |
| Separator | 1 | A |  |  |
| Key Index | 1 | B |  | Key Index used for TDK protection 0,129..250. |
| Key Type | 1 | B |  | 0 – DES 1 – AES |
| Encrypting key mode | 1 | B |  | 0-DES_CBC 1-DES_ECB 2-AES |
| Timeout | 2 | B |  | Read Mag card Timeout, in 10ms interval, big endian. |
| Read Track1 | 1 | B |  | 0- Do not read track 1 data 1- Read track 1 data |
| Read Track2 | 1 | B |  | 0- Do not read track 2 data 1- Read track 2 data |
| Read Track3 | 1 | B |  | 0- Do not read track 3 data 1-Read track 3 data |
| CBC Initial value | 8 | B |  | Only present for CBC Encrypting key mode |
| Display Data | 16 | A |  | Display data for line 1 |
| FS | 1 | S | 1Ch |  |
| Display Data | 16 | A |  | Display data for line 2. |
| FS | 1 | S | 1Ch |  |
| Display Data | 16 | A |  | Display data for line 3. |
| FS | 1 | S | 1Ch |  |
| Display Data | 16 | A |  | Display data for line 4. |
| FS | 1 | S | 1Ch |  |

Response

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “J1” |  |
| Separator | 1 | A |  |  |
| Response Code | 2 | A | X | “00” Good “01”times out “02” key mode error “03”Mag read error “04” track2 error “05”track encrypt error “06” track3 error “07” parameter error “08”key index error “46” cancel |
| PAN Length | 1 | B |  | Length of Plain text PAN |
| Cipher Text PAN |  | B |  | If Key Index>0, Cipher Text PAN Length=32 |
| MASKET PAN |  | A |  | Show top 6 and bottom 4 of PAN, MASKET PAN Length= PAN Length |
| Track1Status | 1 | B |  | 0 = Good 1 = Bad 2 = Not present |
| Track2Status | 1 | B |  | 0 = Good 1 = Bad 2 = Not present |
| Track3Status | 1 | B |  | 0 = Good 1 = Bad 2 = Not present |
| Length1 | 2 | B |  | Length1 is the Plaintext Track1 data length, If Track1Status=’1’ ¦¦’2’, Length1=0 |
| Track1 Data |  | B |  | When the Key Index != 0, if Length1 is not the multiple of 16, Track1 Data length = Length1 +16- Length1 %16. When the Key Index == 0, Track1 Data length is equal to Length1. |
| Length2 | 2 | B |  | Length2 is the real Track2 data length, If Track2Status=’1’ ¦¦’2’, Length2=0 |
| Track2 Data |  | A |  | Refer to Track1 Data |
| Length3 | 2 | S |  | Length3 is the real Track3 data length, If Track3Status=’1’ ¦¦’2’, Length3=0 |
| Track3 Data |  | A |  | Refer to Track1 Data |

e.g.

Send: 02 00 11 4A 30 2F 00 00 01 10 00 01 00 00 03 57

Or: 02 00 19 4A 30 2F 04 00 00 10 00 01 00 00 FF FF FF FF FF FF FF FF 03 5A

Receive:

  - 多卡类寻卡(旧称三界面寻卡)

    - 三重寻卡 (0x42 0x42)

Request

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “BB” |  |
| Separator | 1 | A |  |  |
| Detect RF | 1 | B |  | 0x00-OFF 0x01-ON |
| Detect IC | 1 | B |  | 0x00-OFF 0x01-ON |
| Detect Mag | 1 | B |  | 0x00-OFF 0x01-ON |
| Card type | 1 | B |  | Only present for Detect RF = ON 0x00 - TYPE-A 0x01 - TYPE-B 0x02 - TYPE-A¦¦TYPE-B |
| timeouts | 2 | B |  | s |
| Display Data | 16 | A |  | Display data for line 1 |
| FS | 1 | S | 1Ch |  |
| Display Data | 16 | A |  | Display data for line 2. |
| FS | 1 | S | 1Ch |  |
| Display Data | 16 | A |  | Display data for line 3. |
| FS | 1 | S | 1Ch |  |
| Display Data | 16 | A |  | Display data for line 4. |
| FS | 1 | S | 1Ch |  |

Response

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “BC” |  |
| Separator | 1 | A |  |  |
| Response Code | 2 | A | X | “00” Good “01”times out “02” parameters error “05” cancel “55”not support |
| Card type | 1 | B |  | 0x00:RF 0x01:IC 0x02:MAG |
| atqlen | 2 | B |  |  |
| atqbuf |  | B |  | Only present for Card type=00 |
| AtrLen | 2 | B |  |  |
| AtrBuf |  | B |  | Only present for Card type=01 |

e.g.

Send: 02 00 09 42 42 2F 01 00 00 02 00 FF 03 D9

Receive: (RF) 06 02 00 08 42 43 2F 30 30 00 00 00 03 25

Send: 02 00 08 42 42 2F 00 01 00 00 FF 03 DA

Receive: (IC) 06 02 00 23 42 43 2F 30 30 01 00 0F 3B 6B 00 00 86 88 47 48 F4 50 42 4F 43 33 30 03 B8

Send: 02 00 08 42 42 2F 00 00 01 00 FF 03 DA

Receive: (MAG) 06 02 00 06 42 43 2F 30 30 02 03 29

    - 获取磁道数据 (0x42 0x44)

Request

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “BD” |  |
| Separator | 1 | A |  |  |
| Key Index | 1 | B |  | Key Index used for TDK protection 0,129..250. |
| Key Type | 1 | B |  | 0 – DES 1 – AES |
| Encrypting key mode | 1 | B |  | 0- ECB 1- CBC(unsupported) |
| CBC Initial value | 8 | B |  | Only present for CBC Modes DES CBC Initial value is 8 bytes. AES CBC Initial value is 16 bytes. |
| Read Track1 | 1 | B |  | 0- Do not read track 1 data 1- Read track 1 data |
| Read Track2 | 1 | B |  | 0- Do not read track 2 data 1- Read track 2 data |
| Read Track3 | 1 | B |  | 0- Do not read track 3 data 1-Read track 3 data |

Response

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “BE” |  |
| Separator | 1 | A |  |  |
| Response Code | 2 | A | X | “00” Good “01”get track data err “02” key mode error “03”Mag read error “04” track2 error “05”track encrypt error “06” track3 error “07” Bad key index “09” key index error “46” cancel |
| PAN Length | 1 | B |  | Length of Cipher Text PAN |
| Cipher Text PAN |  | B |  |  |
| MASKET PAN |  | A |  | Show top 6 and bottom 4 of PAN |
| Track1Status | 1 | B |  | 0 = Good 1 = Bad 2 = Not present |
| Track2Status | 1 | B |  | 0 = Good 1 = Bad 2 = Not present |
| Track3Status | 1 | B |  | 0 = Good 1 = Bad 2 = Not present |
| Track1 Data Len | 2 | B |  | Length of data plaintext. If Track1Status!=0, Track1 Data Len==0 |
| Encrypted Track1 Data Length | 2 | B |  | (At present only prompt “Key Index !=0” is available.) If Track1Status!=0, Encrypted Track1 Data Length ==0 |
| Track1 Data | var |  |  | If Key Index !=0, the Track1 Data is encrypted |
| Track2 Data Len | 2 | B |  | Length of data plaintext. If Track2Status!=0, Track2 Data Len==0 |
| Encrypted Track2 Data Length | 2 | B |  | (At present only prompt “Key Index !=0” is available.) If Track2Status!=0, Encrypted Track2 Data Length ==0 |
| Track2 Data | var |  |  | If Key Index !=0, the Track2 Data is encrypted |
| Track3 Data Len | 2 | B |  | Length of data plaintext. If Track3Status!=0, Track3 Data Len==0 |
| Encrypted Track3 Data Length | 2 | B |  | (At present only prompt “Key Index !=0” is available.) If Track3Status!=0, Encrypted Track3 Data Length ==0 |
| Track3 Data | var |  |  | If Key Index !=0, the Track3 Data is encrypted |

说明：本指令结合三重寻卡指令使用，调用三重寻卡指令并刷完磁卡后调用本指令读磁卡数据。

e.g.

Send: 02 00 09 42 44 2F 00 00 00 01 00 00 03 22

Or: 02 00 09 42 44 2F 04 00 00 01 00 00 03 26

Receive:

    - 取消三重寻卡 (0x42 0x46)

Request

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “BF” |  |
| Separator | 1 | A |  |  |

e.g.

Send: 02 00 03 42 46 2F 03 2B

Response

说明：接收到本指令后密码键盘不会返回响应报文，仅会返回06表示接收到了报文并进行解析处理。

- PIN输入指令

  - 设置有效输PIN长度 (0x42 0x48)

Request

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “BH” |  |
| Separator | 1 | B |  |  |
| length | 1 | B |  | Length <=12 |
| Length data | var | B |  |  |

Response

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “BI” |  |
| Separator | 1 | B |  |  |
| response | 2 | B |  | “00” Good “01” par error |

说明：Length data内容为指定的有效PIN输入长度。

e.g.

Send: 02 00 16 42 48 2F 0c 00 00 04 04 06 06 08 08 0A 0A 0C 0C 03 3C

Receive: 06 02 00 05 42 49 2F 30 30 03 22

  - 设置PIN输入密码显示行数 (0x64 0x34)

Request

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “d4” |  |
| Separator | 1 | A |  |  |
| Line Number | 1 | B |  | 01~05 |

Response

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “d5” |  |
| Separator | 1 | A |  |  |
| Response Code | 2 | A |  | “00” – Good “01” – Line Number error |

e.g.

Send: 02 00 04 64 34 2F 03 03 7B

Receive: 06 02 00 05 64 35 2F 30 30 03 78

  - 完成PIN输入 (0x42 0x4C)

Request

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “BL” |  |
| Separator | 1 | A |  |  |

说明：本指令可在PIN输入过程中发送，将完成当前PIN输入过程，本指令仅返回仅会返回06表示接收到了报文并进行解析处理。

e.g.

Send: 02 00 03 33 38 2F 03 24

  - 取消PIN输入 (0x33 0x38)

Request

The cancel PIN request tells the PIN pad to reset and prepare for a new request. Any data in the PIN pad is erased, and the PIN pad will not respond until ready. This command will also cancel any other operation that requires user input.

The PIN pad response consists of just the single [ACK] character.

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “38” |  |
| Separator | 1 | A |  |  |

说明：本指令可在PIN输入过程中发送，将取消当前PIN输入过程，本指令仅返回仅会返回06表示接收到了报文并进行解析处理。

e.g.

Send: 02 00 03 33 38 2F 03 24

  - PIN输入 (0x33 0x32)

Request

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “32” |  |
| Separator | 1 | A |  |  |
| Key Index | 1 | B |  | Index of the key used |
| Key Type | 1 | B |  | 0 – Master/Session, 1 - DUKPT 2 – AES,3 - SM4 |
| PAN Key Index | 1 | B |  | Account Key Index used for account number protection, 0,129..250. |
| PAN Key Type | 1 | B |  | Only present for Index:129..250,0 – DES, 1 – AES,2-SM4 |
| PAN Length | 1 | B |  | Only present for Index:129..250,Primary account length* |
| Cipher Text PAN | 16 | B |  | Only present for Index:129..250,Primary account number* |
| Clear Text PAN | ..19 | A |  | Only present for Index:0, Primary account number (ascii)* |
| FS | 1 | S | 1Ch | Only present for Index:0.* |
| Encrypting key mode | 1 | B |  | 0=CBC encrypted 1= ECB encrypted |
| Encrypting Key Length | 1 | B |  | 16 or 24 for Master/Session key, 0 for DUKPT and FIXED keys. |
| PIN Encrypting Key | var | B |  | Encrypted with the key in the specified index. Depending on the encrypting length the length of this field varies. |
| Max. PIN Digits | 1 | B |  | Maximum number of digits to allow for a PIN entry. Valid values 04 – 0C |
| Enter Key Required | 1 | B |  | 0=Enter Key is required to terminate data entry. 1=Enter key not required when max digits entered |
| Keypad Timeout | 1 | B |  | A timeout value between key presses in 1 s units. A value of 00 IS NOT ALLOWED. |
| ISO9564 Format | 1 | B |  | 0 – Format 0, 1 – Format 1 3 – Format 3, 4 – Format 4 5 - SEC_PIN_SM4_1 /*Don't use the main account, fill in 'F' if the password is not enough*/ 6 - SEC_PIN_SM4_2 /*Use the main account filling method 1, and fill in 'F' for the insufficient number of passwords*/ 7 - SEC_PIN_IBM3624_DES 8 - SEC_PIN_IBM3624_SM4 |
| Display Data | 16 | A |  | Display data for line 1 |
| FS | 1 | S | 1Ch |  |
| Display Data | 16 | A |  | Display data for line 2. |
| FS | 1 | S | 1Ch |  |
| Display Data | 16 | A |  | Display data for line 3. |
| FS | 1 | S | 1Ch |  |
| Display Data | 16 | A |  | Display data for line 4. |
| FS | 1 | S | 1Ch |  |

Note: PIN entry requests to use FIXED keys MUST indicate Key Type = 0 (MK/SK), Encrypting Key Length = 0, and MUST NOT include any PIN Encrypting Key data. The PIN Encrypting Key field is inactive, so the Encrypting Key Length field should always be ZERO.

Response

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “33” |  |
| Separator | 1 | A |  |  |
| Response Code | 2 | A | X | “00” Good “02” General Error “04” Cancelled “42” Bad Key Tag “43” Bad Key Index |
| Function Key | 1 | A | X | Returns the key selected. 0 – Enter(default), 1 – F1, 2 – F2, 3 – F3, 4 – F4 5 – Clear, 6 – Cancel Without any function key, choose 0-Enter. |
| PIN Length | 1 | B |  | Number of PIN digits entered by user (00, 04-0C). If a Function Key is pressed, then the PIN length is 00. |
| Encrypted PIN Block | ..16 | B |  | ANSI X9.8 encrypted PIN block. Not sent if PIN length is zero. |
| DUKPT KSN | 10 | B |  | KSN for DUKPT key type. |

e.g.

Send: 02 00 29 33 32 2F 05 00 00 36 32 31 34 38 33 35 39 31 30 31 38 34 32 31 32 1C 01 00 06 00 05 00 03 14

Receive: 06 02 00 15 33 33 2F 30 30 30 06 8D 2D 07 0C E2 E9 8A B0 03 95

  - 脱机PIN输入(0x64 0x38)

Request

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “d8” |  |
| Separator | 1 | A |  |  |
| Method | 1 | B |  | Specifies the method of PIN block protection. 0 - Directly use existing PIN KEY for PIN block encryption 1 - Generate a random PIN key for PIN block encryption (like temporary session key). And the cipher text PIN key will also be returned under the protection of specified KEK. |
| Key Index | 1 | B |  | Key Index used for PIN protection, 1..250. if method = 0, indicates the PIN key index, If method = 1, indicates the KEK index. |
| Key Type | 1 | B |  | 0 – DES, ISO9564_0 format 1 – AES, ISO9564_4 format |
| PAN Length | 1 | B |  | Length of surrogate PAN |
| Surrogate PAN | ..19 | A |  | Surrogate primary account number (ASCII) |
| Max. PIN Digits | 1 | B |  | Maximum number of digits to allow for a PIN entry. Valid values 04 – 0C |
| Enter Key Required | 1 | B |  | 0=Enter Key is required to terminate data entry. 1=Enter key not required when max digits entered |
| Keypad Timeout | 1 | B |  | A timeout value between key presses in 1 s units. A value of 00 IS NOT ALLOWED. |
| Display Data | 16 | A |  | Display data for line 1 |
| FS | 1 | S | 1Ch |  |
| Display Data | 16 | A |  | Display data for line 2. |
| FS | 1 | S | 1Ch |  |
| Display Data | 16 | A |  | Display data for line 3. |
| FS | 1 | S | 1Ch |  |
| Display Data | 16 | A |  | Display data for line 4. |
| FS | 1 | S | 1Ch |  |

Response

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “d9” |  |
| Separator | 1 | A |  |  |
| Response Code | 2 | A | X | “00” Good “02” General Error “04” Cancelled “42” Bad Key Tag “43” Bad Key Index |
| Function Key | 1 | A | X | Returns the key selected. 0 – Enter(default), 1 – F1, 2 – F2, 3 – F3, 4 – F4 5 – Clear, 6 – Cancel Without any function key, choose 0-Enter. |
| PIN Length | 1 | B |  | Number of PIN digits entered by user (00, 04-0C). If a Function Key is pressed, then the PIN length is 00. |
| Encrypted PIN Block | ..16 | B |  | ANSI X9.8 encrypted PIN block. Not sent if PIN length is zero. PINBlock Length : DES:8 Byte;AES:16 Byte |
| PIN Key Length | 1 | B |  | Length of PIN encryption key. Zero if “Method” is “0”. |
| PIN Key | Var | B |  | Cipher text PIN key encrypted by the specified KEK. |

说明：若通过随机生成TPK密钥的方式计算PINblock，则使用的TMK应是32字节AES密钥或24字节DES密钥。

e.g.

Send: 02 00 26 64 38 2F 00 05 00 10 36 32 31 34 38 33 35 39 31 30 31 38 34 32 31 32 0C 00 05 03 41

Receive: 06 02 00 16 64 39 2F 30 30 30 06 8D 2D 07 0C E2 E9 8A B0 00 03 CB

  - 敏感信息输入 (0x36 0x65)

Request

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “6e” |  |
| Separator | 1 | A |  |  |
| Prompt ID | 1 | B |  | - .250, used to reference the secure prompt (At present only prompt ID “1” is available.): 1 - “Phone Number” entry. |
| Data Key Index | 1 | B |  | Key Index used for data protection, 129..250. |
| Data Key Type | 1 | B |  | 0 – DES 1 – AES |
| Input Mode | 1 | B |  | 0 - Only digits 1 - Hexadecimal characters 2 - All characters |
| Min. | 1 | B |  | Minimum number of characters ( > = 1) |
| Max. | 1 | B |  | Maximum number of characters to allow for entry ( > = Min, < = 32) |
| Keypad Timeout | 1 | B |  | A timeout value between key presses in 1 s units ( < = 90). A value of 00 IS NOT ALLOWED. |

Note: Sensitive Data entry requests to use a data encryption only key which is in slot 129..250. The encryption mode is ECB with zero padding.

Response

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “6f” |  |
| Separator | 1 | A |  |  |
| Response Code | 2 | A | X | “00” Good “02” General Error “04” Cancelled “05” Invalid Parameter “43” Bad Key Index (Not exist) |
| Data Length | 1 | B |  | Number of real character entered by user (00..32). |
| Encrypted Data Length | 1 | B |  | Encrypted data length. Multiple of cipher block size (DES - 8, AES - 16). |
| Encrypted Data | ..32 | B |  | Encrypted data with zero padding. Not sent if Data length is zero. |

e.g.

Send: 02 00 10 36 65 2F 01 81 00 00 04 0C 05 03 E2

Receive: 06 02 00 15 36 66 2F 30 30 07 08 EE A7 37 D2 DD 96 22 27 03 84

  - 获取键值 (0x36 0x67)

Request

The Get Key Code request can get the value of the pressed key when PinEntry or Extended PIN Entry or Sensitive Data Entry Request is being executed. When it was executed alone, [ACK] would be the only response code.

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “6g” |  |
| Separator | 1 | A |  |  |
| timeout | 1 | B |  | Between 0 to 255 (unit: s) |

Note: When this request was executing during PinEntry or Extended PIN Entry or Sensitive Data Entry Request being executed, to get the final status, you need to set timeout parameter so that the three requests would wait for GetKeyCode request to get the final status.

Response

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “6h” |  |
| Separator | 1 | A |  |  |
| Data Length | 1 | B | X | The actual data length(maybe several keys have been pressed), between 1 and 16 |
| Data | 16 | B | X | PIN Entry or Extended PIN Entry: Cipher key value—0x2A; Backspace—0x08; Cancel—0x1B; Enter—0x0D; No key pressed—0x1F Sensitive Data Entry: Backspace—0x08; Cancel—0x1B; Enter—0x0D; Timeout—0x70; 0~9—0x30~0x39 The actual data would be extended to 16 bytes by zero padding. When executing Sensitive Data Entry request the data would be encrypted by using the key in Sensitive Data Entry request. |

  - 脱机PIN验证 (0x64 0x41)

Request

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “dA” |  |
| Separator | 1 | A |  |  |
| PinMaxLen | 1 | B |  | 0x04-0x0C |
| Timeout | 2 |  |  |  |
| PKey ModulusLen | 1 | B |  | 若该长度传0则认为是明文验证 |
| PKey Modulus | var |  |  |  |
| PKey ExponentLen | 1 | B |  | 需要固定传3 |
| PKey Exponent | var |  |  |  |
| textLen | 1 | B |  |  |
| text | var |  |  |  |

Response

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “dB” |  |
| Separator | 1 | A |  |  |
| Response Code | 2 | A |  | 00 – Good 01 – Parameter Error 03 – Timeout 04 – Other Error(maybe PKey Parameter is invalid) 05 – Cancel 45 – Command Len Error |
| sw1sw2 | 2 |  |  | APDU Reponse Code, which only exists when the former Response Code is “00” |

- 安全相关指令

  - 密钥装载 (0x35 0x30)

Request

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “50” |  |
| Separator | 1 | A |  |  |
| KEK Index | 1 | B |  | KEK used to protect the block, 1..250 |
| Block Format | 1 | B |  | 0 - TR31 block, 1 - AESK,2 - DES block,3-SM4, 5-DES->SM4(目前仅ME51) |
| AES Key Length | 1 | B |  | Only present for AESK,16 or 24 or 32 |
| Key Type | 1 | B |  | 0 - PIN, 1- MAC, 2 – Data, 03-Master key |
| Load Key Index | 1 | B |  | Index to be loaded with the incoming key, 1..250 |
| Block Length | 2 | B |  | Length of the block, big endian |
| Block Data | … | B |  | Block data |
| Key Check Value | 3 | B |  | Key Check Value (Left 3 digits of the result of encrypting zeros with the key).* |

Test:

Send: 02 00 25 35 30 2F 00 02 03 01 00 10 31 31 31 31 31 31 31 31 31 31 31 31 31 31 31 31 03 1C

Response

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “51” |  |
| Separator | 1 | A |  |  |
| Response Code | 2 | A |  | “00” Good “02” General error “43” Bad key index “45” Bad data length “46” Invalid block |

  - 获取密钥KCV (0x35 0x32)

Request

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “52” |  |
| Separator | 1 | A |  |  |
| Key Index | 1 | B |  | Key index, 1…250 |
| Key Type | 1 | B |  | 0- DES KEK, 1 - DES PIN, 2 - DES MAC, 3 - DES DATA, 4- AES KEK, 5 - AES PIN, 6 - AES MAC, 7 - AES DATA ,8-SM4 TMK,9-SM4 PIN,10-SM4 MAC,11-SM4 DATA |

Test:

Send: 02 00 05 35 32 2F 01 08 03 27

Response

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “53” |  |
| Separator | 1 | A |  |  |
| Key Index | 1 | B |  | Echoed from request |
| Key Check Value | 3 or 5 | B |  | Key Check Value Iran version: always the left 3 digits of the result of encrypting zeros with the key Others version： (des：Left 3 digits of the result of encrypting zeros with the key Sm4/des：Left 3 digits of the result of encrypting zeros with the key Aes：Left 5 digits of the result of encrypting zeros with the key). |

  - 获取密钥KCV(新) (0x35 0x41)

Request

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “5A” |  |
| Separator | 1 | A |  |  |
| Key Index | 1 | B |  | Key index, 1…250 |
| Key Type | 1 | B |  | 0x00- DES KEK, 0x01 - DES PIN, 0x02 - DES MAC, 0x03 - DES DATA, 0x04- AES KEK, 0x05 - AES PIN, 0x06 - AES MAC, 0x07 - AES DATA , 0x08-SM4 TMK, 0x09-SM4 PIN, 0x0A-SM4 MAC, 0x0B-SM4 DATA |

Response

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “5B” |  |
| Separator | 1 | A |  |  |
| Response Code | 2 | A |  | “00” Good “01” Parameter Error “43” Key Doesn’t Exist “45” Command Length Error |
| KCV Len | 1 | B |  |  |
| Key Check Value | 3 or 4 or 5 | B |  | Key Check Value Iran version: always the left 3 bytes of the result of encrypting zeros with the key Others version： SM4/DES：Left 3 bytes (or 4 bytes) of the result of encrypting zeros with the key. AES：Left 5 bytes of the result of encrypting zeros with the key. |

说明：本指令为满足银商需求开发，后续会统一导入到国内指令集应用，如无特殊需求可继续使用原获取密钥KCV的指令。

Test:

Send: 02 00 05 35 41 2F 01 08 03 54

  - DES数据加解密(0x38 0x30)

Request

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “80” |  |
| Separator | 1 | A |  |  |
| key index | 1 | B |  | Key-encrypting key index, 1..250 |
| Mode | 1 | B |  | 1 = CBC Encryption 2 = ECB Encryption 3 = CBC Decryption 4 = ECB Decryption |
| Input Data Length | 2 | B |  | Input Data Length (hex, big endian). Always a multiple of 8 bytes |
| Input Data | var | B |  | Input Data to DES Encryption Algorithm |
| Encrypting key mode | 1 | B |  | 0=CBC encrypted 1= ECB encrypted |
| Encrypting Key Length | 1 | B |  | 16 or 24 for Master/Session Keys. 0 for FIXED keys. |
| Data Encrypting Session Key | var | B |  | Encrypted with the key in the specified index. |
| CBC Initial value | 8 | B |  | Only present for CBC Modes |

Note: Data processing requests to use FIXED keys MUST indicate Encrypting Key Length = 0, and MUST NOT include any Data Encrypting Session Key data.  The Encrypting Key field is inactive, so the Encrypting Key Length field should always be ZERO, and the Working Key index should only use ID:1~250.

Response

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “81” |  |
| Separator | 1 | A |  |  |
| key index | 1 | B |  | Echoed from request |
| Response Code | 2 | A |  | “00” Good “02” General error “42” Bad key tag “43” Bad key index “45” Bad data length |
| Encrypted Data Length | 2 | B |  | Encrypted Data Length (big endian). Zero if returned if an error occurs. |
| Encrypted Data |  | B |  | Encrypted Data using specified key. Not present if Encrypted Data Length is zero. |

  - MAC生成 (0x38 0x43)

Request

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “8C” |  |
| Separator | 1 | A |  |  |
| Key Index | 1 | B |  | Key-encrypting key index, 1..250 |
| Key Type | 1 | B |  | 0 – Master/Session, 1 – DUKPT 2 – AES |
| MAC generation mode | 1 | B |  | 0 – X99 1 – X919 2 – ECB 3 – 9606 4 – SM4 5 – AES 6 – SM4_UNIONPAY 7 – X99_CBC 8 – X919_CBC 9 – SM4_CBC |
| First/Next/Last Block Flag | 1 | B |  | 0 – First block. PIN pad will starts with IV=null. 1 – Next Block. 2 – Last Block 3 – Only Block |
| Length of Data | 2 | B |  | hex, big endian. Always a multiple of 8 bytes |
| Data | var. | B |  | Data to be MAC’ed. |
| Encrypting key mode | 1 | B |  | 0=CBC encrypted 1= ECB encrypted |
| MAC Session Key Length | 1 | B |  | 16, 24 for MK/SK. 0 for DUKPT and FIXED keys. |
| MAC session Key | var | B |  |  |
| Initial Vector | var | B |  | Only presented for MAC generation mode with suffix “CBC”. The length would be 8 for X99_CBC and X919_CBC, 16 for SM4_CBC |

Notes:

- Use Block Flag set to “Only Block” for data size less than the maximum length allowed. This minimises the command exchange with the PIN pad.

- The data field is optional for messages with Block Flag set to “Last Block”.

- If the MAC key is not present when the First Flag is set the Key referenced by the Key Index is used in the MAC Generation until the Last Flag is set.

- If the MAC key is present when the First Flag is set the derived Working/Session Key is used until the Last Flag is set, even for subsequent data blocks when the MAC Key is not required to be present any more.

- The caller is in charge of padding the incoming data to a 64bit boundary.

Response

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “8D” |  |
| Separator | 1 | A |  |  |
| key index | 1 | B |  | Echoed from request |
| Response Code | 2 | A |  | “00” Good “01” Parameter error “02” General error “04” Invalid command sequence “42” Bad key usage “43” Bad key index “45” Bad data length |
| DES MAC | 8 | B |  | Message Authentication Code (This field is not present if an error occurs.) This field contains the 8 bytes of the calculated MAC. Only returned for the DES/ DUKPT last/only block. |
| AES MAC | 16 | B |  | Message Authentication Code (This field is not present if an error occurs.) This field contains the 16 bytes of the calculated MAC. Only returned for the AES last/only block. |
| DUKPT KSN | 10 | B |  | Current KSN for the selected DUKPT key. Only returned for the DUKPT last/only block. |

  - 装载DUKPT密钥 (0x37 0x63)

Request

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “7c” |  |
| Separator | 1 | A |  |  |
| KEK index | 1 | B |  | Key-encrypting key index ,0…250 |
| Group Index | 1 | B |  | key used to protect the block, 1..250 |
| Block format | 1 |  |  | 0-tr31,1-dukpt |
| Block Length | 2 | B |  | Length of the block, big endian (when format is DUKPT, length must equal to 16） |
| Block Data | … | B |  | Block data |
| KSN | 10 |  |  | Key serial number (When format is DUKPT, KSN is used. Otherwise KSN is in the block data) |

Response

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “7d” |  |
| Separator | 1 | A |  |  |
| Response Code | 2 | A |  | “00” Good “01” Invalid parameter “02” Data length error “03” General error “04” key index error |

  - 数据加解密(DUKPT) (0x37 0x65)

Request

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “7e” |  |
| Separator | 1 | B |  |  |
| group index | 1 | B |  | key used to protect the block, 1..250 |
| Key type | 1 | B |  | 4=TDK (Indicates which key type is used to encrypt/decrypt data) |
| Mode | 1 | B |  | 1 = CBC Encryption 2 = ECB Encryption 3 = CBC Decryption 4 = ECB Decryption |
| Input Data Length | 2 | B |  | Input Data Length (hex, big endian). Always a multiple of 8 bytes |
| Input Data | var | B |  | Input Data |
| CBC Initial value | 8 | B |  | CBC init val(only present CBC Mode) |

Response

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “7f” |  |
| Separator | 1 | A |  |  |
| Response Code | 2 | A |  | “00” Good “01” Bad key index “02” General error “03” Invalid param error “04” data length error |
| Encrypted Data Length | 2 | B |  | Encrypted Data Length (big endian). Zero returned if an error occurs. |
| Encrypted Data |  | B |  | Encrypted Data using specified key. Not present if Encrypted Data Length is zero. |
| KSN | 10 | B |  | Key serial number. Not present when an error occurs |

  - 获取DUKPT密钥的KSN (0x33 0x34)

Request

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “34” |  |
| Separator | 1 | A |  |  |
| Key Index | 1 | B |  | Index of the DUKPT key |

Response

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “35” |  |
| Separator | 1 | A |  |  |
| Response Code | 2 | A |  | “00” Good “01” Bad key index “02” General error |
| DUKPT KSN | 10 | B |  | Current KSN for the corresponding key |

  - KSN增加 (0x37 0x67)

Request

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “7g” |  |
| Separator | 1 | A |  |  |
| group index | 1 | B |  | Index of the dukpt key 1..250 |

Response

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “7h” |  |
| Separator | 1 | A |  |  |
| Response Code | 2 | A |  | “00” Good “01” Bad key index “02” General error “03” Invalid param error “04” Data len error |

  - AES数据加解密(0x38 0x32)

Request

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “82” |  |
| Separator | 1 | A |  |  |
| key index | 1 | B |  | Key-encrypting key index, 1..250 |
| Mode | 1 | B |  | 1 = AES Encryption 2 = AES Decryption |
| Input Data Length | 2 | B |  | Input Data Length (hex, big endian). Always a multiple of 16 bytes |
| Input Data | var | B |  | Input Data to AES Encryption Algorithm |
| Encrypting Key Length | 1 | B |  | 0 for FIXED keys. |

Response

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “83” |  |
| Separator | 1 | A |  |  |
| key index | 1 | B |  | Echoed from request |
| Response Code | 2 | A |  | “00” Good “02” General error “43” Bad key index “45” Bad data length |
| Encrypted Data Length | 2 | B |  | Encrypted Data Length (big endian). Zero if returned if an error occurs. |
| Encrypted Data |  | B |  | Encrypted Data using specified key. Not present if Encrypted Data Length is zero. |

  - SM4数据加解密(0x37 0x6d)

Request

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “7m” |  |
| Separator | 1 | A |  |  |
| key index | 1 | B |  | Key-encrypting key index, 1..250, 255-SNK |
| Mode | 1 | B |  | 1 = CBC Encryption 2 = ECB Encryption 3 = CBC Decryption 4 = ECB Decryption |
| Input Data Length | 2 | B |  | Input Data Length (hex, big endian). Always a multiple of 16 bytes |
| Input Data | var | B |  |  |
| CBC Initial value | 16 | B |  | Only present for CBC Modes |

Response

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “7n” |  |
| Separator | 1 | A |  |  |
| Response Code | 2 | A |  | “00” Good “01” Bad key index “02” General error “03” Invalid param error “04” Data len error |
| Data Len | 2 | B |  | Encrypted Data Length (big endian). Not present if an error occurs. |
| Data |  | B |  | Encrypted Data using specified key. Not present if an error occurs. |

  - 删除密钥 (0x42 0x4A)

Request

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | ”BJ” |  |
| Separator | 1 | B | “2F” |  |
| Key Index | 1 | B |  | 1~255 |
| Key Type | 1 | B |  | 0 - PIN, 1- MAC, 2 – Data, 03-Master key,04-Dukpt,05-RSA |
| Block Format | 1 | B |  | 0-DES,1-AES |

Response

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “BK” |  |
| Separator | 1 | B | “2F” |  |
| Response Code | 2 | B |  | ‘00’ -success ‘01’ - Bad key index ‘02’ -delete error |

  - 清除密钥 (0x36 0x30)

Request

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “60” |  |
| Separator | 1 | B | “2F” |  |

Response

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “61” |  |
| Separator | 1 | B | “2F” |  |
| Response Code | 2 | B |  | ‘00’ -success ‘01’ - other error |

e.g.

Send: 02 00 03 36 30 2F 03 29

Receive: 06 02 00 05 36 31 2F 30 30 03 2E

  - 生成SM2密钥对 (0x39 0x36)

Request

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “96” |  |
| Separator | 1 | A |  |  |

Note: Randomly generate key pair which would be stored in pin-pad. You could use

02 00 04 39 49 2F 01 03 59 to get public key or

02 00 04 39 49 2F 00 03 58 to get private key.

When you generate new key pair, the old one would be covered.

Response

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “97” |  |
| Separator | 1 | A |  |  |
| Response Code | 2 | A |  | “00”: Success “01”: Parameter Error “02”: Other Error |

Eg：02 00 03 39 36 2F 03 20

  - 装载主控密钥 (0x39 0x38)

Request

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “98” |  |
| Separator | 1 | A |  |  |
| DstKey ID | 1 | B |  | 1~250 |
| Block Format | 1 | B |  | 0: DES 1: SM4 |
| Permutation | 1 | B |  | 0 – C1C2C3 1 – C1C3C2 (Default) |
| Cipher Key Length | 1 | B |  |  |
| Cipher Key | var |  |  |  |

Note: The original key would be turned into hex-string which means 16 bytes key would be turned into 32 bytes (0x11->”11”). After encrypting with public key, you could use it as the Cipher Key.

Response

| Message Type | 2 | A | “99” |  |
| --- | --- | --- | --- | --- |
| Separator | 1 | A |  |  |
| Response Code | 2 | A |  | “00” Good “01” Parameter Error “02” Bad Data Length “03” Failed to Get Private Key “04” SM2 Decryption Failed “05” Failed to Load Key |

  - 装载厂商公钥 (0x39 0x41)

Request

| Message Type | 2 | A | “9A” |  |
| --- | --- | --- | --- | --- |
| Separator | 1 | A |  |  |
| Public Key | var |  |  |  |

Note: Load supplier public key into pin-pad. When you load a new supplier public key, the old one would be covered.

Response

| Message Type | 2 | A | “9B” |  |
| --- | --- | --- | --- | --- |
| Separator | 1 | A |  |  |
| Response Code | 1 | A |  | “00” Good “02” Bad Data Length “03” Load Into File Failed |

  - 获取设备公钥 (0x39 0x43)

Note: Before executing this request, make sure that device key pair has been generated successfully and supplier public key has been loaded successfully. The device public key would be encrypted by SM2 algorithm with supplier public key as return value.

| Message Type | 2 | A | “9C” |  |
| --- | --- | --- | --- | --- |
| Separator | 1 | A |  |  |
| Permutation | 1 | B |  | 0 – C1C2C3 1 – C1C3C2 (Default) |

Response

| Message Type | 2 | A | “9D” |  |
| --- | --- | --- | --- | --- |
| Separator | 1 | A |  |  |
| Response Code | 1 | A |  | “00” Good “02” Get Supplier Public Key Failed “03” Get Device Public Key Failed “04” Encryption Failed |
| Cipher Length | 1 | B |  |  |
| Encrypted Device Public Key | 160 | B |  |  |

e.g. 02 00 04 39 43 2F 01 03 53

  - 生成密钥认证信息 (0x39 0x45)

Request

| Message Type | 2 | A | “9E” |  |
| --- | --- | --- | --- | --- |
| Separator | 1 | A |  |  |
| Random Number Length | 1 | B |  | Default: Should be 16 |
| Random Number | var |  |  |  |
| Permutation | 1 | B |  | 0 – C1C2C3 1 – C1C3C2 (Default) |

Note: The Random Number would be encrypted with supplier public key, and a byte (0x04) would be add to the head of the array which stored the Random Number encrypted. Then the hex bytes would be turn into hex string (like 0x11->”11”) which would be the KeyAuthInfo after Base64 transcoding.

Response

| Message Type | 2 | A | “9F” |  |
| --- | --- | --- | --- | --- |
| Separator | 1 | A |  |  |
| Response Code | 2 | A |  | “00” Good “02” Bad Data Length “03” Get Supplier Public Key Failed “04” Encrypt Failed |
| KeyAuthInfo Length | 2 | B |  |  |
| KeyAuthInfo | Var | B |  |  |

  - 获取SM2算法签名信息 (0x39 0x47)

Request

| Message Type | 2 | A | “9G” |  |
| --- | --- | --- | --- | --- |
| Separator | 1 | A |  |  |
| Random Number Length | 1 | B |  | Default: Should be 16 |
| Random Number | var |  |  |  |

Note: First, generate abstract of the Random Number. Then sign the abstract to get the Sign-Info.

Response

| Message Type | 2 | A | “9F” |  |
| --- | --- | --- | --- | --- |
| Separator | 1 | A |  |  |
| Response Code | 1 | A |  | “00” Good “02” Bad Data Length “03” Get Device Key Pair Failed “04” Generate E Failed “05” Generate Signature Failed |
| Sign-Info Length | 2 | B |  |  |
| Sign-Info |  |  |  |  |

  - 数据加解密(银商DES) (0x39 0x4F)

Request

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “9O” |  |
| Separator | 1 | A |  |  |
| key index | 1 | B |  | Key-encrypting key index, 1..128 |
| Key type | 1 | B |  | 1- MAC, 2 – Data |
| Mode | 1 | B |  | 1= CBC Encryption 2= ECB Encryption 3= CBC Decryption 4= ECB Decryption 5 = KEYLEN_8_ECB Encryption 6 = KEYLEN_8_ECB Decryption 7 = KEYLEN_16_ECB Encryption 8 = KEYLEN_16_ECB Decryption 9 = KEYLEN_24_ECB Encryption 0x0A = KEYLEN_24_ECB Decryption 0x0B = KEYLEN_8_CBC Encryption 0x0C = KEYLEN_8_CBC Decryption 0x0D = KEYLEN_16_CBC Encryption 0x0E = KEYLEN_16_CBC Decryption 0x0F = KEYLEN_24_CBC Encryption 0x10 = KEYLEN_24_CBC Decryption |
| Input Data Length | 2 | B |  | Input Data Length (hex, big endian). Always a multiple of 8 bytes |
| Input Data | var | B |  | Input Data to DES Encryption Algorithm |
| Encrypting key mode | 1 | B |  | 0=CBC encrypted 1= ECB encrypted |
| Encrypting Key Length | 1 | B |  | 16 or 24 for Master/Session Keys. 0 for FIXED keys. |
| Data Encrypting Session Key | var | B |  | Encrypted with the key in the specified index. |
| CBC Initial value | 8 | B |  | Only present for CBC Modes |

Note: Data processing requests to use FIXED keys MUST indicate Encrypting Key Length = 0, and MUST NOT include any Data Encrypting Session Key data.  The Encrypting Key field is inactive, so the Encrypting Key Length field should always be ZERO, and the Working Key index should only use ID:1~128.

Response

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “9P” |  |
| Separator | 1 | A |  |  |
| key index | 1 | B |  | Echoed from request |
| Response Code | 2 | A |  | “00” Good “02” General error “42” Bad key tag “43” Bad key index “45” Bad data length |
| Encrypted Data Length | 2 | B |  | Encrypted Data Length (big endian). Zero if returned if an error occurs. |
| Encrypted Data |  | B |  | Encrypted Data using specified key. Not present if Encrypted Data Length is zero. |

  - 数据加解密(银商SM4) (0x39 0x51)

Request

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “9Q” |  |
| Separator | 1 | B |  |  |
| key Index | 1 | B |  | Key-encrypting key index, 1..250 |
| Key type | 1 | B |  | 1- MAC, 2 – Data |
| mode | 2 | B |  | 1 = CBC Encryption 2 = ECB Encryption 3 = CBC Decryption 4 = ECB Decryption |
| Data len | 2 | B |  | Input Data Length (hex, big endian). Always a multiple of 16 bytes |
| Data | var |  |  |  |
| CBC Initial value | 16 | B |  | Only present for CBC Modes |

Response

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “9R” |  |
| Separator | 1 | B |  |  |
| Response Code | 2 | A |  | “00” Good “01” Bad key index “02” General error “03” Invalid param error “04” Data len error |
| Data Len | 2 | B |  | Encrypted Data Length (big endian). Not present if an error occurs. |
| Data |  | B |  | Encrypted Data using specified key. Not present if an error occurs. |

- EMV Command Oerder Set

  - EMV Configuration Command

    - Termial Configuration Commands

      - Update Terminal Configuration Command

This command is used to Update Terminal configuration.

Request message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “L0” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 01 |  |
| Card Interface | 1 | B |  | 0x01 Contact card 0x02 Contactless card |
| TLV Len | 2 | B |  | The length of TLV data |
| TLV List | var | B |  | TLV list of Termial Configuration param |

Response message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “L1” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 01 |  |
| Response Code | 2 | A |  | 00 Good 01 Invalid command parameters 02 Command Failed |

      - Get Terminal Configuration Command

This command is used to get Terminal configuration.

Request message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “L0” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 02 |  |
| Card Interface | 1 | B |  | 0x01 Contact card 0x02 Contactless card |

Response message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “L1” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 02 |  |
| Response Code | 2 | A |  | 00 Good 01 Invalid command parameters 02 Command Failed |
| TLV Len | 2 | B |  | The length of TLV data (if Response Code == 0) |
| TLV List | var | B |  | TLV list of Termial Configuration param (if Response Code == 0) |

    - AID Configuration Commands

      - Update AID Configuration Command

This command is used to Update an AID configuration.

Request message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “L0” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 03 |  |
| Card Interface | 1 | B |  | 0x01 Contact card 0x02 Contactless card |
| TLV Len | 2 | B |  | The length of TLV data |
| TLV List | var | B |  | TLV list of AID param |

Response message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “L1” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 03 |  |
| Response Code | 2 | A |  | 00 Good 01 Invalid command parameters 02 Command Failed |

      - Get AID Configuration Command

This command is used to get the specified AID configuration.

Request message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “L0” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 04 |  |
| Card Interface | 1 | B |  | 0x01 Contact card 0x02 Contactless card |
| AID Entry | 27 | B |  | AID Entry data, see the below |

AID Entry data:

| Data Element Name Format | Format | Length In Bytes |
| --- | --- | --- |
| AID | b | 16 |
| AID length | b | 1 |
| Kernel ID | b | 8 |
| Extern Check Flag: 0x00-default, will no check 0x01-should be matching transaction Type 0x02-should be matching extern String | b | 1 |
| Transaction Type | b | 1 |

Response message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “L1” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 04 |  |
| Response Code | 2 | A |  | 00 Good 01 Invalid command parameters 02 Command Failed |
| TLV Len | 2 | B |  | The length of TLV data (if Response Code == 0) |
| TLV List | var | B |  | TLV list of AID param (if Response Code == 0) |

      - Remove AID Configuration Command

This command is used to Remove the specified AID configuration.

Request message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “L0” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 05 |  |
| Card Interface | 1 | B |  | 0x01 Contact card 0x02 Contactless card |
| AID Entry | 27 | B |  | AID Entry data, see the below |

AID Entry data:

| Data Element Name Format | Format | Length In Bytes |
| --- | --- | --- |
| AID | b | 16 |
| AID length | b | 1 |
| Kernel ID | b | 8 |
| Extern Check Flag: 0x00-default, will no check 0x01-should be matching transaction Type 0x02-should be matching extern String | b | 1 |
| Transaction Type | b | 1 |

Response message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “L1” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 05 |  |
| Response Code | 2 | A |  | 00 Good 01 Invalid command parameters 02 Command Failed |

      - Remove All AID Configuration Command

This command is used to Remove all Terminal/AID configuration.

Request message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “L0” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 06 |  |
| Card Interface | 1 | B |  | 0x01 Contact card 0x02 Contactless card |

Response message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “L1” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 06 |  |
| Response Code | 2 | A |  | 00 Good 01 Invalid command parameters 02 Command Failed |

    - CAPK Management Command

      - Update CAPK Command

This command is used to Update CAPK.

Request message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “L0” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 07 |  |
| PK Modulus | 248 | B |  | PK modulus |
| PK Modulus Len | 1 | B |  | The length of PK modulus |
| PK Exponent | 3 | B |  | PK exponent |
| Hash Value | 20 | B |  | Hash value |
| Expired Date | 4 | B |  | Expired date |
| RID | 5 | B |  | RID |
| Index | 1 | B |  | Index |
| PK Algorithm Indicator | 1 | B |  | PK algorithm indicator |
| Hash Algorithm Indicator | 1 | B |  | Hash algorithm indicator |

Response message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “L1” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 07 |  |
| Response Code | 2 | A |  | 00 Good 01 Invalid command parameters 02 Command Failed |

      - Get CAPK Command

This command is used to Get CAPK.

Request message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “L0” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 08 |  |
| RID | 5 | B |  | RID |
| Index | 1 | B |  | Index |

Response message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “L1” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 08 |  |
| Response Code | 2 | A |  | 00 Good 01 Invalid command parameters 02 Command Failed |
| PK Modulus | 248 | B |  | PK modulus (if Response Code == 0) |
| PK Modulus Len | 1 | B |  | The length of PK modulus (if Response Code == 0) |
| PK Exponent | 3 | B |  | PK exponent (if Response Code == 0) |
| Hash Value | 20 | B |  | Hash value (if Response Code == 0) |
| Expired Date | 4 | B |  | Expired date (if Response Code == 0) |
| RID | 5 | B |  | RID (if Response Code == 0) |
| Index | 1 | B |  | Index (if Response Code == 0) |
| PK Algorithm Indicator | 1 | B |  | PK algorithm indicator (if Response Code == 0) |
| Hash Algorithm Indicator | 1 | B |  | Hash algorithm indicator (if Response Code == 0) |

      - Remove One CAPK Command

This command is used to Remove One CAPK.

Request message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “L0” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 09 |  |
| RID | 5 | B |  | RID |
| Index | 1 | B |  | Index |

Response message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “L1” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 09 |  |
| Response Code | 2 | A |  | 00 Good 01 Invalid command parameters 02 Command Failed |

      - Remove All CAPK Command

This command is used to Remove All CAPK.

Request message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “L0” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 0A |  |

Response message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “L1” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 0A |  |
| Response Code | 2 | A |  | 00 Good 01 Invalid command parameters 02 Command Failed |

    - Revocation List Management Command

      - Update Revocation List Command

This command is used to Update certification revocation list.

Request message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “L0” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 0B |  |
| RID | 5 | B |  | RID |
| Index | 1 | B |  | Index |
| CSN | 3 | B |  | CSN |

Response message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “L1” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 0B |  |
| Response Code | 2 | A |  | 00 Good 01 Invalid command parameters 02 Command Failed |

      - Get Revocation List command

This command is used to Get certification revocation list.

Request message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “L0” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 0C |  |
| RID | 5 | B |  | RID |
| Index | 1 | B |  | Index |
| CSN | 3 | B |  | CSN |

Response message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “L1” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 0C |  |
| Response Code | 2 | A |  | 00 Good 01 Invalid command parameters 02 Command Failed |
| Result | 1 | B |  | 0x00 Find 0x01 Not Find |

      - Remove One Revocation List Command

This command is used to Remove One certification revocation list.

Request message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “L0” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 0D |  |
| RID | 5 | B |  | RID |
| Index | 1 | B |  | Index |
| CSN | 3 | B |  | CSN |

Response message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “L1” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 0D |  |
| Response Code | 2 | A |  | 00 Good 01 Invalid command parameters 02 Command Failed |

      - Remove All Revocation List command

This command is used to Remove All certification revocation list.

Request message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “L0” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 0E |  |

Response message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “L1” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 0E |  |
| Response Code | 2 | A |  | 00 Good 01 Invalid command parameters 02 Command Failed |

    - Exception List Management Command

      - Update Exception List Command

This command is used to Update exception list.

Request message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “L0” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 0F |  |
| PAN | 10 | B |  | PAN |
| PAN Len | 1 | B |  | The length of PAN |
| PAN SN | 1 | B |  | PAN SN |

Response message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “L1” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 0F |  |
| Response Code | 2 | A |  | 00 Good 01 Invalid command parameters 02 Command Failed |

      - Get Exception List Command

This command is used to Get exception list.

Request message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “L0” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 10 |  |
| PAN | 10 | B |  | PAN |
| PAN Len | 1 | B |  | The length of PAN |
| PAN SN | 1 | B |  | PAN SN |

Response message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “L1” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 10 |  |
| Response Code | 2 | A |  | 00 Good 01 Invalid command parameters 02 Command Failed |
| Result | 1 | B |  | 0x00 Find 0x01 Not Find |

      - Remove One Exception List Command

This command is used to Remove One exception list.

Request message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “L0” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 11 |  |
| PAN | 10 | B |  | PAN |
| PAN Len | 1 | B |  | The length of PAN |
| PAN SN | 1 | B |  | PAN SN |

Response message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “L1” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 11 |  |
| Response Code | 2 | A |  | 00 Good 01 Invalid command parameters 02 Command Failed |

      - Remove All Exception List Command

This command is used to Remove All exception list.

Request message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “L0” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 12 |  |

Response message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “L1” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 12 |  |
| Response Code | 2 | A |  | 00 Good 01 Invalid command parameters 02 Command Failed |

    - GET AID/CAPK NUM

      - Get Aid Num

This command is used to Update certification revocation list.

Request message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “L0” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 13 |  |
| Card Interface | 1 | B |  | 0x01 Contact card 0x02 Contactless card |

Response message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “L1” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 13 |  |
| Response Code | 2 | A |  | 00 Good 01 Invalid command parameters 02 Command Failed |
| AID NUM | 2 | B |  | If (Response Code == 0) |
| AID TLV LEN | 2 | B |  | The length of AID tlv |
| TLV aid | Var | B |  | Primary Account Number Tag: 9F06 Format: cn (up to 19) |

Note：if AID NUM>1 ,command see below.
command  example:

Message Type +Separator+Func ID + Response Code +AID NUM +  AID  1 Length+ aid 1 Tlv string + AID 2 Length + aid 2 Tlv string  ....

      - Get CAPK Num

This command is used to Update certification revocation list.

Request message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “L0” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 14 |  |

Response message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “L1” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 14 |  |
| Response Code | 2 | A |  | 00 Good 01 Invalid command parameters 02 Command Failed |
| CAPK NUM | 2 | B |  | If (Response Code == 0) |
| RID | 5 | B |  |  |
| INDEX | 1 | B |  |  |

Note：if CAPK NUM>1 ,command see below.
command  example:

Message Type +Separator+Func ID + Response Code +CAPK NUM +  rid 1 + index 1  + rid 2 + index 2  ....

  - Function Command

    - Send Debug Message Command

Request message

NULL

Response message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “L1” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 21 |  |
| Response Code | 2 | A |  | 00 Good 01 Invalid command parameters 02 Command Failed |
| Debug message data Length | 2 | B |  | Debug message data length |
| Debug message data | Var |  |  | Debug message data |

    - Init EMV Command

This Command initializes Newland Level3 module, set configuration, set callback function, set file path and configuration bitmap to kernel.

Request message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “L0” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 22 |  |
| Config | 8 | B |  | byte 1 According to the demand: Configuration bitmap definitions: L3_CFG_SUPPORT_EC (0x0001):Support pboc election currency transaction L3_CFG_SUPPORT_SM (0x0002) :Support sm algorithm L3_CFG_SUPPORT_EXTERNAL_READER(0x0004):Support external contactless reader Byte 2-byte 8 Rfu |

Response message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “L1” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 22 |  |
| Response Code | 2 | A |  | 00 Good 01 Invalid command parameters 02 Command Failed |

    - Set Data Command

This command is used to set a TLV data by the tag name

Request message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “L0” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 23 |  |
| TLV data name | 4 | B |  | TLV data name |
| TLV Data value length | 2 | B |  | TLV data value Length |
| TLV Data value | var | B |  | TLV data value |

Response message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “L1” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 23 |  |
| Response Code | 2 | A |  | 00 Success 01 Invalid parameters 02 Command Failed |

    - Get Data Command

This command uses to get the EMVL3 data according L3_DATA type.

Request message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “L0” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 24 |  |
| Key Index | 1 | B |  | Used to encrypt the command data. Only support Data Key Type and ID from 129~250. |
| L3_DATA type | 4 | B |  | TLV data name or data type, see the below |
| Max length | 2 | B |  | The max tag values. |

L3_DATA type

| Data Item | Description |
| --- | --- |
| L3_DATA_PAN | Application Primary Account Number (PAN) Format: ANS Length: var. up to 19 |
| L3_DATA_TRACK1 | Track 1 Data Format: ANS Length: var. up to 100 |
| L3_DATA_TRACK2 | Track 2 Data Format: ANS Length: var. up to 37 |
| L3_DATA_TRACK3 | Track 3 Data Format: ANS Length: var. up to 104 |
| L3_DATA_DD_CARD_TRACK1 | If Track 1 Data is present, then DD Card (Track1) contains a copy of the discretionary data field of Track 1 Data as returned by the Card in the file read using the READ RECORD command during a mag-stripe mode transaction (i.e. without Unpredictable Number (Numeric), Application Transaction Counter, CVC3 (Track1) and nUN included) Format: ANS Length :var. up to 56 |
| L3_DATA_DD_CARD_TRACK2 | DD Card (Track2) contains a copy of the discretionary data field of Track 2 Data as returned by the Card in the file read using the READ RECORD command during a mag-stripe mode transaction (i.e. without Unpredictable Number (Numeric), Application Transaction Counter, CVC3 (Track2) and nUN included). Format: CN Length: var. up to 11 bytes |
| L3_DATA_EXPIRE_DATE | Application Expiration Date Format: ANS Length: 4 (MSR, YYMM) or 6 (CT/CLSS, YYMMDD) |
| L3_DATA_SERVICE_CODE | Service Code Format: ANS Length: 3(MSR) or 4 (CT/CLSS) |
| L3_DATA_CARDHOLDER_NAME | Cardholder Name Format: ANS Length: var. up to 26 |
| L3_DATA_POS_ENTRY_MODE | POS Entry Mode. Indicates the method by which the PAN was entered Length: 1 Possible values: 0x01 = Entry mode - manual 0x02 = Entry mode - magnetic stripe reader 0x05 = Entry mode - Integrated circuit card 0x07 = Entry Mode - proximity payment using ICC data (i.e. CL EMV) 0x80 = Entry mode - ICC fallback to magnetic stripe |
| L3_DATA_CARD_SCHEME_ID | Card Scheme ID. This data indicates the type of card, not only in terms of the card type but also card association and whether the transaction was an EMV or non-EMV. Length: 1 Possible values: 0x01: MSR transaction 0x02: Contact EMV 0x03: Contact PBOC 0x06: Contactless UnionPay QPBOC 0x08: Contactless UnionPay MSD 0x0B: Contactless Visa QVSDC 0x0C: Contactless Visa VSDC 0x0D: Contactless Visa WAVE2 0x0E: Contactless Visa MSD 0x0F: Contactless Visa MSD Legacy 0x10: Contactless Mastercard Paypass M/Chip 0x11: Contactless Mastercard Paypass Magstripe 0x15: Contactless Discover DPAS EMV 0x16: Contactless Discover DPAS Magstripe 0x17: Contactless Discover DPAS ZIP 0x1A: Contactless Amex EMV 0x1B: Contactless Amex Magstripe 0x1C: Contactless Amex Mobile EMV 0x1D: Contactless Amex Mobile Magstripe 0x1F: Contactless JCB EMV 0x20: Contactless JCB Magstripe 0x21: Contactless JCB Legacy 0x24: Contactless Pure EMV 0x29: Contactless Interac EMV 0x2E: Contactless Rupay EMV 0x33: Contactless Mir EMV 0x38: Contactless Multibanco EMV |
| L3_DATA_SIGNATURE | Signature Flag Length: 1 Possible values: 0x01: request signature 0x00 |
| L3_DATA_ADVISE | Advise Flag, used for Contact EMV Length: 1 Possible values: 0x01: request advise 0x00 |
| L3_DATA_ISSUER_SCRIPT_RESULT | Issuer Script Results. Indicates the result of the terminal script processing Length: up to 90 |
| L3_DATA_ONLINEPIN | OnlinePin Flag Length: 1 Possible values: 0x01: request online pin 0x00 |
| L3_DATA_GENERIC_TLV | Generic TLV encoded data(e.g. 0x5F2A, 0x9C, 0x9F26, 0x57) |

Response message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “L1” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 24 |  |
| Response Code | 2 | A |  | 00 Success 01 Invalid parameters 02 Command Failed |
| Get Tlv Data Status | 1 | B |  | SUCC 0 Fail 1 No Exist 2 |
| The Actual Data Length | 2 | B |  | If (Get Tlv Data Status == 0) The actual length of the data before use CBC to encrypt |
| TLV Value Length | 2 | B |  | If (Get Tlv Data Status == 0) |
| TLV Value | var | B |  | If (Get Tlv Data Status == 0) The value will use CBC to encrypt |

    - Set Tlv List Data Command

This command is used to set one or more TLV data by the tag name

Request message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “L0” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 25 |  |
| TLV value length | 2 | B |  | TLV value Length |
| TLV value | var | B |  | TLV value |

Response message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “L1” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 25 |  |
| Response Code | 2 | A |  | 00 Success 01 Invalid parameters 02 Command Failed |

    - Get Tlv List Data Command

This command uses to get the TLV list data value from current kernel.

Request message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “L0” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 26 |  |
| Key Index | 1 | B |  | Used to encrypt the command data. Only support Data Key Type and ID from 129~250. |
| Tag num | 1 | B |  | The number of tag in the list. |
| Max len | 2 | B |  | The size of tlvData. |
| Control code. | 1 | B |  | bit 1 = 1: Zero Len Tag Allow. |
| Tag list length | 2 | B |  |  |
| Tag list | var | B |  | The tag list which will be packed to tlvData. |

Response message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “L1” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 26 |  |
| Response Code | 2 | A |  | 00 Success 01 Invalid parameters 02 Command Failed |
| Get Tlv Data Status | 1 | B |  | SUCC 0 Fail 1 No Exist 2 |
| The Actual Data Length | 2 | B |  | If (Get Tlv Data Status == 0) The actual length of the data before use CBC to encrypt |
| Tlv list value length | 2 | B |  | If (Get Tlv Data Status == 0) |
| Tlv list value | var | B |  | If (Get Tlv Data Status == 0) The value will use CBC to encrypt. |

    - Set Debug Mode Command

This command uses to set the L2 kernel Debug mode.

Request message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “L0” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 27 |  |
| Debug LV | 1 | B |  | LV_CLOSE 0x00:close debug LV_DEBUG 0x01:log with normal debug informaiton, LV_ALL 0x03:log with all of the debug information |

Response message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “L1” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 27 |  |
| Response Code | 2 | A |  | 00 Success 01 Invalid parameters 02 Command Failed |

    - Get Version Command

This command uses to get the kernel version

Request message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “L0” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 28 |  |
| L3_MODULE | 1 | B |  | typedef enum{ L3_MODULE_API, L3_MODULE_EMV, L3_MODULE_EP, L3_MODULE_QPBOC, L3_MODULE_PAYPASS, L3_MODULE_PAYWAVE, L3_MODULE_EXPRESSPAY, L3_MODULE_DPAS, L3_MODULE_JCB, L3_MODULE_PURE, L3_MODULE_RUPAY, L3_MODULE_INTERAC, L3_MODULE_MIR, L3_MDDULE_MULTIBANCO, }L3_MODULE; |

Response message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “L1” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 28 |  |
| Response Code | 2 | A |  | 00 Success 01 Invalid parameters 02 Command Failed |
| Version length | 1 | B |  | The length of kernel version |
| Version | var | A |  | Kernel version |

    - CALLBACK DEAL Management Command

      - L3_CALLBACK_Notification

L3_CALLBACK_Notification are one-way messages that are sent out by the SP100 to notify the upper computer of various events that occur in the SP100. L3_CALLBACK_Notification may be of different types (such as UI Events, command acknowledgements, etc.) .The command see below.in this command you can not use set/get data command.

Request message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “L1” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 36 |  |
| L3_Callback | 1 | B | 1 | L3_CALLBACK_UI_EVENT |
| uiEventID | 1 | B |  | typedef enum { 0:UI_PRESENT_CARD, 1:UI_PROCESSING, 2:UI_CAPK_LOAD_FAIL, 3:UI_SEE_PHONE, 6:UI_PIN_STATUS, } L3_UIID; see below |
| uiEventData | 2 | B |  | The length of uiEventdata |
| uiEventData | var | B |  | UI event data or NULL. see below |

Response message

NULL

7.2.9.1.1  L3_UIID

| UI Event ID | UI Event Data | Description |
| --- | --- | --- |
| UI_PRESENT_CARD | value: L3_UI_CARD length: 1 | Reader is ready to read a card, the first byte in ‘UI Event Data’ indicate the pecified event [L3_UI_CARD] see below |
| UI_PROCESSING | NULL | The Reader is processing the transaction. Suggested User Interface Indication: LCD Display: ‘Processing…’ |
| UI_CAPK_LOAD_FAIL | NULL | Load CAPK fails, checksum error. Suggested User Interface Indication: LCD Display: Special CAPK is wrong |
| UI_SEE_PHONE | NULL | The cardholder must interact with their mobile device to complete the transaction. Suggested User Interface Indication: LCD Display: ‘See Phone for Instructions’ Buzzer: Alert Tone (optional) |
| UI_PIN_STATUS | Value: L3_UI_PIN length: var | UI event data include:PIN_ENTRY_STATUS,PINBLOCK. PIN_ENTRY_STATUS: LENGTH 4 0:L3_ERR_SUCC -501:L3_ERR_FAIL -502:L3_ERR_CANCLE -503:L3_ERR_TIMEOUT -508:L3_ERR_BYPASS PINBLOCK: Tag: 1F8155 ANSI X9.8 encrypted PIN block. If not enter pinblock is all 0,TLV Length is 0; Tag: 1F8153 KSN for DUKPT key type. PINBLOCK add after PIN_ENTRY_STATU by TLV module.such as PIN_ENTRY_STATUS +TLV Length(2 bit) + TLV. |

7.2.9.1.2  L3_UI_CARD

| UI Event ID | Description |
| --- | --- |
| UI_KEYIN | Reserved |
| UI_STRIPE | Reader is ready to read a card on the magstripe swipe interface. Suggested User Interface Indication: LCD Display: "Swipe Card", Amount |
| UI_INSERT | Reader is ready to read a card on the contact interface. Suggested User Interface Indication: LCD Display: "Insert Card", |
| UI_TAP | Reader is ready to read a card on the contactless interface. Suggested User Interface Indication: LCD Display: "Tap Card", Amount |
| UI_INSERTC_TAP | Reader is ready to read a card on the contact or contactless interface. Suggested User Interface Indication: LCD Display: "Tap or Insert a Card", Amount |
| UI_STRIPE_INSERT | Reader is ready to read a card on the contact or magstripe interface. Suggested User Interface Indication: LCD Display: "Insert or Swipe a Card", Amount |
| UI_STRIPE_TAP | Reader is ready to read a card on the contactless or magstripe interface. Suggested User Interface Indication: LCD Display: "Tap or Swipe a Card", Amount |
| UI_STRIPE_INSERT_TAP | Reader is ready to read a card on any of the three interfaces i.e. contactless, contact or magstripe. Suggested User Interface Indication: LCD Display: "Present Card", Amount |
| UI_PRESENTCARD_AGAIN | The user must present the card again to continue the transaction. Suggested User Interface Indication: LCD Display: "Present Card Again", Amount |
| UI_USE_CHIP | The user swipe a CHIP card though MSR, we should indicate to the user that this card has chip. Suggested User Interface Indication: LCD Display: ”Please Use Chip” |
| UI_FALLBACK_CT | A fallback from Contact (Chip) to MSR occurred and the Reader is ready to read a card on the magstripe swipe interface. Suggested User Interface Indication: LCD Display: "Chip Error - Swipe Card", Amount Buzzer: Alert Tone |
| UI_FALLBACK_CLSS | A fallback from Contactless to Contact/MSR occurred and the Reader is ready to read a card on the contact or magstripe swipe interface. Suggested User Interface Indication: LCD Display: "Insert or Swipe a card", Amount Buzzer: Alert Tone |
| UI_STRIPE_INSERT_TAP_ MANUAL | Reader is ready to read a card on any of the four interfaces i.e. magstripe, contact, contactless or manual . Suggested User Interface Indication: LCD Display: "Swipe/Insert/Tap/Manual", Amount |
| UI_STRIPE_INSERT_MANUAL | Reader is ready to read a card on any of the three interfaces i.e. magstripe, contact or manual . Suggested User Interface Indication: LCD Display: "Insert/ Swipe /Manual", Amount |
| UI_STRIPE_TAP_MANUAL | Reader is ready to read a card on any of the three interfaces i.e. magstripe, contactless or manual . Suggested User Interface Indication: LCD Display: "Swipe / Tap / Manual", Amount |
| UI_INSERT_TAP_MANUAL | Reader is ready to read a card on any of the three interfaces i.e. , contact, contactless or manual . Suggested User Interface Indication: LCD Display: "Insert/ Tap / Manual", Amount |
| UI_STRIPE_MANUAL | Reader is ready to read a card on the magstripe or manual interface. Suggested User Interface Indication: LCD Display: "Swipe or Manual", Amount |
| UI_INSERT_MANUAL | Reader is ready to read a card on the contact or manual interface. Suggested User Interface Indication: LCD Display: "Insert or Manual", Amount |
| UI_TAP_MANUAL | LCD Display: "Tap or Manual", Amount |

      - L3_CALLBACK_SELECT_CANDIDATE_LIST

When cardholder interaction is required for application selection or cardholder confirmation, SP100 need send request command to upper computer in which cardholder select application.and then upper computer send the selected index to SP100.in this command you can use set/get data command.

Request message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “L1” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 36 |  |
| L3_Callback | 1 | B | 4 | L3_CALLBACK_SELECT_CANDIDATE_LIST |
| listnum | 1 | B |  | AID candidate list num. |
| AID TLV len | 2 | B |  | Other Struct L3_CANDIDATE_LIST’s member add into TLV. total length is one aid’s struct member TLV data length. |
| TLV terminalCodeTable | 2 | B |  | This contains, among other fields, the Code Tables that are supported on the Terminal. Terminal Code Table TAG:9F40 BYTE 4、BYTE 5. |
| TLVperferName | var | A |  | TAG:9F12 Preferred mnemonic associated with the AID |
| TLV lable | var | A |  | TAG:50 Mnemonic associated with the AID |
| TLV priority | 1 | B |  | TAG:87 tag indicating whether Cardholder Confirmation is required for this application or not. |
| TLV aid | var | B |  | TAG:4F DF name. |
| TLVIssuerCodeTableIndex | 1 | B |  | Tag:9F11 The code table according to ISO/IEC 8859 for displaying the Application Preferred Name associated with this ADF name (Card AID). |
| TLV languagePreference | 8 | A |  | TAG:0x5F2D Language Preference |
| TLV kernal id | 8 | B |  | TAG:0xDF37 kernel id |
| TLV terminalPriority | 1 | B |  | TAG:0xDF65 terminal priority |
| TLV custom tag | var | B |  | TAG:0x1F811F Custom defined tags |

Note：if aidlistnum >1 ,command see below.
command  example:

Message Type +Separator+Func ID + CallBack ID+List Num+  Aid 1 Length+ Aid 1 Tlv string + Aid 2 Length + Aid 2 Tlv string+ ....

(Deprecated)Response message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “L0” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 36 |  |
| L3_Callback | 1 | B | 4 | L3_CALLBACK_SELECT_CANDIDATE_LIST |
| Index | 1 | B |  | CallBack Select Index(which AID is selected，index start form 0). |
| TLV LENGTH | 2 | B |  |  |
| AID | var | B |  | The selected AID Tag: 9F06 Format: b (up to 16) |

Response message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “L0” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 36 |  |
| L3_Callback | 1 | B | FF | L3_CALLBACK_General_Response |
| result | 4 | B |  |  |
| Data length | 4 | B |  | The data len need to be response. |
| data | var | B |  | This data need to be set selectdata +TLV data. Please see blow. |
| Index(non-TLV) (belongs to data) | 1 | B |  | CallBack Select Index(which AID is selected，index start form 0). |
| TLV DATA (belongs to data) | var | B |  | you can add tlv data after Index.and index and tlv data all belongs to data. |

      - L3_CALLBACK_AFTER_FINAL_SELECT

This command is used to deal Extension processing after appselect. AFTER_FINAL_SELECT is used to set data after appselect.it is applicable that processing Multi-language selection and Account Selection after final select in RFU.

Request message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “L1” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 36 |  |
| L3_Callback | 1 | B | 0A | L3_CALLBACK_AFTER_FINAL_SELECT |
| interface | 1 | B |  | typedef enum { L3_CONTACT = 0x01, L3_CONTACTLESS = 0x02, }L3_CARD_INTERFACE; |
| AIDLen | 2 | B |  |  |
| AID | var |  |  | Primary Account Number Tag: 9F06 Format: cn (up to 16) |
|  |  |  |  | RFU |

(Deprecated)Response message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “L0” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 36 |  |
| L3_Callback | 1 | B | 0A | L3_CALLBACK_AFTER_FINAL_SELECT |
| TLV Length | 4 | B |  | The data len need to be setted after appsel |
| TLV Data AID | var | B |  | Transaction currency code Tag: 5F24 |
|  |  |  |  | RFU |

Response message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “L0” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 36 |  |
| L3_Callback | 1 | B | FF | L3_CALLBACK_General_Response |
| result | 4 | B |  |  |
| Data length | 4 | B |  | The data len need to be response. |
| data | var | B |  | This data need to be set selectdata +TLV data. Please see blow. |
| TLV DATA | var | B |  | you can add tlv data to data. tlv data belongs to data. |

      - L3_CALLBACK_PIN_ENTRY_DEAL

This command is used to deal pin entry.SP100 send request command to upper computer.accoring to the pin type,uppercomputer need to send different command.this command only use to CT transaction.in this command you can use set/get data command.

Request message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “L1” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 36 |  |
| L3_Callback | 1 | B | 2 | L3_CALLBACK_GET_PIN |
| Pin type | 1 | B |  | typedef enum { L3_PIN_ONLINE, L3_PIN_OFFLINE, L3_PIN_OFFLINE_ENCIPHERED, }L3_PIN_TYPE; |
| TLV LENGTH | 2 | B |  |  |
| TLV Data val PAN | var | B |  | Primary Account Number Tag: 5A Format: cn (up to 19) |
| TLV Data val Offline pin cnt | 1 | B |  | (At present only prompt “Pin type !=L3_PIN_ONLINE”is available.) Offline pin remind entry times Tag: 1F8157 Format: cn |

(Deprecated)Response message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “L0” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 36 |  |
| L3_Callback | 1 | B | 2 | L3_CALLBACK_GET_PIN |
| TLV Length | 2 | B |  | The data len need to be setted after appsel |
| TLV Data value KeyType | 1 | B | 0 | (At present only prompt “Pin type ==L3_PIN_ONLINE”is available.) TAG:1F8136 This data item used to callback report PIN deal. Key type: 0x00; |
| TLV Data value Keyindex | 1 | B | 1 | (At present only prompt “Pin type ==L3_PIN_ONLINE”is available.) TAG:1F8137 This data item used to callback report PIN deal.Key index: 0x01; |
| TLV Data value Timeout | 1 | B |  | TAG:1F8138 This data item used to callback report PIN deal.Key Default timeout is 50s. if you want to set,you need to turn to hex.(as for if you want to set timeout to 40s,you need send 1F81380128 ) |
| TLV Data value PIN RANGE | Var up 12 | B |  | TAG:1F8135 This data item used to callback report PIN deal.Key You can set up pin range by yourself. If not,default is (0,4~12). Such as :1F8135 0c 00 00 04 04 06 06 08 08 0a 0a 0c 0c 03 3C |

Response message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “L0” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 36 |  |
| L3_Callback | 1 | B | FF | L3_CALLBACK_General_Response |
| result | 4 | B |  |  |
| Data length | 4 | B |  | The data len need to be response. |
| data | var | B |  | This data need to be set selectdata +TLV data. Please see blow. |
| TLV DATA | var | B |  | you can add tlv data to data. tlv data belongs to data. This callbcak function you can add tlv data see blow. |
| TLV Data value KeyType | 1 | B | 0 | (At present only prompt “Pin type ==L3_PIN_ONLINE”is available.) TAG:1F8136 This data item used to callback report PIN deal. Key type: 0x00; |
| TLV Data value Keyindex | 1 | B | 1 | (At present only prompt “Pin type ==L3_PIN_ONLINE”is available.) TAG:1F8137 This data item used to callback report PIN deal.Key index: 0x01; |
| TLV Data value Timeout | 1 | B |  | TAG:1F8138 This data item used to callback report PIN deal.Key Default timeout is 50s. if you want to set,you need to turn to hex.(as for if you want to set timeout to 40s,you need send 1F81380128 ) |
| TLV Data value PIN RANGE | Var up 12 | B |  | TAG:1F8135 This data item used to callback report PIN deal.Key You can set up pin range by yourself. If not,default is (0,4~12). you can set the length limit to enter pin. Such as :1F8135 0a 00 04 05 06 07 08 09 0a 0b 0c or 1F8135 04 00 04 05 06 |

      - L3_CALLBACK_CHECK_CREDENTIALS

This command is used to check credentials.During a China Union Pay PBOC transaction, if this non-EMV CVM Method is to be used to verify the identity of the cardholder.SP100 send request message to upper computer,and then upper computer send  response message.in this command you can use set/get data command.

Request message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “L1” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 36 |  |
| L3_Callback | 1 | B | 7 | L3_CALLBACK_CHECK_CREDENTIALS |
| TLV total length | 2 | B |  | Tlv total length. |
| TLV ucType | 1 | B |  | TAG:0x9F62 (PBOC) card holder certificate type Possible Values: 00: ID Card 01: Military ID Card 02: Passport 03: Entry Permit 04: Temporary ID Card 05: Other |
| szNumber | var | A |  | TAG:0x9F61 (PBOC) card holder certificate identify.China Union Pay (PBOC) data element from the card indicating the unique number associated with the specified ID. |

(Deprecated)Response message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “L0” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 36 |  |
| L3_Callback | 1 | B | 7 | L3_CALLBACK_CHECK_CREDENTIALS |
| ErrorCode | 4 | B |  |  |

Response message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “L0” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 36 |  |
| L3_Callback | 1 | B | FF | L3_CALLBACK_General_Response |
| result | 4 | B |  |  |
| Data length | 4 | B |  | The data len need to be response. |
| data | var | B |  | This data need to be set selectdata +TLV data. Please see blow. |
| TLV DATA | var | B |  | you can add tlv data.and tlv data all belongs to data. |

      - L3_CALLBACK_CARDNUM_CONFIRM

This command is used to confirm cardnum during transaction. This is a callback function belong to UI.Originally belonged toL3_CALLBACK_Notification.in this command you can use set/get data command.

Request message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “L1” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 36 |  |
| L3_Callback | 1 | B | 0D | L3_CALLBACK_CARDNUM_CONFIRM |
| Masket PanLen | 1 | B |  | Masket pan len (up to 19) |
| Masket Pan | var | B |  | You can set the rule in performtrasnaction with the tag 1F8134. |

(Deprecated)Response message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “L0” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 36 |  |
| L3_Callback | 1 | B | 0D | L3_CALLBACK_CHECK_CREDENTIALS |
| ErrorCode | 4 | B |  |  |

Response message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “L0” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 36 |  |
| L3_Callback | 1 | B | FF | L3_CALLBACK_General_Response |
| result | 4 | B |  | The callbcak error code you return. |
| Data length | 4 | B |  | The data len need to be response. |
| data | var | B |  | This data need to be set selectdata +TLV data. Please see blow. |
| TLV DATA | var | B |  | you can add tlv data .tlv data all belongs to data. |

      - L3_CALLBACK_DEK

Reserved.    Use for MasterCard Contactless Data Exchange or Data Storage flow.

      - L3_CALLBACK_DET

Reserved.    Use for MasterCard Contactless Data Exchange or Data Storage flow

。

  - Transaction Command

    - Performed Transaction Command

This command allows the user to performed a transaction.

When this performed-transaction-command is sent to the reader, it starts seek cards. Processing a transaction and sending the transaction response to terminal.

And then you shall always execute “Complete Transaction” command to complete the transaction if the transaction is online.

Otherwise, you should execute the “Terminate Transaction” command to stop the transaction and release resource.

Request message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “L0” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 31 |  |
| Card interfaces | 1 | B |  | Card interfaces on which to look for a card. Each interface has a bit in this field. If the bit for an interface is set, the Reader will look for a card on that interface. Any of the bits can be set for a transaction. The format of this byte is given below. Bit 1: Use MSR Interface if set to 1. Bit 2: Use Contact Interface if set to 1. Bit 3: Use Contactless Interface if set to 1. Bits 4-8: RFU. The RFU bits must be set to 0. One or more of the defined bits can be set at the same time. Presence: Mandatory. |
| Timeout | 4 | B |  | Time in seconds for which the Reader will look for a card on the specified interfaces before returning a timeout error. |
| Command Data Length | 2 | B |  | Command Data length |
| Command Data | var |  |  | Command Data see the below |

Command Data:

| Tag | Data Element Name Format |  |  |  | Format | Length In Bytes |
| --- | --- | --- | --- | --- | --- | --- |
| 9F02 | Transaction Amount Authorized (numeric) Authorised amount of the transaction (excluding adjustments) Presence: Mandatory. |  |  |  | n12 | 6 |
| 9F03 | Amount, Other (Numeric) Secondary amount associated with the transaction according to EMVCo definition. (Tag optional, zero amount if not present) Presence: Optional. Mandatory for Cashback Transactions. |  |  |  | n12 | 6 |
| 9C | Transaction Type Indicates the type of financial transaction, represented by the first two digits of ISO 8583, Processing Code. Presence: Mandatory. |  |  |  | n2 | 1 |
| 9A | Transaction local Date (YYMMDD) |  |  |  | n6 | 3 |
| 9F21 | Transaction local Time (HHMMSS) |  |  |  | n6 | 3 |
| 1F8126 | Force Online Enable:01 |  |  |  | b | 1 |
| 1F8164 | App Selection Configuration Bit 8: Always callback App Selection Function for CT. Bit 7: Always callback App Selection Function for CL Bit 6~Bit : RFU |  |  |  | b | 1 |
| 1F8122 | This data item allows configuration of Contact (Chip) to MSR fallback based on EMV, various card association, acquirer or regional (L3) rules. Tag: 1F8122 Format: b3 default value: 00 1F 00 Applies to: CT EMV The format of this data item is as follows: Byte 1: Disable “Contact (Chip) Card to MSR Fallback” By default “Contact to MSR fallback” is supported. However, this data item can be used to disable support for this feature. Possible Values 00h: Contact Card to MSR Fallback supported. This is the default if this data item is not present. 01h: Disable “Contact Card to MSR Fallback” support. If CT to MSR fallback is supported, then the standard method for fallback specified by EMV will always be supported. However, support for non-standard fallback methods can be configured via Byte 2. Byte 2: Non-EMV Fallback Methods Supported. This byte can be used to configure only the nonstandard fallback methods that are usually outside the scope of EMV and are usually specified by the card associations, acquirers or regional rules. The format of this byte is as follows. Bit 8: RFU (must always be set to 0) Bit 7: Enable Fallback on Card Blocked Bit 6: Enable Fallback on App Blocked Bit 5: Enable Fallback on Chip Read Error during App Selection Bit 4: Enable Fallback on Chip Read Error during GPO Bit 3: Enable Fallback on Chip Read Error during Read Records Bit 2: Enable Fallback on Chip Read Error during Gen AC 1 Bit 1: Enable Fallback on Chip Read Error during Gen AC 2 Byte 3: RFU |  |  |  | b | 3 |
| 1F8123 | CL to CT/MSR ‘Fallback’ Configuration Bit 8: Enable when transaction amount exceed contactless limit amount Bit 7~Bit1:RFU |  |  |  | b | 1 |
| 1F8131 | Key Index | 1 | B | Used to encrypt the command data. Only support Data Key Type and ID from 129~250. |  | var |
|  | Tag num | 1 | B | The number of tag in the list. |  |  |
|  | Max len | 2 | B | The size of tlvData. |  |  |
|  | Control code. | 1 | B | bit 1 = 1: Zero Len Tag Allow. |  |  |
|  | Tag list length | 2 | B |  |  |  |
|  | Tag list | var | B | The tag list which will be packed to tlvData. |  |  |
| 1F8139 | This data item used to control callback to upper computer. Tag: 1F8139 Format: b3 default value: FF 00 00. Byte1: used to control callback to Upper computer, Bit 8: L3_CALLBACK_Notification Bit 7:L3_CALLBACK_SELECT_ CANDIDATE_LIST Bit 6:L3_CALLBACK_CHECK_ CREDENTIALS Bit 5:L3_CALLBACK_AFTER_ FINAL_SELECT Bit 4:L3_CALLBACK_GET_PIN Byte 2 :RFU Byte 3:RFU |  |  |  | b | 3 |
| 1F8134 | This tag is used to control the masket pan rule. If this tag not set,the deafult rule :First Clear PAN Digits Length is 0x06; Last Clear PAN Digits Length is 0x04; Byte 1:First Clear PAN Digits Len 0x06-0x0A Byte 2:Last Clear PAN Digits Len 0x01-0x04 |  |  |  | b | 2 |
| Extern | the EMV data element, such as 5F2A Transaction currency code, 5F57 Account Type DF8104 Balance Read Before Gen AC, (CL Mastercard) DF8105 Balance Read After Gen AC, (CL Mastercard) an so on. You can use transaction data to override the Tag defined in AID/Terminal configuration in current transaction. |  |  |  |  | var |

Response message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “L1” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 31 |  |
| Response Code | 2 | A |  | 00 Good 01 Invalid parameters 02 Command Failed 03 Cancelled by the host |
| Transaction status | 1 | B |  | If (Response Code == 0) L3_TXN_OK, 0 L3_TXN_TERMINATE, 1 L3_TXN_TRY_ANOTHER, 2 L3_TXN_DECLINE, 3 L3_TXN_APPROVED, 4 L3_TXN_ONLINE, 5 |
| CVM status | 1 | B |  | OP_NO_CVM 0x00 OP_OBTAIN_SIGNATURE 0x10 OP_ONLINE_PIN 0x20 OP_CONFIRMATION_CODE_VERIFIED 0x30 OP_CVM_NA 0xF0 |
| Error Code | 4 | B |  | If (Response Code == 0) |
| Get Tlv Data Status | 1 | B |  | If( 1F8131 Exist) SUCC 0 Fail 1 No Exist 2 |
| The Actual Data Length | 2 | B |  | If (Get Tlv Data Status == 0&& If( 1F8131 Exist)) The actual length of the data before use CBC to encrypt |
| Tlv list value length | 2 | B |  | If (Get Tlv Data Status == 0&& If( 1F8131 Exist)) |
| Tlv list value | var | B |  | If (Get Tlv Data Status == 0&& If( 1F8131 Exist)) The value will use CBC to encrypt. |

    - Complete Transaction Command

This command allows the user to Complete the transaction.

When Perform Transaction Command return a status of Online Authorization Required.

the terminal must send the transaction online for authorization.

The back-end host may respond back with a result or there may be a timeout or a network error.

In any case, the terminal should use this Command conveying the result of the Online Authorization Request to complete the transaction.

And you should execute the “Terminate Transaction” command to stop the transaction and release resource.

Request message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “L0” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 32 |  |
| Online Result | 1 | B |  | This data element that indicates whether the terminal was able to go online and connect to the back-end host to complete the authorization or not. Possible Values: 01h: Online Processing with back-end host was completed 00h: Online Processing not completed. Could not connect to host or no response from host |
| Command Data Length | 2 | B |  | Command Data Length |
| Command Data | var |  |  | Command Data |

Command Data:

Tlv data:

| Tag | Data Element Name Format |  |  |  | Format | Length In Bytes |
| --- | --- | --- | --- | --- | --- | --- |
| 8A | Authorization Response Code Presence: Mandatory if Online Result is 01h |  |  |  | A | 2 |
| 71 | Script 71 Presence: Optional if Online Result is 01h |  |  |  | B | var |
| 72 | Script 72 Presence: Optional if Online Result is 01h |  |  |  | B | var |
| 91 | Issuer Authentication Data |  |  |  | B | var |
| 1F8112 | Unable online force decline If online communication is unsuccessful then Decline the transaction. Tag: 1F8112 Format:b1 00h: Normal Operation. This is the default if this data item is not present. 01h: Force decline the transaction |  |  |  |  |  |
| 1F8131 | Key Index | 1 | B | Used to encrypt the command data. Only support Data Key Type and ID from 129~250. |  | var |
|  | Tag num | 1 | B | The number of tag in the list. |  |  |
|  | Max len | 2 | B | The size of tlvData. |  |  |
|  | Control code. | 1 | B | bit 1 = 1: Zero Len Tag Allow. |  |  |
|  | Tag list length | 2 | B |  |  |  |
|  | Tag list | var | B | The tag list which will be packed to tlvData. |  |  |
| Extern | the EMV data element You can use transaction data to override the Tag defined in AID/Terminal configuration in current transaction. |  |  |  | B | var |

Response message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “L1” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 32 |  |
| Response Code | 2 | A |  | 00 Success 01 Invalid parameters 02 Command Failed |
| Transaction result | 1 | B |  | If (Response Code == 0) L3_TXN_TERMINATE, 1 L3_TXN_DECLINE, 3 L3_TXN_APPROVED, 4 |
| Error Code | 4 | B |  | If (Response Code == 0) |
| Get Tlv Data Status | 1 | B |  | If( 1F8131 Exist) SUCC 0 Fail 1 No Exist 2 |
| The Actual Data Length | 2 | B |  | If (Get Tlv Data Status == 0&& If( 1F8131 Exist)) The actual length of the data before use CBC to encrypt |
| Tlv list value length | 2 | B |  | If (Get Tlv Data Status == 0&& If( 1F8131 Exist)) |
| Tlv list value | var | B |  | If (Get Tlv Data Status == 0&& If( 1F8131 Exist)) The value will use CBC to encrypt. |

    - Terminate Transaction Command

This command allows the user to stop the transaction, reset the internal transaction status and release resource.

Request message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “L0” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 33 |  |
| Display data | var | B |  | If host need display data see the below |

Display data:

| Data Item | Format | Length In Bytes | Description |
| --- | --- | --- | --- |
| Timeout | B | 2 | Display transaction results,and go back to idle after timeout |
| Display data lenght | B | 2 |  |
| Display data | A | var |  |

Response message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “L1” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 33 |  |
| Response Code | 2 | A |  | 00 Success 01 Invalid parameters 02 Command Failed |

    - Transaction PreProcess Command

This command allows the user to PreProcess the transaction.

Request message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “L0” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 34 |  |
| Command Data Length | 2 | B |  | Command Data length |
| Command Data | var |  |  | Command Data see the below |

Command Data:

| Tag | Data Element Name Format | Format | Length In Bytes |
| --- | --- | --- | --- |
| 9F02 | Transaction Amount Authorized (numeric) Authorised amount of the transaction (excluding adjustments) Presence: Mandatory. | n12 | 6 |
| 9F03 | Amount, Other (Numeric) Secondary amount associated with the transaction according to EMVCo definition. (Tag optional, zero amount if not present) Presence: Optional. Mandatory for Cashback Transactions. | n12 | 6 |
| 9C | Transaction Type Indicates the type of financial transaction, represented by the first two digits of ISO 8583, Processing Code. Presence: Mandatory. | n2 | 1 |

Response message

| Field | Len | Type | Value | Description |
| --- | --- | --- | --- | --- |
| Message Type | 2 | A | “L1” |  |
| Separator | 1 | A |  |  |
| Function ID | 1 | B | 34 |  |
| Response Code | 2 | A |  | 00 Success 01 Invalid parameters 02 Command Failed |
| Error Code | 4 | B |  |  |

    - Cancel Transaction Command

This command uses to Cancel a transaction during poll the card.

This command can only be used for the performed-transaction-command.

This command is not fit for the Message Framing of this document.

Request message(host send to Pinpad ) is “+++CANCEL”(0x2B 0x2B 0x2B 0x43 0x41 0x4E 0x43 0x45 0x4C ),and the Pinpad has no reply。

  - Use the EMV Command Order Set

First you can use “Init EMV Command” to initialite the Level3 module, set configuration, set callback function, set file path and configuration bitmap to kernel. (maybe you can just do it during the first transaction).

Then, using “Performed Transaction” to performed a transaction. When this performed transaction command is sent to the reader, it starts seek cards. Processing a transaction and sending the transaction response to terminal. And then you shall always execute “Complete Transaction command” to complete the transaction if the transaction is online. Otherwise, you should execute the “Terminate Transaction command” to stop the transaction and release resource.

Next, using the “Get Tlv List Data Command” to get the tlv list data value you need.

Ps: During the transaction, maybe you need to get or set a tag data, you can use “Get Data Command” or “Set Data Command” or “Set Tlv List Data Command” to get or set tag data.

If you want to get the EMV debug log, you can use the “Set Debug Mode Command” to open the EMV debug.

If you want to get the EMV kernel version, you can use the “Get Version Command” to Get the version message to the host.

We also provide a serise of EMV Configuration File Commands to “update, get, remove one and remove all” “the EMV Aid configuration file, Capk file, Revocation list file and Exception file”.

  - Terminal Configuration

The Newland Reader maintains two separate Terminal configurations, one each for contact and contactless interfaces. A terminal configuration for a card interface applies to all card applications (AIDs) supported by the Reader for that card interface. The terminal configurations are retained over power cycles.

When the Reader powers up for the first time, it will have no terminal configurations. In this case the Reader will itself set a default terminal configuration for each card interface based on the defaults, Once the initial default terminal configurations are set for each card interface, the Reader will not automatically set or change the terminal configuration at the next power up.

    - Contact

Terminal Configuration Data Items for the Contact Interface

| Configuration Item | Tag | Description |
| --- | --- | --- |
| _EMVPARAM_5F2A_TRANSCCODE | Tag:5F2A Length: 2 Format: n3 | Transaction Currency Code. Indicates the currency code of the transaction according to ISO 4217. For CT this is supposed to be an AID item but adding it here since it is defined as a Terminal parameter by some CT EMV Kernels and makes better sense. |
| _EMVPARAM_5F36_TRANSCEXP | Tag:5F36 Length: 1 Format: n1 | Transaction Currency Exponent. Indicates the implied position of the decimal point from the right of the transaction amount represented according to ISO 4217 |
| _EMVPARAM_9F1A_TERMCCODE | Tag:9F1A Length: 2 Format: n3 | Terminal Country Code Indicates the country of the terminal, represented according to ISO 3166 |
| _EMVPARAM_9F35_TYPE | Tag:9F35 Length:1 Format:n2 | Terminal Type Indicates the environment of the terminal its communications capability, and its operational control |
| _EMVPARAM_9F1B_FLOORLIMIT | Tag:9F1B Length:4 Format: b | Terminal Floor Limit Indicates the floor limit in the terminal in conjunction with the AID |
| _EMVPARAM_9F1C_TERMID | Tag:9F1C Length:8 Format: an 8 | Terminal Identification Designates the unique location of a terminal at a merchant |
| _EMVPARAM_9F1D_TRMDATA | Tag:9F1D Length : 1-8 Format: b | Terminal Risk Management Data Application-specific value used by the card for risk management purposes |
| _EMVPARAM_9F1E_IFDSERNUM | Tag:9F1E Length:8 Format: an 8 | Interface Device (IFD) Serial Number Unique and permanent serial number assigned to the IFD by the manufacturer |
| _EMVPARAM_9F33_CAP | Tag:9F33 Length:3 Format: b | Terminal Capabilities Indicates the card data input, CVM, and security capabilities of the terminal |
| _EMVPARAM_9F40_ADDCAP | Tag:9F40 Length:5 Format: b | Additional Terminal Capabilities Indicates the data input and output capabilities of the terminal |
| _EMVPARAM_9F01_ACQID | Tag:9F01 Length:6 Format: n 6-11 | Acquirer Identifier Uniquely identifies the acquirer within each payment system |
| _EMVPARAM_9F15_MERCACODE | Tag:9F15 Length:2 Format: n 4 | Merchant Category Code Classifies the type of business being done by the merchant, represented according to ISO 8583:1993 for Card Acceptor Business Code |
| _EMVPARAM_9F16_MERCHID | Tag:9F16 Length:15 Format: ans15 | Merchant Identifier When concatenated with the Acquirer Identifier, uniquely identifies a given merchant |
| _EMVPARAM_9F3C_TRANSREFCCODE | Tag:9F3C Length:2 Format: n 3 | Transaction Reference Currency Code Code defining the common currency used by the terminal in case the Transaction Currency Code is different from the Application Currency Code |
| _EMVPARAM_9F4E_MERCNAME | Tag:9F4E Length: var Format: ans | Merchant Name and Location Indicates the name and location of the merchant |
| _EMVPARAM_DF44_DEDDOL | Tag:DF44 Length: var Format: b | Default Dynamic Data Authentication Data Object List (DDOL) DDOL to be used for constructing the INTERNAL AUTHENTICATE command if the DDOL in the card is not present |
| _EMVPARAM_DF45_DETDOL | Tag:DF45 Length: var Format: b | Default Transaction Certificate Data Object List (TDOL) TDOL to be used for generating the TC Hash Value if the TDOL in the card is not present |
| _EMVPARAM_DF15_THRESHOLDVA | Tag:DF15 Length:4 Format: n 4 | Threshold Value for Biased Random Selection Value used in terminal risk management for random transaction selection |
| _EMVPARAM_DF16_MAXTARPER | Tag:DF16 Length:1 Format: n 1 | Maximum Target Percentage to be used for Biased Random Selection Value used in terminal risk management for random transaction selection |
| _EMVPARAM_DF17_TARGETPER | Tag:DF17 Length:1 Format: n 1 | Target Percentage to be Used for Random Selection Value used in terminal risk management for random transaction selection |
| _EMVPARAM_DF22_TRANSCONV | Tag:DF22 Length: 4 Format: n 8 | Transaction Reference Currency Conversion Factor used in the conversion from the Transaction Currency Code to the Transaction Reference Currency Code |
| _EMVPARAM_DF01_APPSELIND | Tag:DF01 Length: 1 Format: b | Application Selection Indicator For an application in the ICC to be supported by an application in the terminal, the Application Selection Indicator indicates whether the associated AID in the terminal must match the AID in the card exactly, including the length of the AID, or only up to the length of the AID in the Terminal There is only one Application Selection Indicator per AID supported by the terminal. 0x01-partial matching |
| _EMVPARAM_DF13_TACDENIAL | Tag:DF13 Length: 5 Format: b | Terminal Action Code-Denial Specifies the acquirer’s conditions that cause the denial of a transaction without attempt to go online |
| _EMVPARAM_DF12_TACONLINE | Tag:DF12 Length: 5 Format: b | Terminal Action Code-Online Specifies the acquirer’s conditions that cause a transaction to be transmitted online |
| _EMVPARAM_DF11_TACDEFAULT | Tag:DF11 Length: 5 Format: b | Terminal Action Code-Default Specifies the acquirer’s conditions that cause a transaction to be rejected if it might have been approved online, but the terminal is unable to process the transaction online |
| _EMVPARAM_DF24_ICS | Tag:DF24 Length: 7 Format: b | Implementation Conformance Statement Byte 1 Application Selection bit 8 Support PSE selection method bit 7 Support Cardholder confirmation bit 6 Have a preferred order of displaying applications bit 5 Does the terminal perform partial AID selection bit 4 Does the terminal support multiple language selection bit 3 Common Character Set("Annex B table 20 Book 4") bit 2- bit 1 Rfu Byte 2 Data Authentication bit 8 Revocation of Issuer Public Key Certificate bit 7 Does the terminal contain a default DDOL bit 6 Manual action when loading CAPK fails bit 5 CAPK verified with CAPK checksum bit 4 – bit 1 Rfu Byte 3 Cardholder Verification bit 8 Terminal supports bypass PIN entry bit 7 Terminal supports subsequent bypass PIN entry bit 6 Terminal supports Get Data for PIN Try Counter bit 5 Terminal supports Fail CVM bit 4 Amount known before CVM processing bit 3- bit 1 Rfu Byte 4 Terminal Risk Management bit 8 Floor Limit Checking bit 7 Random Transaction Selections bit 6 Velocity checking bit 5 Transaction Log bit 4 Exception File bit 3 Terminal Risk Management irrespective of AIP setting bit 2 Transaction Log mode (1-EMV , 0-PBOC) bit 1 Rfu Byte 5 Terminal Action Analysis bit 8 Terminal Action Codes supported bit 7 process TAC-default prior to first GenAC bit 6 process TAC-default after first GenAC bit 5 Rfu bit 4 TAC/IAC-Default process when unable to go online bit 3-bit 1 Rfu Byte 6 Completion Process bit 8 Rfu bit 7 Rfu bit 6 Rfu bit 5 Issuer Referrals bit 4-bit1 Rfu Byte 7 Others bit 8 support account type selection bit 7 Rfu bit 6 Rfu bit 5 support default UDOL (used for Paypass only ) bit 4-bit1 Rfu |

    - Contactless

Terminal Configuration Data Items for the Contactless Interface

| Configuration Item | Tag | Description |
| --- | --- | --- |
| _EMVPARAM_5F2A_TRANSCCODE | Tag:5F2A Length: 2 Format: n3 | Transaction Currency Code. Indicates the currency code of the transaction according to ISO 4217. For CLSS this is supposed to be an AID item but adding it here since it is defined as a Terminal parameter by some CT EMV Kernels and makes better sense. |
| _EMVPARAM_5F36_TRANSCEXP | Tag:5F36 Length: 1 Format: n1 | Transaction Currency Exponent. Indicates the implied position of the decimal point from the right of the transaction amount represented according to ISO 4217 |
| _EMVPARAM_9F1A_TERMCCODE | Tag:9F1A Length: 2 Format: n3 | Terminal Country Code Indicates the country of the terminal, represented according to ISO 3166 |
| _EMVPARAM_9F35_TYPE | Tag:9F35 Length:1 Format:n2 | Terminal Type Indicates the environment of the terminal its communications capability, and its operational control |
| _EMVPARAM_9F1B_FLOORLIMIT | Tag:9F1B Length:4 Format: b | Terminal Floor Limit Indicates the floor limit in the terminal in conjunction with the AID |
| _EMVPARAM_9F1C_TERMID | Tag:9F1C Length:8 Format: an 8 | Terminal Identification Designates the unique location of a terminal at a merchant |
| _EMVPARAM_9F1D_TRMDATA | Tag:9F1D Length : 1-8 Format: b | Terminal Risk Management Data Application-specific value used by the card for risk management purposes |
| _EMVPARAM_9F1E_IFDSERNUM | Tag:9F1E Length:8 Format: an 8 | Interface Device (IFD) Serial Number Unique and permanent serial number assigned to the IFD by the manufacturer |
| _EMVPARAM_9F33_CAP | Tag:9F33 Length:3 Format: b | Terminal Capabilities Indicates the card data input, CVM, and security capabilities of the terminal |
| _EMVPARAM_9F40_ADDCAP | Tag:9F40 Length:5 Format: b | Additional Terminal Capabilities Indicates the data input and output capabilities of the terminal |
| _EMVPARAM_9F01_ACQID | Tag:9F01 Length:6 Format: n 6-11 | Acquirer Identifier Uniquely identifies the acquirer within each payment system |
| _EMVPARAM_9F15_MERCACODE | Tag:9F15 Length:2 Format: n 4 | Merchant Category Code Classifies the type of business being done by the merchant, represented according to ISO 8583:1993 for Card Acceptor Business Code |
| _EMVPARAM_9F16_MERCHID | Tag:9F16 Length:15 Format: ans15 | Merchant Identifier When concatenated with the Acquirer Identifier, uniquely identifies a given merchant |
| _EMVPARAM_9F3C_TRANSREFCCODE | Tag:9F3C Length:2 Format: n 3 | Transaction Reference Currency Code Code defining the common currency used by the terminal in case the Transaction Currency Code is different from the Application Currency Code |
| _EMVPARAM_9F4E_MERCNAME | Tag:9F4E Length: var Format: ans | Merchant Name and Location Indicates the name and location of the merchant |
| _EMVPARAM_DF44_DEDDOL | Tag:DF44 Length: var Format: b | Default Dynamic Data Authentication Data Object List (DDOL) DDOL to be used for constructing the INTERNAL AUTHENTICATE command if the DDOL in the card is not present |
| _EMVPARAM_DF45_DETDOL | Tag:DF45 Length: var Format: b | Default Transaction Certificate Data Object List (TDOL) TDOL to be used for generating the TC Hash Value if the TDOL in the card is not present |
| _EMVPARAM_DF15_THRESHOLDVA | Tag:DF15 Length:4 Format: n 4 | Threshold Value for Biased Random Selection Value used in terminal risk management for random transaction selection |
| _EMVPARAM_DF16_MAXTARPER | Tag:DF16 Length:1 Format: n 1 | Maximum Target Percentage to be used for Biased Random Selection Value used in terminal risk management for random transaction selection |
| _EMVPARAM_DF17_TARGETPER | Tag:DF17 Length:1 Format: n 1 | Target Percentage to be Used for Random Selection Value used in terminal risk management for random transaction selection |
| _EMVPARAM_DF24_ICS | Tag:DF24 Length: 7 Format: b | Implementation Conformance Statement Byte 1 Application Selection bit 8 Support PSE selection method bit 7 Support Cardholder confirmation bit 6 Have a preferred order of displaying applications bit 5 Does the terminal perform partial AID selection bit 4 Does the terminal support multiple language selection bit 3 Common Character Set("Annex B table 20 Book 4") bit 2- bit 1 Rfu Byte 2 Data Authentication bit 8 Revocation of Issuer Public Key Certificate bit 7 Does the terminal contain a default DDOL bit 6 Manual action when loading CAPK fails bit 5 CAPK verified with CAPK checksum bit 4 – bit 1 Rfu Byte 3 Cardholder Verification bit 8 Terminal supports bypass PIN entry bit 7 Terminal supports subsequent bypass PIN entry bit 6 Terminal supports Get Data for PIN Try Counter bit 5 Terminal supports Fail CVM bit 4 Amount known before CVM processing bit 3- bit 1 Rfu Byte 4 Terminal Risk Management bit 8 Floor Limit Checking bit 7 Random Transaction Selections bit 6 Velocity checking bit 5 Transaction Log bit 4 Exception File bit 3 Terminal Risk Management irrespective of AIP setting bit 2 Transaction Log mode (1-EMV , 0-PBOC) bit 1 Rfu Byte 5 Terminal Action Analysis bit 8 Terminal Action Codes supported bit 7 process TAC-default prior to first GenAC bit 6 process TAC-default after first GenAC bit 5 Rfu bit 4 TAC/IAC-Default process when unable to go online bit 3-bit 1 Rfu Byte 6 Completion Process bit 8 Rfu bit 7 Rfu bit 6 Rfu bit 5 Issuer Referrals bit 4-bit1 Rfu Byte 7 Others bit 8 support account type selection bit 7 Rfu bit 6 Rfu bit 5 support default UDOL (used for Paypass only ) bit 4-bit1 Rfu |
| _EMVPARAM_DF22_TRANSCONV | Tag:DF22 Length: 4 Format: n 8 | Transaction Reference Currency Conversion Factor used in the conversion from the Transaction Currency Code to the Transaction Reference Currency Code |
| _EMVPARAM_DF01_APPSELIND | Tag:DF01 Length: 1 Format: b | Application Selection Indicator For an application in the ICC to be supported by an application in the terminal, the Application Selection Indicator indicates whether the associated AID in the terminal must match the AID in the card exactly, including the length of the AID, or only up to the length of the AID in the Terminal There is only one Application Selection Indicator per AID supported by the terminal. 0x01-partial matching |
| _EMVPARAM_DF20_CLLIMMIT | Tag:DF20 Length : 6 Format: n12 | Reader Contactless Transaction Limit (No On-device CVM)- 'DF8124' Indicates the transaction amount above which the transaction is not allowed, when on device cardholder verification is not supported. |
| _EMVPARAM_DF21_CVMLIMT | Tag:DF21 Length : 6 Format: n12 | Reader CVM Required Limit - 'DF8126’ Indicates the transaction amount above which the Kernel instantiates the CVM capabilities field in Terminal Capabilities with CVM Capability – CVM Required. |
| _EMVPARAM_DF19_CLOFFLIMIT | Tag:DF19 Length : 6 Format: n12 | Reader Contactless Floor Limit- ‘DF8123’ Indicates the transaction amount above which transactions must be authorized online. |
| _EMVPARAM_DF27_LIMITEXIST | Tag:DF27 Length : 1 Format: b | Indicate whether using the Limit Amount. bit 8: Rfu bit 7: Rfu bit 6: Rfu bit 5: Rfu bit 4: contactless CVM required limit supported bit 3: contactless floor limit supported bit 2: contactless terminal limit supported bit 1: Rfu |
| _EMVPARAM_DF39_STATUSCHECK | Tag:DF39 Length : 1 Format: b | Status check supported 0x01 supported 0x00 Not supported |
| _EMVPARAM_DF3A_ZEROALLOW | Tag:DF3A Length : 1 Format: b | Zero Amount Allowed 0x01: Zero amount transactions are supported. 0x00: Zero amount transactions are not supported. |
| _EMVPARAM_DF63_IOOPTION | Tag: DF63 Length:1 Format: b | Contactless POS Implementation Options Used for PURE kernel Byte 1 bit 8: IO Options 1 bit 7: IO Options 2 bit 6: IO Options 3 bit 5: IO Options 4 bit 4: IO Options 5 bit3-1: Rfu |
| _EMVPARAM_DF64_TERMINALAIDVALUE | Tag:DF64 Length: 5-16 Format: b | Terminal AID values Used for PURE kernel |
| _EMVPARAM_1F8102_PPSEAIDSEL | Tag:1F8102 Length : 1 Format: b | Select by AID supported(Select via List of AIDs supported?) For some contactless kernel, such as Amex, if PPSE failed, will try to select by AID. 0x00: Select by AID is not supported (Default). 0x01: Select by AID is supported. |

  - AID Configuration

Every configuration item defined in Terminal configuration, you can setup again in AID configuration. The value of the TLV defined in the AID configuration will override the value in the Terminal configuration when it is loaded

    - Contact EMV AIDs

Data Items Specific to Contact EMV AIDs

| Configuration Item | Tag | Description |
| --- | --- | --- |
| _EMVPARAM_9F06_AID | Tag:9F06 Length: 5-16 Format: b | Application Identifier (AID) Identifies the application as described in ISO/IEC 7816-5 |
| _EMVPARAM_DF13_TACDENIAL | Tag:DF13 Length: 5 Format: b | Terminal Action Code-Denial Specifies the acquirer’s conditions that cause the denial of a transaction without attempt to go online |
| _EMVPARAM_DF12_TACONLINE | Tag:DF12 Length: 5 Format: b | Terminal Action Code-Online Specifies the acquirer’s conditions that cause a transaction to be transmitted online |
| _EMVPARAM_DF11_TACDEFAULT | Tag:DF11 Length: 5 Format: b | Terminal Action Code-Default Specifies the acquirer’s conditions that cause a transaction to be rejected if it might have been approved online, but the terminal is unable to process the transaction online |
| _EMVPARAM_9F09_APPVER | Tag:9F09 Length: 2 Format: b | Application Version Number Version number assigned by the payment system for the application |
| _EMVPARAM_9F1B_FLOORLIMIT | Tag: 9F1B Length: 4 Format: b | Terminal Floor Limit Indicates the floor limit in the terminal in conjunction with the AID |
| _EMVPARAM_9F1D_TRMDATA | Tag: 9F1D Length: 1-8 Format: b | Terminal Risk Management Data Application-specific value used by the card for risk management purposes |
| _EMVPARAM_9F7B_ECLIMIT | Tag:9F7B Length:6 Format: n12 | EC Terminal Transaction Limit. If authorized amount higher than EC limit, transaction will request online authorization. Note: Used for PBOC EC function |
| _EMVPARAM_9F7A_ECIND | Tag:9F7A Length:1 Format: b | EC Terminal Support Indicator Indicate supporting E-Cash or not. Note: Used for PBOC EC function |
| _EMVPARAM_1F811F_CUSTOM_TAG | Tag: 1F811F Length: Var Format: b | Custom Defined Tags. EMV kernel will store these custom tags during Application Selection or Read Application Date (If available). And We can get their values in Candidate List Or via ‘ NAPI_L3GetData’. |
| …… |  |  |

    - Contactless Visa AIDs (Kernel 3)

Data Items Specific to Contactless Visa AIDs

| Configuration Item | Tag | Description |
| --- | --- | --- |
| _EMVPARAM_9F06_AID | Tag:9F06 Length: 5-16 Format: b | Application Identifier (AID) Identifies the application as described in ISO/IEC 7816-5 |
| _EMVPARAM_DF37_KERNELID | Tag:DF37 Length: 8 Format: b | Kernel ID Valid Value: “\x03\x00\x 00\x 00\x 00\x 00\x 00\x 00” |
| _EMVPARAM_9F66_TRANSPROP | Tag:9F66 Length:4 Format: b 32 | Terminal Transaction Qualifiers (TTQ) Indicates reader capabilities, requirements, and preferences to the card. TTQ byte 2 bits 8-7 are transient values, and reset to zero at the beginning of the transaction. All other TTQ bits are static values, and not modified based on transaction conditions. TTQ byte 3 bit 7 shall be set by the acquirer-merchant to 1b. Values: |
| _EMVPARAM_DF34_PWCONFIG | Tag:DF34 Length: Format: |  |
| _EMVPARAM_DF3D_DRLSTATUS | Tag:DF3D Length:1 Format: b | DRL Support Status 0x01: Support DRL |
| _EMVPARAM_DF3F_DRLDATA | Tag:DF3F Length: var Format: b | DRL Data |
| …… |  |  |

    - Contactless MasterCard AIDs (Kernel 2)

Data Items Specific to Contactless Mastercard Paypass AIDs

| Configuration Item | Tag | Description |
| --- | --- | --- |
| _EMVPARAM_9F06_AID | Tag:9F06 Length: 5-16 Format: b | Application Identifier (AID) Identifies the application as described in ISO/IEC 7816-5 |
| _EMVPARAM_DF37_KERNELID | Tag:DF37 Length: 8 Format: b | Kernel ID Valid Value: “\x02\x00\x 00\x 00\x 00\x 00\x 00\x 00” |
| _EMVPARAM_9F33_CAP | Tag: 9F33 Length: 3 Format: b | Byte 1: Card Data Input Capability-'DF8117 Indicates the card data input capability of the Terminal and Reader. The Card Data Input Capability is coded according to Annex A.2 of [EMV Book 4]. Byte 2: CVM Capability – CVM Required-'DF8118' Indicates the CVM capability of the Terminal and Reader when the transaction amount is greater than the Reader CVM Required Limit. Byte 3: Security Capability-'DF811F Indicates the security capability of the Kernel. |
| _EMVPARAM_DF48_CAPNOCVM | Tag:DF48 Length: 1 Format: b | CVM Capability – No CVM Required-'DF8119' Indicates the CVM capability of the Terminal and Reader when the transaction amount is less than or equal to the Reader CVM Required Limit. |
| _EMVPARAM_DF42_MAGSCVM | Tag:DF42 Length: 1 Format: b | Mag-stripe CVM Capability – CVM Required-'DF811E' Indicates the CVM capability of the Terminal/Reader in the case of a mag-stripe mode transaction when the Amount, Authorized (Numeric) is greater than the Reader CVM Required Limit. |
| _EMVPARAM_DF47_MAGSNOCVM | Tag:DF47 Length: 1 Format: b | Mag-stripe CVM Capability – No CVM Required-'DF812C' Indicates the CVM capability of the Terminal/Reader in the case of a mag-stripe mode transaction when the Amount, Authorized (Numeric) is less than or equal to the Reader CVM Required Limit. |
| _EMVPARAM_DF2F_KERNELCONF | Tag:DF2F Length:1 Format: b | Kernel Configuration- ‘DF811B’. |
| _EMVPARAM_DF2B_DEUDOL | Tag:DF2B Length:3 Format: b | Default UDOL- 'DF811A' The Default UDOL is the UDOL to be used for constructing the value field of the COMPUTE CRYPTOGRAPHIC CHECKSUM command if the UDOL in the Card is not present. The Default UDOL must contain as its only entry the tag and length of the Unpredictable Number (Numeric) and has the value: '9F6A04'. |
| _EMVPARAM_DF46_MOSUPPIND | Tag:DF46 Length: 1 Format: b | Mobile Support Indicator-'9F7E’ The Mobile Support Indicator informs the Card that the Kernel supports extensions for mobile and requires on device cardholder verification. |
| _EMVPARAM_9F7B_ECLIMIT | Tag:9F7B Length : 6 Format: n12 | Reader Contactless Transaction Limit (On-device CVM)- 'DF8125’ Indicates the transaction amount above which the transaction is not allowed, when on device cardholder verification is supported. Note: History reason, this tag is same as Contact PBOC EC transaction limit. |
| _EMVPARAM_DF20_CLLIMMIT | Tag:DF20 Length : 6 Format: n12 | Reader Contactless Transaction Limit (No On-device CVM)- 'DF8124' Indicates the transaction amount above which the transaction is not allowed, when on device cardholder verification is not supported. |
| _EMVPARAM_DF21_CVMLIMT | Tag:DF21 Length : 6 Format: n12 | Reader CVM Required Limit - 'DF8126’ Indicates the transaction amount above which the Kernel instantiates the CVM capabilities field in Terminal Capabilities with CVM Capability – CVM Required. |
| _EMVPARAM_DF19_CLOFFLIMIT | Tag:DF19 Length : 6 Format: n12 | Reader Contactless Floor Limit- ‘DF8123’ Indicates the transaction amount above which transactions must be authorized online. |
| _EMVPARAM_9F1D_TRMDATA | Tag:9F1D Length : 1-8 Format: b | Terminal Risk Management Data Application-specific value used by the card for risk management purposes |
| _EMVPARAM_DF13_TACDENIAL | Tag:DF13 Length: 5 Format: b | Terminal Action Code-Denial Specifies the acquirer’s conditions that cause the denial of a transaction without attempt to go online |
| _EMVPARAM_DF12_TACONLINE | Tag:DF12 Length: 5 Format: b | Terminal Action Code-Online Specifies the acquirer’s conditions that cause a transaction to be transmitted online |
| _EMVPARAM_DF11_TACDEFAULT | Tag:DF11 Length: 5 Format: b | Terminal Action Code-Default Specifies the acquirer’s conditions that cause a transaction to be rejected if it might have been approved online, but the terminal is unable to process the transaction online |
| _EMVPARAM_DF54_MAXRRPGP | Tag:DF54 Length: 2 Format: b | Maximum Relay Resistance Grace Period- 'DF8133’ The Minimum Relay Resistance Grace Period and Maximum Rela Resistance Grace Period represent how far outside the window defined by the Card that the measured time may be and yet still be considered acceptable. The Maximum Relay Resistance Grace Period is expressed in units of hundreds of microseconds. Note: this tag is mandatory for ‘RRP’ (if kernel configuration(DF2F) support ‘RRP’) |
| _EMVPARAM_DF55_MINRRPGP | Tag:DF55 Length: 2 Format: b | Minimum Relay Resistance Grace Period- 'DF8132' The Minimum Relay Resistance Grace Period and Maximum Relay Resistance Grace Period represent how far outside the window defined by the Card that the measured time may be and yet still be considered acceptable. The Minimum Relay Resistance Grace Period is expressed in units of hundreds of microseconds. Note: this tag is mandatory for ‘RRP’ (if kernel configuration(DF2F) support ‘RRP’) |
| _EMVPARAM_DF56_RRPAT | Tag:DF56 Length: 2 Format: b | Relay Resistance Accuracy Threshold- 'DF8136' Represents the threshold above which the Kernel considers the variation between Measured Relay Resistance Processing Time and Max Time For Processing Relay Resistance APDU no longer acceptable. The Relay Resistance Accuracy Threshold is expressed in units of hundreds of microseconds. Note: this tag is mandatory for ‘RRP’ (if kernel configuration(DF2F) support ‘RRP’) |
| _EMVPARAM_DF57_RRPTTMT | Tag:DF57 Length: 1 Format: b | Relay Resistance Transmission Time Mismatch Threshold- 'DF8137’ Represents the threshold above which the Kernel considers the variation between Device Estimated Transmission Time For Relay Resistance R-APDU and Terminal Expected Transmission Time For Relay Resistance R-APDU no longer acceptable. The Relay Resistance Transmission Time Mismatch Threshold is a percentage and expressed as an integer. Note: this tag is mandatory for ‘RRP’ (if kernel configuration(DF2F) support ‘RRP’) |
| _EMVPARAM_DF58_TETTFRRC | Tag:DF58 Length: 2 Format: b | Terminal Expected Transmission Time For Relay Resistance C-APDU- ‘DF8134’ Represents the time that the Kernel expects to need for transmitting the EXCHANGE RELAY RESISTANCE DATA command to the Card. The Terminal Expected Transmission Time For Relay Resistance C-APDU is expressed in units of hundreds of microseconds. Note: this tag is mandatory for ‘RRP’ (if kernel configuration(DF2F) support ‘RRP’) |
| _EMVPARAM_DF59_TETTFRRR | Tag:DF59 Length: 2 Format: b | Terminal Expected Transmission Time For Relay Resistance R-APDU- 'DF8135’ Represents the time that the Kernel expects that the Card will need for transmitting the EXCHANGE RELAY RESISTANCE DATA R-APDU. The Terminal Expected Transmission Time For Relay Resistance R-APDU is expressed in units of hundreds of microseconds. Note: this tag is mandatory for ‘RRP’ (if kernel configuration(DF2F) support ‘RRP’) |
| _EMVPARAM_DF2E_DEXCHANGE | Tag: DF2E Length: 1 Format: b | Indicate whether terminal supports Data Exchange function. Newland custom tag. 0x01-support |
| _EMVPARAM_DF52_PPTLV | Tag: DF52 Length: Var Format: b | Extern TLV string. You can setup other configuration into this item. Such as configuration for Data Exchange. e.g. DS Input (card)-‘DF60’ DS ODS Info-‘DF62’ DS Requested Operator ID- ‘9F5C’ …. |
| _EMVPARAM_1F8101_TRANSTYPECKFLAG | Tag: 1F8101 Length: 1 Format: b | Indicate whether matching trans type when picking up this aid. Newland custom tag. 0x01-support |
| _EMVPARAM_DF7D_TRANSTYPE | Tag: DF7D Length: 1 Format: n2 | Transaction type. the value of 9C (00/01/09/20) when 1F8101==0x01, only this item is equal to the value of current transaction type (0x9C) then we select this aid. |
| …… |  |  |

    - Contactless American Express AIDs (Kernel 4)

Data Items Specific to Contactless American Express AIDs

| Configuration Item | Tag | Description |
| --- | --- | --- |
| _EMVPARAM_9F06_AID | Tag:9F06 Length: 5-16 Format: b | Application Identifier (AID) Identifies the application as described in ISO/IEC 7816-5 |
| _EMVPARAM_DF37_KERNELID | Tag:DF37 Length: 8 Format: b | Kernel ID Valid Value: “\x04\x00\x 00\x 00\x 00\x 00\x 00\x 00” |
| _EMVPARAM_9F66_TRANSPROP | Tag:9F66 Length: 4 Format: b | Enhanced contactless reader capabilities-Tag ‘9F6E’ Note: The Terminal must always be capable of recognizing the CVM code for “No CVM Required”. However, the inclusion of this CVM in the list of methods supported by the Terminal, constructed during CVM processing (Section 9.2.2), is based on the transaction amount and the Reader CVM required limit. |
| _EMVPARAM_DF49_EXTERMCAP | Tag:DF49 Length:1 Format: b | Contactless Reader Capabilities - Tag ‘9F6D’ ‘00’ = Expresspay 1.0 ‘40’ = Expresspay 2.0Magstripe only ‘48’ = Expresspay 2.0Magstripe – Mobile CVMRequired ‘80’ = Expresspay 2.0EMV and Magstripe ‘C0’ = ExpresspayMobile (XPM) ‘C8’ = ExpresspayMobile (XPM) - MobileCVM Required |
| _EMVPARAM_DF4A_EXRANDOM | Tag:DF4A Length:1 Format: b | Unpredictable Number Range Specifies the range in which the unpredictable number must be generated in for contactless mag-stripe mode. Usage: The default minimum range is 0 to 60. Note that the number range is inclusive, so a range of 0 to 60 should be capable of generating 61 integer numbers in the range 0 to 60. |
| _EMVPARAM_DF13_TACDENIAL | Tag:DF13 Length: 5 Format: b | Terminal Action Code-Denial Specifies the acquirer’s conditions that cause the denial of a transaction without attempt to go online |
| _EMVPARAM_DF12_TACONLINE | Tag:DF12 Length: 5 Format: b | Terminal Action Code-Online Specifies the acquirer’s conditions that cause a transaction to be transmitted online |
| _EMVPARAM_DF11_TACDEFAULT | Tag:DF11 Length: 5 Format: b | Terminal Action Code-Default Specifies the acquirer’s conditions that cause a transaction to be rejected if it might have been approved online, but the terminal is unable to process the transaction online |
| …… |  |  |

    - Contactless Discover AIDs (Kernel 6)

Data Items Specific to Contactless Discover AIDs

| Configuration Item | Tag | Description |
| --- | --- | --- |
| _EMVPARAM_9F06_AID | Tag:9F06 Length: 5-16 Format: b | Application Identifier (AID) Identifies the application as described in ISO/IEC 7816-5 |
| _EMVPARAM_DF37_KERNELID | Tag:DF37 Length: 8 Format: b | Kernel ID Valid Value: “\x06\x00\x 00\x 00\x 00\x 00\x 00\x 00” |
| _EMVPARAM_9F66_TRANSPROP | Tag:9F66 Length:4 Format: b 32 | Terminal Transaction Qualifiers (TTQ) Indicates reader capabilities, requirements, and preferences to the card. Terminal transaction Qualifiers are online and CVM processing options that are described in [EMV CTL: BOOK B, v2.1] and [EMV CTL: BOOK B, v2.1]. These Qualifiers include options that may be supported by the Terminal during Entry Point processing |
| …… |  |  |

    - Contactless JCB AIDs (Kernel 5)

Data Items Specific to Contactless JCB AIDs

| Configuration Item | Tag | Description |
| --- | --- | --- |
| _EMVPARAM_9F06_AID | Tag:9F06 Length: 5-16 Format: b | Application Identifier (AID) Identifies the application as described in ISO/IEC 7816-5 |
| _EMVPARAM_DF37_KERNELID | Tag:DF37 Length: 8 Format: b | Kernel ID Valid Value: “\x05\x00\x 00\x 00\x 00\x 00\x 00\x 00” |
| _EMVPARAM_9F53_TIP | Tag:9F53 Length: 3 Format: b | Terminal Interchange Profile (static/dynamic) |
| _EMVPARAM_DF60_COMBINATIONOPT | Tag:DF60 Length: 2 Format: b | Combination Options |
| _EMVPARAM_DF13_TACDENIAL | Tag:DF13 Length: 5 Format: b | Terminal Action Code-Denial Specifies the acquirer’s conditions that cause the denial of a transaction without attempt to go online |
| _EMVPARAM_DF12_TACONLINE | Tag:DF12 Length: 5 Format: b | Terminal Action Code-Online Specifies the acquirer’s conditions that cause a transaction to be transmitted online |
| _EMVPARAM_DF11_TACDEFAULT | Tag:DF11 Length: 5 Format: b | Terminal Action Code-Default Specifies the acquirer’s conditions that cause a transaction to be rejected if it might have been approved online, but the terminal is unable to process the transaction online |
| …… |  |  |

    - Contactless Unionpay AIDs (Kernel 7)

Data Items Specific to Contactless Uinionpay AIDs

| Configuration Item | Tag | Description |
| --- | --- | --- |
| _EMVPARAM_9F06_AID | Tag:9F06 Length: 5-16 Format: b | Application Identifier (AID) Identifies the application as described in ISO/IEC 7816-5 |
| _EMVPARAM_DF37_KERNELID | Tag:DF37 Length: 8 Format: b | Kernel ID Valid Value: “\x07\x00\x 00\x 00\x 00\x 00\x 00\x 00” |
| _EMVPARAM_9F66_TRANSPROP | Tag:9F66 Length:4 Format: b 32 | Terminal Transaction Qualifiers (TTQ) Indicates reader capabilities, requirements, and preferences to the card. TTQ byte 2 bits 8-7 are transient values, and reset to zero at the beginning of the transaction. All other TTQ bits are static values, and not modified based on transaction conditions. TTQ byte 3 bit 7 shall be set by the acquirer-merchant to 1b. Values: |
| _EMVPARAM_5F2A_TRANSCCODE | Tag:5F2A Length: 2 Format: n3 | Transaction Currency Code. Indicates the currency code of the transaction according to ISO 4217. |
| …… |  |  |

    - Contactless Pure(MCCS/MADA) AIDs

Data Items Specific to Contactless Pure(MCCS/MADA) AIDs

| Configuration Item | Tag | Description |
| --- | --- | --- |
| _EMVPARAM_9F06_AID | Tag:9F06 Length: 5-16 Format: b | Application Identifier (AID) Identifies the application as described in ISO/IEC 7816-5 |
| _EMVPARAM_DF37_KERNELID | Tag:DF37 Length: 8 Format: b | Kernel ID MCCS:Valid Value: “\x20\x00\x 00\x 00\x 00\x 00\x 00\x 00” MADA:Valid Value: “\x2D\x00\x 00\x 00\x 00\x 00\x 00\x 00” |
| _EMVPARAM_DF62_CONTAPPCAP | Tag:DF62 Length: 5 Format: b | Contactless Application Capabilities |
| _EMVPARAM_DF13_TACDENIAL | Tag:DF13 Length: 5 Format: b | Terminal Action Code-Denial Specifies the acquirer’s conditions that cause the denial of a transaction without attempt to go online |
| _EMVPARAM_DF12_TACONLINE | Tag:DF12 Length: 5 Format: b | Terminal Action Code-Online Specifies the acquirer’s conditions that cause a transaction to be transmitted online |
| _EMVPARAM_DF11_TACDEFAULT | Tag:DF11 Length: 5 Format: b | Terminal Action Code-Default Specifies the acquirer’s conditions that cause a transaction to be rejected if it might have been approved online, but the terminal is unable to process the transaction online |
| …… |  |  |

    - Contactless Rupay AIDs

Data Items Specific to Contactless Rupay AIDs

| Configuration Item | Tag | Description |
| --- | --- | --- |
| _EMVPARAM_9F06_AID | Tag:9F06 Length: 5-16 Format: b | Application Identifier (AID) Identifies the application as described in ISO/IEC 7816-5 |
| _EMVPARAM_DF37_KERNELID | Tag:DF37 Length: 8 Format: b | Kernel ID Valid Value: “\x0D\x00\x 00\x 00\x 00\x 00\x 00\x 00” |
| _EMVPARAM_DF13_TACDENIAL | Tag:DF13 Length: 5 Format: b | Terminal Action Code-Denial Specifies the acquirer’s conditions that cause the denial of a transaction without attempt to go online |
| _EMVPARAM_DF12_TACONLINE | Tag:DF12 Length: 5 Format: b | Terminal Action Code-Online Specifies the acquirer’s conditions that cause a transaction to be transmitted online |
| _EMVPARAM_DF11_TACDEFAULT | Tag:DF11 Length: 5 Format: b | Terminal Action Code-Default Specifies the acquirer’s conditions that cause a transaction to be rejected if it might have been approved online, but the terminal is unable to process the transaction online |
| _EMVPARAM_9F09_APPVER | Tag:9F09 Length: 2 Format: b | Application Version Number Version number assigned by the payment system for the application Rupay need to set 0002 |
| Optional _EMVPARAM_1F8103_ADDTERCAP | Tag:1F8103 Length: 2 Format: b | Additional Terminal Capabilities Extension DF3A Byte2 bit7 |
| Optional _EMVPARAM_1F8104_SERVICEID | Tag:1F8103 Length: 2 Format: b | Service ID DF16 |
| Optional _EMVPARAM_1F8105_SERVICEQUA | Tag:1F8105 Length: 5 Format: b | Service Qualifier |
| Optional _EMVPARAM_1F8106_SERVICEDATA | Tag:1F8106 Length: max96 Format: b | Service Data DF45 |
| Optional _EMVPARAM_1F8107_SERVICEPRMISS | Tag:1F8107 Length: 16 Format: b | Service PRMiss DF47 |
| Optional _EMVPARAM_1F8108_SERVICEPRMACQKEY | Tag:1F8108 Length: 24 Format: b | Service PRMacq Key Value 1F8108 /**PRMacq key*/ typedef struct { uchar ucServiceID[2]; uchar ucPRMacqKeyIndex; uchar ucPRMacq[16]; uchar ucKCV[3]; uchar ucRfu[2]; }STPRMACQKEY; |

    - Contactless Interac AIDs

Data Items Specific to Contactless Interac AIDs

| Configuration Item | Tag | Description |
| --- | --- | --- |
| _EMVPARAM_9F06_AID | Tag:9F06 Length: 5-16 Format: b | Application Identifier (AID) Identifies the application as described in ISO/IEC 7816-5 |
| _EMVPARAM_DF37_KERNELID | Tag:DF37 Length: 8 Format: b | Kernel ID Valid Value: “\x21\x00\x 00\x 00\x 00\x 00\x 00\x 00” |
| _EMVPARAM_DF13_TACDENIAL | Tag:DF13 Length: 5 Format: b | Terminal Action Code-Denial Specifies the acquirer’s conditions that cause the denial of a transaction without attempt to go online |
| _EMVPARAM_DF12_TACONLINE | Tag:DF12 Length: 5 Format: b | Terminal Action Code-Online Specifies the acquirer’s conditions that cause a transaction to be transmitted online |
| _EMVPARAM_DF11_TACDEFAULT | Tag:DF11 Length: 5 Format: b | Terminal Action Code-Default Specifies the acquirer’s conditions that cause a transaction to be rejected if it might have been approved online, but the terminal is unable to process the transaction online |
| _EMVPARAM_9F59_TTI | Tag:9F59 Length: 3 Format: b | Terminal Transaction Information |
| _EMVPARAM_9F5E_TOS | Tag:9F5E Length:2 Format: b | Terminal Option Status |
| _EMVPARAM_9F58_MTI | Tag:9F58 Length:1 Format: b | Merchant Type Indicator |

    - Contactless MIR AIDs

Data Items Specific to Contactless MIR AIDs

| Configuration Item | Tag | Description |
| --- | --- | --- |
| _EMVPARAM_9F06_AID | Tag:9F06 Length: 5-16 Format: b | Application Identifier (AID) Identifies the application as described in ISO/IEC 7816-5 |
| _EMVPARAM_DF37_KERNELID | Tag:DF37 Length: 8 Format: b | Kernel ID Valid Value: “\x81\x06\x 43\x 00\x 00\x 00\x 00\x 00” |
| _EMVPARAM_DF13_TACDENIAL | Tag:DF13 Length: 5 Format: b | Terminal Action Code-Denial Specifies the acquirer’s conditions that cause the denial of a transaction without attempt to go online |
| _EMVPARAM_DF12_TACONLINE | Tag:DF12 Length: 5 Format: b | Terminal Action Code-Online Specifies the acquirer’s conditions that cause a transaction to be transmitted online |
| _EMVPARAM_DF11_TACDEFAULT | Tag:DF11 Length: 5 Format: b | Terminal Action Code-Default Specifies the acquirer’s conditions that cause a transaction to be rejected if it might have been approved online, but the terminal is unable to process the transaction online |
| _EMVPARAM_1F811A_NOCVMLIMIT | Tag:1F811A Length: 6 Format: b | Terminal No Cvm Limit DF52 |
| _EMVPARAM_1F811B_CONTACTLESSLIMIT_NOCDCVM | Tag:1F811B Length: 6 Format: b | Terminal Contactless Limit NOCDCVM DF53 |
| _EMVPARAM_1F811C_CONTACTLESSLIMIT_CDCVM | Tag:1F811C Length: 6 Format: b | Terminal Contactless Limit CDCVM DF54 |
| _EMVPARAM_1F811D_TERTPM_CAP | Tag:1F811D Length: 2 Format: b | Terminal TMP Cap DF55 |

    - Contactless Multibanco AIDs

Data Items Specific to Contactless Multibanco AIDs

| Configuration Item | Tag | Description |
| --- | --- | --- |
| _EMVPARAM_9F06_AID | Tag:9F06 Length: 5-16 Format: b | Application Identifier (AID) Identifies the application as described in ISO/IEC 7816-5 |
| _EMVPARAM_DF37_KERNELID | Tag:DF37 Length: 8 Format: b | Kernel ID Valid Value: “\xC1\x4D\x42\x 00\x 00\x 00\x 00\x 00” |
| _EMVPARAM_DF13_TACDENIAL | Tag:DF13 Length: 5 Format: b | Terminal Action Code-Denial Specifies the acquirer’s conditions that cause the denial of a transaction without attempt to go online |
| _EMVPARAM_DF12_TACONLINE | Tag:DF12 Length: 5 Format: b | Terminal Action Code-Online Specifies the acquirer’s conditions that cause a transaction to be transmitted online |
| _EMVPARAM_DF11_TACDEFAULT | Tag:DF11 Length: 5 Format: b | Terminal Action Code-Default Specifies the acquirer’s conditions that cause a transaction to be rejected if it might have been approved online, but the terminal is unable to process the transaction online |
| _EMVPARAM_DF8118_CVMCAP_ABOVEEQUALLIMIT | Tag:DF8118 Length: 1 Format: b | CVM Cap-Above CVM required DF8118 |
| _EMVPARAM_DF8119_CVMCAP_BELOWLIMIT | Tag:DF8119 Length: 1 Format: b | CVM Cap-Below CVM required DF8119 |

- Annex A: Sample Communication Traces

You can see below some examples of protocol level communication traces between the terminal and the PIN pad. The data sent from the terminal is shown in blue, the PIN pad response in red.

  - Display Text Request

02 00 19 33 43 2F 52 45 41 44 59 20 20 20 20 20 20 20 20 20 20 20 03

2E

06

  - Card Insertion Status Request

02 00 04 45 36 2F 00 03 5B

06

02 00 05 45 37 2F 31 30 03 5A

06

- ANNEX B. ADDITIONAL SPECIFICATION ASPECTS

  - Public Key Infrastructure for Newland Devices

All Newland devices make use of Newland’s Public Key Infrastructure. The following figure summarises the structure of this PKI, which is fully specified in the most up to date version of document NEWLAND KEY LOADING AND SW SIGNING / AUTHENTICATION. SPECIFICATION.

For different customers, there is a corresponding CA chain, so that the customer’s CA can be embed in the firmware during the compiling time. And this prevents different customers to load firmware/keys to the device of the others.

In the case of the SP100, the following certificates and uses are contemplated:

- Manufacturing CA Public Certificate: this public certificate is part of the Firmware, and it is used in the Key Loading Device authentication process executed as part of an initial key loading session executed in a secure environment. The specification of the KLD authentication mechanism can be found in the most up to date version of document NEWLAND INITIAL KEY LOADING FUNCTIONAL SPECIFICATION.

- Production CA Public Certificate: this public certificate is part of the Pinpad Firmware. It is used to authenticate the trust chain included in firmware.

- Application Provider CA Public Certificate: this public certificate identifies an organization with the ability to sign applications and prompts for Newland devices. This certificate is passed to the device as part of the trust chain it will have to validate when verifying a Secure Prompt Load request.

- Application Signer Public Certificate: this public certificate identifies an application signer key pair which can be used to sign applications and prompts for Newland devices. This certificate is passed to the device as part of the trust chain it will have to validate when verifying a Secure Prompt Load request. Prompts loaded into devices are signed with the private key corresponding to the public key in the certificate passed to the device.