TTF Script print command standard

CONTENTS

1.	Command Overview	3

2.	Note: Font enumeration of !NLFONT  command is as follows:	6

2.1	Chinese character enumeration	6

2.2	English font enumeration	7

2.3	Bar code enumeration	8

Bar code description	8

2.4	QR Code enumeration	8

QR Code description	8

3. Cmd Definitions	9

3.1	Cmd classify	9

3.2 Cmd Tag	9

3.3 Params	9

3.4	Output	9

4.	Example	10

- Command Overview

| Cmd classify | Cmd Tag | Param1 | Param2 | Param 3 | Param 4 | description |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ! | hz | font size (s,n,l,sn,sl,nl)---- s（Small font）： print 24 Chinese characters in a row n（standard font）：print 16 Chinese characters in a row l（Large font）： print 12 Chinese characters in a row sn：Small font width, standard font height； sl：Small font width, large font height； nl：standard font width, large font height； |  |  |  | Chinese style |  |
|  | asc | font size (s,n,l,sn,sl,nl) ---- s（Small font）： print 48 characters in a row n（standard font）：print 32 characters in a row l（Large font）: print 24 characters in a row sn：Small font width, standard font height; sl：Small font width, large font height; nl：standard font width, large font height; |  |  |  | English style |  |
|  | gray | gray value:[1，10] default value: 5 |  |  |  | Print grayscale 1~5 -Normal 6~10 -Bold） |  |
|  | yspace | Line Space value： [0, 60] default value : 6 |  |  |  | Line Space |  |
|  | barcode | barcode width: [1，2] default value: 2 recommend >=2 | barcode height： [1，320] default value 64 It’s Must be a multiple of 8; |  |  | barcode style |  |  |  |
|  | BARCODE | Barcode width [1,8] Default 2 | Expect height [1,384] Default 64 | The switch button of showing tips below barcode 0: No display 1: display | Barcode Code（Corresponding Barcode enumeration） | New function to print barcode. Support more kinds of barcode. Corresponding “*BARCODE” |  |  |  |

| ! | qrcode | side lengt： default value: 100 max width 384 | fault tolerance level: [0,3] default value: 2 |  |  | QR code style |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | QRCODE | Except width[1,384] Default 100 | fault tolerance level: [0,3] default value: 2 Only support QRcode. | QRcode Code（Corresponding QRcode enumeration） |  | New function to print barcode. Support more kinds of barcode. Corresponding “*QRCODE |  |  |
|  | NLFONT | Chinese Font Index Please reference below. | English Font Index Please reference below. | Print mode(optional) 0: Horizontal twice, vertical twice 1: Horizontal twice, vertical normal 2: Horizontal twice, vertical zoom 3: normal 4: Horizontal triple, vertical triple 5: Horizontal triple, vertical normal 6: Horizontal normal, vertical triple | Space mode (optional) 0：normal 1：zoom with print mode | Set Chinese font size, English font size, print mode. |  |  |
|  | NLPRNO VER |  |  |  |  | print some blank paper |  |  |
|  | font | .ttf file |  |  |  | Used to load the font |  |  |
|  | reverse | on: reverse off: not reverse |  |  |  | reverse the current text color |  |  |
| * | text | the alignment of text (l, c, r ) l - left c - center r - right | String data |  |  | Print text |  |  |
|  | TEXT | the alignment of text (l, c, r ) l - left c - center r - right | String data |  |  | The usage of this is the same as *text. The differences are that *text can change line automatically but *TEXT can’t. And *TEXT will print when the next *text or *underline prints. |  |  |

| * | underline | the alignment of text (l, c, r ) l - left c - center r - right |  | String data |  |  | will print when the next *text or *underline prints |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | UNDERLINE | the alignment of text (l, c, r ) l - left c - center r - right |  | String data |  |  | The usage of this is the same as *underline. The differences are that *text can change line automatically but *TEXT can’t. And *TEXT will print when the next *text or *underline prints. |  |  |  |
|  | feedline | Rows (int) Eg. 2 |  |  |  |  | print some blank paper |  |  |  |
|  | line |  |  |  |  |  | print a broken line |  |  |  |
|  | barcode | the alignment of barcode (l, c, r) l - left c - center r - right |  | String data |  |  | Print barcode |  |
|  | BARCODE | the alignment of barcode (l, c, r) l - left c - center r - right |  | String data |  |  | New function to print barcode. Support more kinds of barcode. |  |

| * | qrcode | the alignment of qrcode (l, c, r) l - left c - center r - right | String data |  |  | Print QRcode |
| --- | --- | --- | --- | --- | --- | --- |
|  | QRCODE | the alignment of qrcode (l, c, r) l - left c - center r - right | String data |  |  | New function to print QRcode. Support more kinds of QRcode. |
|  | image | The alignment of image (l, c, r) l - left c - center r - right | Image size X*Y X:width Y:height Eg.:128*64 | Image data (please reference demo) |  | Prtint image |
|  | pause | Pause time (Unit -second) |  |  |  | Pause for a while then continue to print |
|  | cut |  |  |  |  | Paper cutting，used for cpos x3 or cops x5 device |

- Note: Font enumeration of !NLFONT  command is as follows:

  - Chinese character enumeration

Chinese characters enumeration(EM_PRN_HZ_FONT)

PRN_HZ_FONT_24x24 = 1, PRN_HZ_FONT_16x32 , PRN_HZ_FONT_32x32 , PRN_HZ_FONT_32x16 , PRN_HZ_FONT_24x32 , PRN_HZ_FONT_16x16 , PRN_HZ_FONT_12x16 , PRN_HZ_FONT_16x8 , PRN_HZ_FONT_24x24A , /**< Private song typeface 24x24 lattice */ PRN_HZ_FONT_24x24B , /**< Imitation song typeface 24x24 lattice */ PRN_HZ_FONT_24x24C , /**< Regular script 24x24 lattice */ PRN_HZ_FONT_24x24USER , PRN_HZ_FONT_12x12A , /**<Song typeface 12x12 lattice */ PRN_HZ_FONT_16x24 , PRN_HZ_FONT_16x16BL, /**16x16 bold*/ PRN_HZ_FONT_24x24BL, /**24x24 bold*/ PRN_HZ_FONT_48x24A , PRN_HZ_FONT_48x24B , PRN_HZ_FONT_48x24C , PRN_HZ_FONT_24x48A , PRN_HZ_FONT_24x48B , PRN_HZ_FONT_24x48C , PRN_HZ_FONT_48x48A , PRN_HZ_FONT_48x48B , PRN_HZ_FONT_48x48C,

  - English font enumeration

English font enumeration(EM_PRN_ZM_FONT)

PRN_ZM_FONT_8x16 = 1, PRN_ZM_FONT_16x16 , PRN_ZM_FONT_16x32 , PRN_ZM_FONT_24x32 , PRN_ZM_FONT_6x8 , PRN_ZM_FONT_8x8 , PRN_ZM_FONT_5x7 , PRN_ZM_FONT_5x16 , PRN_ZM_FONT_10x16 , PRN_ZM_FONT_10x8 , PRN_ZM_FONT_12x16A , /**<MSGothic bold 12x16 lattice*/ PRN_ZM_FONT_12x24A , /**<Gulimche font 12x24 lattice*/ PRN_ZM_FONT_16x32A , /**<MSGothic bold 16x32 lattice*/ PRN_ZM_FONT_12x16B , /**<MSGothic bold 12x16 lattice*/ PRN_ZM_FONT_12x24B , /**<MSGothic bold 12x24 lattice*/ PRN_ZM_FONT_16x32B , /**<MSGothic bold 16x32 lattice*/ PRN_ZM_FONT_12x16C , /**< Regular script bold 12x16 lattice */ PRN_ZM_FONT_12x24C , /**< Regular script bold 12x24 lattice */ PRN_ZM_FONT_16x32C , /**< Regular script bold 16x32 lattice */ PRN_ZM_FONT_24x24A , PRN_ZM_FONT_32x32A , PRN_ZM_FONT_24x24B , PRN_ZM_FONT_32x32B , PRN_ZM_FONT_24x24C , PRN_ZM_FONT_32x32C , PRN_ZM_FONT_12x12 , PRN_ZM_FONT_12x12A , PRN_ZM_FONT_12x12B , PRN_ZM_FONT_12x12C , PRN_ZM_FONT_8x12 , PRN_ZM_FONT_8x24, PRN_ZM_FONT_8x32, PRN_ZM_FONT_12x32A, PRN_ZM_FONT_12x32B, PRN_ZM_FONT_12x32C, PRN_ZM_FONT_8x16BL, /**8x16 bold, enum36*/ PRN_ZM_FONT_16x16BL, PRN_ZM_FONT_12x24BL, /**12x24 bold, enum38*/

  - Bar code enumeration

Bar code enumeration(PRN_BARCODE_SYMBOLOGY)

BARCODE_SYMBOLOGY_CODABAR = 0, /**CODABAR*/ BARCODE_SYMBOLOGY_CODE39 = 1, /**CODE39*/ BARCODE_SYMBOLOGY_CODE93 = 2, /**CODE93*/ BARCODE_SYMBOLOGY_CODE128 = 3, /**CODE128*/ BARCODE_SYMBOLOGY_EAN = 4, /**EAN-8/JAN-8 & EAN-13/JAN-13 are using this code and are separated according to the length*/ BARCODE_SYMBOLOGY_ITF = 5, /**ITF*/ BARCODE_SYMBOLOGY_UPCA = 6, /**UPC-A*/ BARCODE_SYMBOLOGY_UPCE = 7 /**UPC-E*/

Bar code description

- CODABAR: The first and last code should be one of 'A'、'B'、'C'、'D', and the rest should be number.

- CODE39: Only support 10 numbers, 26 letters and 7 special characters (‘-’, ‘.’, ‘$’, ‘/’, ‘+’, ‘%’, space). The 43 characters can encode data of any length.

- CODE93: 10 numbers, 26 letters and 7 special characters (‘-’, ‘.’, ‘$’, ‘/’, ‘+’, ‘%’, space).

- CODE128: 128 characters from ASCII 0 to ASCII 127, including numbers, case letters and special characters.

- EAN-8/JAN-8: Only support numbers. The length is 8, 7 for data and 1 for check. If the data is not enough, zeros will be in front.

- EAN-13/JAN-13: Only support numbers. The length is 13, 12 for data and 1 for check. If the data is not enough, zeros will be in front.

- ITF: Only support numbers. The length is 14, 13 for data and 1 for check. If the data is not enough, zeros will be in front.

- UPC-A: Only support numbers. The length is 8, the first number is 0 or 1, then 6 for data and 1 for check. UPC-E:

  - QR Code enumeration

QR Code enumeration(PRN_QRCODE_SYMBOLOGY)

QRCODE_SYMBOLOGY_DATAMATRIX = 0, /**Data Matrix*/ QRCODE_SYMBOLOGY_MAXICODE = 1, /**Maxicode*/ QRCODE_SYMBOLOGY_PDF417 = 2, /**PDF417*/ QRCODE_SYMBOLOGY_QRCODE = 3 /**QR Code*/

QR Code description

- DC(Data Matrix): The encoded characters includes all ASCII and extended ASCII characters. 256 characters in total.

- MC(MaxiCode): Ditto

- PDF-417: Support letters, numbers and about 500 Chinese characters.

- QR Code: Support Chinese character well.

3. Cmd Definitions

  - Cmd classify

- "!"  Indicates a setting command that sets the style. Once set, subsequent printing command will be affected.

- "*"  Indicates the output command. It can be used to output text, images, bar codes, QR codes and so on.

- 3.2 Cmd Tag

A unique instruction identifier constitutes with the Cmd Tag and the Cmd classify.

- 3.3 Params

Each print command requires should setting params, the total number of params for different instructions varies from 1 to 4. The params are separated by a space.

  - Output

It will line feed automatically after developer use any command to output content. If the content expresses serval lines, developer should transfer newline character. Using ’\n’ to express newline.

- Example

Instruction:

!hz n

!asc n

!gray 10

!yspace 8

*image c 128*64 #ums

or

*image c 128*64 data: base64;R0lGODlhuAE8D/i…..

*text c ChinaUMS-UMS Pay

...................

!hz l

!asc l

*text c Convenient to use，finish your contactless payment with mobile phone.

*text c TEL:95534

" ChinaUMS-UMS Pay"

……

“Convenient to use，finish

your contactless payment with

mobile phone.”

Service TEL：95534